Document of the Inter-American Development Bank

JAMAICA

Energy Efficiency and Conservation Program

(JA-L1025)

- Economic Analysis Annex

This document was prepared by the project team consisting of: Natacha Marzolf and Sylvia Larrea (INE/ENE) Co-Team Leaders; Paola Mendez (INE/ENE); under the supervision of Leandro Alves, Energy Division Chief (INE/ENE) and Ancile Brewster, Representative (CCB/CJA).

NOTE: Project teams have two options for ex ante Economic Analysis of projects.

Option 1 is conducting a cost-benefit analysis (CBA). If this option is taken, the CBA template should be followed.

Option 2 is conducting a cost effectiveness analysis (CEA). If this option is taken, the CEA template should be followed.

If neither option is taken, an explanation must be provided in the Economic Analysis Annex of why no analysis was undertaken.

Option 1: Cost Benefit Analysis

- Introduction

The EECTA technical assistance (JA-T1044) intended to assess the benefits of investments in Energy Efficiency (EE) and Energy Conservation measures (EC) within the Jamaican public sector. This project conducted 36 energy audits of facilities of the Government of Jamaica (GoJ), which form the basis for a wider techno-commercial analysis to establish the viability of the project. In this context, a cost benefit analysis has been conducted to evaluate the costs and benefits over a 20 year project life cycle. Primarily, the benefits accrued from the cost of energy saved resulting from the reduced consumption of electricity from energy efficient technologies. Since Jamaican electricity generation is heavily dependent on imported oil, the project is likely to offset oil imports required for electricity generation, reducing at same extent the pressure on the country’s payment balance. One other indirect benefit is the reduction in Carbon Dioxide (CO2eq) emissions resulting from the reduction of electricity consumption.

- Assumptions and Methodology

The project considers existing equipment found in the energy audits of 33 facilities of the Government of Jamaica. The existing equipment/energy conservation measures are categorized into the following way:

  - lighting

  - air conditioning

  - building envelope

  - street lighting

It has been observed that the equipment currently installed is not energy efficient, leading to increased energy consumption in the public sector.

Based on the energy efficiency and conservation master model (see electronic link attached to the loan proposal) and with the interaction of energy efficient lights, air conditioners and building envelope measures, an overall 5% of the yearly energy consumption of the Jamaican public sector can be saved.

- Assumptions used in the Cost Benefit Analysis:

- The useful lifetime for each piece of retrofit equipment is summarized in the table below. These lifetimes are used to determine the frequency and spacing of replacements within the model period.

| Table 1: Equipment life and replacement frequency |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Equipment Life |  | Frequency of Replacement |  |
| LED | 35,000 | hours | 10 | years |
| CFL | 10,000 | hours | 3 | years |
| Mini-split AC | 15 | years | 15 | years |
| Window Film | 15 | years | 15 | years |
| Cool Roof Coating | 5 | years | 5 | years |

- The replacement frequency for lighting and air conditioning measures is based on the annual operating hours of equipment, which are derived from the energy audits.

- A life cycle period of 20 years is used in the CBA.

- The electricity rate used to determine the cost savings from energy is US$ 0.30 /kWh. The electricity prices is considered constant over time (i) given the volatility in the international oil prices what makes very difficult to foreseen electricity prices over a 20 year period and (II) since the GoJ may consider the possibility for fuel switching from oil to gas which may bring down the electricity price. Within these constraints, it is difficult to project electricity price over time and hence a constant price approach is considered plausible and conservative.

- The annual operating hours taken from the audits are:

  - Lighting:	3366 hr/yr

  - Air Conditioning: 3759 hr/yr

  - Street Lighting: 4450 hr/yr

The life cycle cost benefit analysis methodology has been used. The benefit streams capture the cost of energy saved due to energy efficiency improvements in public buildings. The cost streams include the initial investment cost for equipment, the replacement cost over a 20 year project life cycle, and the annual maintenance cost.

This CBA analysis follows then a conservative pattern. The barrels of oil saved due to reduced energy consumption were estimated but were not included in the cash flow analysis. Nevertheless from the country’s point of view, these are also direct benefits since it reduces the pressure on the payments balance. Following the same logic, the benefits that could arise from CO2 carbon credits were not included.. These benefits were however roughly estimated.

- A discount rate of 12% is used.

- Economic Benefits

- Benefits included in the cash flow

The benefits quantified are the annual electricity saved in MWh and the cost of electricity saved in US$. Annually, 22,609 MWh will be saved and the first year electricity cost savings will be US$ 6.7 million.

Table 2: Project benefit streams

- Benefits not included in the cash flow

In addition, the amount of oil saved due to reduced electricity consumption in public facilities, stemming from energy efficiency improvement, has also been quantified. It is estimated that 13,886 barrels of oil will be saved annually.

The project will also offset CO2e emissions by 19,150 tonnes per year.

- Economic Costs

The investments are attributed to the installation and replacement of the equipment. The total initial investment of US$ 17 million comprises of the following measure investments:

| Table 3: Initial Investment |  |
| --- | --- |
| Equipment | Initial Investment [US$] |
| Lighting | 7,503,428 |
| Air Conditioning | 7,317,005 |
| Cool Roof | 724,776 |
| Window Film | 300,869 |
| Door Sealant | 153,922 |
| Street Lighting | 1,000,000 |
| TOTAL | 17,000,000 |

The replacement cost of equipment has also been considered which is linked to the lifetime of the equipment.

Table 4: Project cost streams

- No factor contributing to a shadow price assessment was necessary.

- An annual maintenance cost of 5% of the electricity saved, has been accounted in this CB analysis.

- No incremental cost has been considered in this CB analysis.

- The project implementation will not have any impact on the resource costs.

- The project is not expected to generate any negative external impacts.

- Economic Returns

- The NPV at a 12% discount rate is US$ 32,271,600.

- The ERR is 61%.

Table 5: Total benefit and cost streams

- The project is highly attractive, providing an NPV of over US$ 32 million in 20 years, with a rate of return of 61%.

- Sensitivity analysis

The sensitivity analysis takes into account changes in electricity rate tariff from US$ 0.10/kWh to US$ 0.30/kWh. The analysis shows that the project is attractive even at low electricity tariffs.

| Change in electricity tariff | NPV | ERR |
| --- | --- | --- |
| 10 Cents/kWh | (3,666,485) | 6% |
| 15 Cents/kWh | 5,318,036 | 19% |
| 20 Cents/ kWh | 14,302,557 | 32% |
| 25 Cents/kWh | 23,287,079 | 45% |
| 30 Cents/kWh | 32,271,600 | 61% |

- Conclusions

The project intends to implement energy efficiency improvement programs in the Jamaican public sector. The cost benefit analysis shows that with an initial investment of US$ 17 million, for installation of energy efficient equipment and building envelope improvements will result in an electricity savings of 5% over the baseline (year 2010)..

Over a period of 20 years, it is expected that the project will have a Net Present Value of US$ 32 million with an ERR of 61%.