DOCUMENT OF THE INTER-AMERICAN DEVELOPMENT BANK

Jamaica

Energy Management and Efficiency Programme

(JA-L1056)

Cost Benefit Analysis

(Retrofitting Buildings – Component I)

and

(Fuel Efficiency – Component II)

October, 2016

CBA analysis for Component I is based on the report “Energy Conservation & Efficiency for Jamaica: Economic Assessment and Investment Plan prepared by: DNV-GL dated August 2016

CBA analysis for Component II was prepared by: Klaus Banse, Prof.-Univ. Dipl.-Ing.

INDEX

CBA COMPONENT I – RETROFITTING BUILDINGS

I.	EXECUTIVE SUMMARY

II.	INTRODUCTION AND BACKGROUND

III.	ECONOMIC ASSESSMENT

3.1	Main assumptions and methodology

3.2	Base Case

IV.	INVESTMENT ASSESSMENTS

4.1	Economic benefits

4.2	Economic cost

4.3	Economic returns

4.4	Sensitive analysis

V.	CONCLUSIONS

VI.	REFERENCES

CBA COMPONENT II – FUEL EFFICIENCY

ABBREVIATIONS

- INTRODUCTION

- ASSUMPTIONS AND METHODOLOGY

- ECONOMIC BENEFITS

- ECONOMIC COSTS

- ECONOMIC RETURN

- SENSITIVITY ANALYSIS

- ADDITIONAL ANALYSIS

- CONCLUSIONS

- REFERENCES

- ANNEXES

- APPENDIX A – Detailed Sensitivity Analysis

CBA COMPONENT I – RETROFITTING BUILDINGS

Glossary

| Bls | Barrels |
| --- | --- |
| BOE | Barrels of Oil Equivalent |
| BoJ | Bank of Jamaica |
| DSM | Demand Side Management |
| EC | Energy Conservation |
| EE | Energy Efficiency |
| EIRR | Economic Internal Rate of Returt |
| ENPV | Economic Net Present Value |
| ESET | Electricity Sector Enterprise Team |
| EWP | East West Power |
| GCT | General Consumption Tax |
| GER | Government Electricity Regulator |
| GNI | Gross National Income |
| GoJ | Government of Jamaica |
| HF | Heavy Fuel |
| ICT | Information and Communication Technologies |
| IPP | Independent power producers |
| IRP | Integrated Resource Plan |
| JAMALCO | Jamaica Aluminum Company |
| JEC | Jamaica Energy Council |
| JEP | Jamaica Energy Partners |
| JPPC | Jamaica Private Power Company |
| JPS | Jamaica Public Service Company |
| LCE | Low Carbon Economy |
| LNG | Liquefied Natural Gas |
| MSET | Ministry of Science, Energy and Technology |
| NECEP | National Energy Conservation and Efficiency Policy |
| NEP | National Energy Policy |
| NGCC | Natural Gas Combined Cycle |
| OUR | Office of Utilities Regulation |
| PCJ | Petroleum Corporation of Jamaica |
| Petrojam | Petroleum of Jamaica - PDVSA/PCJ owned company |
| PIoJ | Planning Institute of Jamaica |
| PM | Project Manager |
| PPA | Power Purchase Agreement |
| PS | Project Sponsor |
| PSA | Production Sharing Agreement |
| PV | Photovoltaic |
| QoS | Quality of Service |
| RE | Renewable Energies |
| REP | Rural Electrification Program |
| RFP | Request for Proposal |
| SAIDI | System Average Interruption Duration Index |
| SAIFI | System Average Interruption Frequency Index |
| T&D | Transmission and Distribution |
| TFP | Total Factor Productivity |
| URA | Office of Utilities Regulation Act of 1995 |
| US$ | United States Dollars |
| VAT | Value Added Tax |
| VRE | variable renewable energy |
| WACC | weighted average cost of capital |
| WKPP | West Kingston Power Partners |
| WTI | West Texas Intermediate |

I.	EXECUTIVE SUMMARY

Energy efficiency implementation includes improvements to building envelope, lighting systems, mechanical systems, plumbing systems, controls, miscellaneous equipment, and renewable energy system implementation if applicable. In most cases, the facility includes more than one building; however they are operated as a single facility and should be considered as such. The facilities included in this scope of work include:

- Kingston Public Hospital and Victoria Jubilee Hospital

- Cornwall Western Regional Hospital

- Mandeville Public Hospital

- Marcus Garvey High School

- Heart Trust Ebony Park Academy

- Falmouth Hospital

In addition to this, we have been provided with investment figures for lighting retrofit in 45 buildings that constitutes a second project.

This report includes the cost benefit analysis for building and lighting projects.

DNV-GL has executed an assessment over six different buildings in Jamaica and has used the information for another 38 buildings for lighting retrofit, in order to assess its conditions and potential improvement measures that could lead to savings during the following years.

After the technical inspections, DNV-GL has run a series of financial tests considering the potential loan interest rates and the investments that were needed to be carried out.

As a result, excellent financial results which have been Economic Internar Rate of Return (EIRR) values between 14% and 23% for the buildings project and 53% and 83% in the lighting project, and extremely positive Economic Net Present Value (ENPV), both depicted in section 4.1 have been obtained considering that, the implementation of the energy efficiency measures shall be accepted on economic merit.

II.	INTRODUCTION AND BACKGROUND

Jamaica has been one of the most active countries in the Caribbean from an energy policy perspective. Some of the main issues addressed include renewable energy penetration, energy efficiency and conservation, and system losses. Despite these efforts, dependence on energy imports and electricity costs for the consumer remains high; volatile oil markets create uncertainty in prices and deliveries; and adoption of renewable energy to reduce emissions requires planning for these unique resources. 1 Planning for these efforts requires an integrated resource plan to:

- Communicate strategy and implementation thereof

- Coordinate infrastructure requirements

- Notify all interested parties of changes to allow said parties to adapt to the changes

- Determine the scope of potential impacts

The National Energy Conservation and Efficiency Policy 2010-2030 (NECEP) provides the overarching framework for energy efficiency (EE) in Jamaica with a national level target for a reduction in energy intensity and consumption across all sectors of the economy from 21,152 BTU in 2009 to produce US$1 of output to 6,000 BTU/US$1 of output by 2030.

Table 2.1. Goals of Jamaica National Plan.

Source: Jamaica Energy Policy 2009-2030.

Of particular highlight in NECEP for this Program are Goal 1 and Goal 6.

An IDB-funded study of 6 walk through and investment grade audits investigated the potential of deep retrofits in government facilities. The findings revealed that it is possible to achieve a reduction in electricity consumption of up to 30% within selected public sector (named as Health, Education and Public Agencies – HEPA) facilities, producing saving the GOJ electricity bills and therefore in imported oil barrels for those public facilities.

Previous to EMEP - The Energy Efficiency and Conservation Technical Assistance (EECTA) report which was completed in 2011 and reviewed 2009-2010, was based on 22 Walk through audits and 14 detailed audits. The EECTA indicated that an investment of approximately US$113 million had the potential of saving 101GWh/year, or 25% of the electricity consumption of the Jamaican public sector at the time (estimated at 411kWh/year).

Based on discussions with MSET, for a projected spend of US$20 million, it was determined that a 4-5% reduction in electricity consumption in public sector facilities could be realized.

In this project, between the six buildings and the lighting retrofit, the total investment costs are over US$10 million.

DNV-GL provided technical research related to energy efficiency and renewable energy retrofitting of government buildings. The facilities included in the analysis were the following:

- Kingston Public Hospital and Victoria Jubilee Hospital

- Cornwall Western Regional Hospital

- Mandeville Public Hospital

- Marcus Garvey High School

- Heart Trust Ebony Park Academy

- Falmouth Hospital

In addition to this, we have been provided with investment figures for lighting retrofit in 45 buildings that constitutes a second project.

Besides, DNV-GL is assisting in the preparation of necessary documents for an IDB’s loan preparation. The economic analysis documented herein supports that activity.

The analysis includes a detailed description of assumptions and methodology, economic benefits and costs, economic return and a sensitivity analysis of key variables.

III. ECONOMIC ASSESSMENT

3.1	Main assumptions and methodology

The following are the assumptions and methodology for the economic evaluation of the recommended energy efficiency (EE) measures:

- The economic analysis is based on the energy technical assessment performed to 6 public buildings, mainly related to schools and hospitals, in Jamaica to improve their energy characteristics. Energy efficiency implementation includes improvements to building envelope, lighting systems, mechanical systems, plumbing systems, controls, miscellaneous equipment, and renewable energy system, if applicable. In most cases, the facility includes more than one building; however they are operated as a single facility and should be considered as such.

- The economic analysis follows a Cost Benefit Analysis (CBA):

- Economic Benefits consist of electricity savings (direct effect);

- Economic Costs consist mainly on the investment cost of the different recommended measures (O&M are lower than BAU therefore with very little impact on the figures);

- Other externalities such as reduction of CO2 emissions have been considered with an assessment purpose, but have not been considered as a benefit for the base case scenario for the project as there is no CO2 market where to sell the emissions. However a scenario including a monetary valuation of CO2 emissions reduction was also run.

- For each public building under investigation DNV-GL has recommended a number of energy efficiency investment measures. The economic life of each measure varies from 5 to 20 years. The stream of economic benefits takes into account the economic life of the different measures.

- The analysis is performed using a 20-year reference period to be able to capture the overall economic life of the project assets.

- The reductions of CO2 emissions were determined based on an emission factor (ton CO2 per kWh): 0.000637 (source: IEA).

- The economic discount rate used was the standard 12% typically used by the IDB.

- Electricity prices are based on an average of historical costs (US$/kWh) per building without considering inflation.

- Jamaica has high energy rates and a large dependence upon oil. Therefore, the escalation rates were set based on the WTI real crude forecast from the Long Term Energy Outlook published by the US Department of Energy, Energy Information Agency (DNV-GL 2016). For the annual escalation rate we have used a rate of 0.29% for the first 5 years and 2.18% for the remaining years (based on WTI crude forecast growth rate).

- In the lighting project, several kinds of facilities have been considered, from transportation centres to ministries.

This assessment is not using efficient (shadow) pricing but market prices. This is because there are no recent estimates of shadow prices that could be used. Shadow prices were not addressed in the CBA analysis because of (i) limited recent data for key variable that are required for a shadow price assessment and, (ii) an ex-ante assessment suggests that applying shadow prices would increase the project's ENPV and EIRR. Shadow prices could be applied to the Jamaica dollar exchange rate (with the US dollar), to the price of imported fuels and materials, and to the cost of labor (both skilled and unskilled. An analysis of Jamaica shadow prices that was performed in 1987 estimated national economic parameters

3.2	Base Case

The energy savings are based on the energy technical assessments performed by DNV-GL Energy Engineers of the 6 public buildings in Jamaica. The goal of the work was to provide a path to widespread energy savings across municipal and federal buildings throughout Jamaica. The expected results of the energy project are:

- Reduced electricity consumption within government facilities;

- Decreased oil imports through improved EE;

- Reduced GHG emissions which can contribute to Jamaica’s INDC commitment;

- An increase capacity to promote and supervise electricity planning in Jamaica.

Having defined the objective of the intervention, the next step is to present in detail the proposed project to be implemented. Information and data about the project’s engineering features, technical characteristics and expected effects were provided in separate reports for each building in Jamaica. For further details please refer to the Assessment Reports submitted to the Ministry of Science, Energy and Technology (MSET).

The following table provides a summary of the recommended EE measures for each of the 6 buildings in Jamaica.

Table 3.1. Summary of the recommended measures per building.

| Measures/Building | Kingston Public & Victoria Jubilee Hospital | Cornwall Hospital | Heart Trust Ebony Park Academy | Falmouth Hospital | Mandeville Hospital | Marcus Garvey School |
| --- | --- | --- | --- | --- | --- | --- |
| Window Replacements with Insulated Glazing | X | X |  |  |  | X |
| Automatic Door Closers | X | X | X | X | X | X |
| Air Seals on Doors | X | X | X | X | X | X |
| Cool Roof Installation |  |  |  | X |  |  |
| Variable Refrigerant Flow (VRF) Systems | X |  |  |  |  |  |
| Proper Spacing between Split Units | X | X |  |  | X |  |
| Replace split systems with high efficiency, inverter-driven units | X |  | X |  | X | X |
| VFD Installation on Kitchen Fans | X |  |  |  | X |  |
| VFD Installation on Chiller Pumps | X |  |  |  | X |  |
| Fluorescent to LED Fixture Retrofit | X | X | X | X | X | X |
| Occupancy Sensor for Interior Lights | X | X | X | X | X | X |
| Compressed Air Leakage | X | X |  |  | X |  |
| Centralized Chiller Controls | X | X |  |  | X |  |
| Solar PV | X | X | X | X | X | X |
| VFD on supply and return fans |  | X |  | X |  |  |
| VFD on Exhaust Fans |  | X |  |  |  |  |
| VFD on constant volume pumps |  | X |  |  |  |  |
| BAS upgrades |  | X |  | X | X |  |
| Installment of Solar Tubes |  |  | X | X |  | X |
| Exterior Light Retrofit to LED |  |  | X |  | X |  |
| Replace Broken Windows |  |  |  | X |  |  |
| Proper Spacing Behind Condenser Units |  |  |  | X |  | X |

In the lighting project, there has been a consideration of the investment amounts to be done at each facility and the potential savings in terms of kWh, that have been translated to US$, by using the same approach as in the previous project.

IV.	INVESTMENT ASSESSMENTS

4.1	Economic benefits

4.1.1	Energy savings

Projects involving a refurbishment of public buildings or works to improve building envelope, lighting systems, mechanical systems, controls, miscellaneous equipment, and renewable energy system are associated with an increase of energy efficiency, which is reflected in a reduction of energy consumption costs. The benefit is valued via the decrease of energy costs incurred when attaining the same final useful effect as in the without-the-project scenario. The saved costs compared to the counterfactual (BAU) scenario represent the project’s benefit.

Table 4.1 Energy savings in each building.

| Buildings | Annual Energy Savings (kWh) | Annual Energy Savings (kWh)/year 3 |
| --- | --- | --- |
| Kingston Public and Victoria Jubilee Hospital | 3,401,935 | 860,923 |
| Cornwall Hospital | 1,964,284 | 497,099 |
| HEART Trust Ebony Park Academy | 742,008 | 187,779 |
| Falmouth Hospital | 259,292 | 65,618 |
| Mandeville Hospital | 448,348 | 11,3462 |
| Marcus Garvey School | 103,160 | 26,106 |

These figures are referenced to the audits executed in the buildings. References included in section VI.

The annual energy savings above were valued considering the average utility electricity rate (of previous 3 to 5 years depending on data availability) expressed in US$/kWh. To account for the potential rate fluctuation in the future the rates were escalated based on projections of oil price growth rates (given that the price of crude oil is one of the major drivers of electricity prices).

In the Lighting project, the target facilities and savings are the following:

Table 4.2 Energy savings in each building in the lighting project.

| Facilities | Estimated pre retrofit consumption (kWh) | Estimated post retrofit consumption (kWh) | Estimated savings per year (kWh) | Estimated savings per year (US$) |
| --- | --- | --- | --- | --- |
| Half-Way-Tree Transportation Centre | 261,921.60 | 143,640.00 | 118,281.60 | 53,605.70 |
| Ministry of Industry and Works/Bureau of Standards | 315,482.11 | 145,525.25 | 169,956.86 | 77,025.14 |
| Ministry of Labour and Social Security | 82,295.14 | 47,931.84 | 34,363.30 | 15,573.58 |
| Ministry of Education – Head Office | 209,750.11 | 121,042.94 | 88,707.17 | 40,202.45 |
| Office of the Prime Minister – Jamaica House | 102,108.60 | 58,395.74 | 43,712.86 | 19,810.84 |
| St. Andrew High School | 179,510.76 | 82,814.69 | 96,696.07 | 43,823.05 |
| Moneague Teachers College | 150,287.81 | 37,154.30 | 113,133.50 | 51,272.56 |
| Constant Spring Police Station | 126,336.24 | 47,796.48 | 78,539.76 | 35,594.54 |
| Central Sorting Office | 137,266.27 | 51,964.70 | 85,301.57 | 38,659.02 |
| Ministry of Youth Culture (VTDI) | 112,270.75 | 43,027.78 | 69,242.98 | 31,381.20 |
| Heart Trust – Runaway Bay | 131,623.54 | 49,015.30 | 82,608.24 | 37,438.39 |
| Montego Bay Community College | 211,780.80 | 79,504.13 | 132,276.67 | 59,948.32 |
| Elect. Advisory Comm., Elect. Office of Jamaica | 113,778.72 | 54,261.50 | 59,517.22 | 26,973.44 |
| Jamaica Information Service | 121,920.48 | 49,769.28 | 72,151.20 | 32,699.22 |
| JDF New Castle | 207,621.00 | 52,410.24 | 155,210.76 | 70,342.14 |
| KSAC Church Street | 46,772.35 | 23,367.17 | 23,405.18 | 10,607.32 |
| Jamaica College – Hope Road | 96,253.34 | 38,519.71 | 57,733.63 | 26,165.12 |
| Sir John Golding Rehabilitation Centre | 96,192.00 | 35,717.76 | 60,474.24 | 27,407.17 |
| Nurse’s and Doctor’s Quarters | 81,311.04 | 27,609.12 | 53,701.92 | 24,337.93 |
| JCF - Mobile Reserve Facility | 225,979.20 | 78,278.40 | 147,700.80 | 66,938.60 |
| Blood Bank; National Public Health Laboratory | 166,881.60 | 69,258.24 | 97,623.36 | 44,243.30 |
| TAJ - May Pen Revenue Services Centre | 93,028.32 | 41,196.67 | 51,831.65 | 23,490.31 |
| St. James Resident Magistrate's Court | 25,757.16 | 11,084.83 | 14,672.33 | 6,649.56 |
| Hanover Resident Magistrate's Court | 31,111.08 | 14,024.74 | 17,086.34 | 7,743.60 |
| Court of Appeal | 59,051.52 | 26,253.22 | 32,798.30 | 14,864.32 |
| Corporate Area Criminal Court | 85,250.88 | 38,469.02 | 46,781.86 | 21,201.73 |
| Manchester Resident Magistrate's Court | 12,685.20 | 5,214.53 | 7,470.67 | 3,385.74 |
| Corporate Area Civil Court | 105,739.92 | 37,692.86 | 68,047.06 | 30,839.20 |
| The Supreme Court of Jamaica | 106,065.96 | 46,319.33 | 59,746.63 | 27,077.42 |
| St. Catherine Resident Magistrate's Court | 34,768.80 | 16,720.70 | 18,048.10 | 8,179.47 |
| Bellevue Hospital | 399,556.80 | 149,731.20 | 249,825.60 | 113,221.97 |
| National Chest Hospital | 125,193.60 | 45,722.88 | 79,470.72 | 36,016.45 |
| Stadium National Indoor Sports Centre | 53,716.61 | 16,169.47 | 37,547.14 | 17,016.51 |
| Stadium National Arena | 66,718.08 | 24,330.24 | 42,387.84 | 19,210.34 |
| Stadium National Stadium | 342,315.07 | 110,911.68 | 231,403.39 | 104,872.95 |
| Stadium Stadium East | 67,605.12 | 18,754.56 | 48,850.56 | 22,139.27 |
| Stadium Aquatic Centre | 16,334.21 | 6,462.72 | 9,871.49 | 4,473.80 |
| Stadium Outdoor Netball Court | 37,224.00 | 8,933.76 | 28,290.24 | 12,821.25 |
| Stadium Outdoor Basketball Court | 8,712.00 | 2,090.88 | 6,621.12 | 3,000.72 |
| Stadium Almond Walk | 3,868.13 | 2,803.68 | 1,064.45 | 482.41 |
| CF TCND, 230 Spanish Town Rd | 529,545.60 | 232,588.80 | 296,956.80 | 134,582.02 |
| CF Transport Management Maintenance Division, Swallowfield | 203,567.04 | 84,792.96 | 118,774.08 | 53,828.89 |
| CF Norman Manley Police Station | 110,053.44 | 46,483.20 | 63,570.24 | 28,810.29 |
| CF May Pen | 107,136.00 | 42,508.80 | 64,627.20 | 29,289.31 |
| SERHA Bustamante | 88,413.60 | 36,284.16 | 52,129.44 | 23,625.27 |
| TOTAL | 5,890,761 | 2,402,549 | 3,488,212 | 1,580,871 |

4.1.2	CO2 emissions reductions

As with any other energy project, the projects aimed at improving consumption EE are also characterized by environmental externalities, such as reduction of CO2 emissions.

The annual CO2 emissions reduction were determined based on the estimated annual energy savings (expressed in kWh) and converted into ton CO2 (using an emission factor of 0.000637 from IEA 2015). As shown in table below it is expected that the implementation of the EE measures will reduce CO2 in 9,214 Ton per year.

Table 4.3 CO2 savings in each building.

| Buildings | Annual CO2 emissions (ton CO2) |
| --- | --- |
| Kingston Public and Victoria Jubilee Hospital | 2,167 |
| Cornwall Hospital | 1,251 |
| HEART Trust Ebony Park Academy | 473 |
| Falmouth Hospital | 165 |
| Mandeville Hospital | 286 |
| Marcus Garvey School | 66 |
| TOTAL | 4,407 |

For the lighting project, the CO2 emissions reduction is approximately 2,220 CO2 tons per year.

Please note that a monetary valuation of CO2 emission reductions results were not quantified for the base case scenario and used in the EIRR and ENPV calculation.

4.1.3	Other savings/benefits

It is worth mentioning that job retention/creation can be seen as an indirect benefit of the recommended EE measures. However, such co-benefit is beyond the scope of this analysis.

4.2	Economic cost

The economic costs include all investments or non-recurring costs as well as all operation and maintenance costs needed to implement the EE measures during the life of the project.

The investment cost of each measure was estimated including fixed and variable costs i.e. based on quantities and respective cost per unit and labor per unit (expressed in US$), as applicable. It is assumed that all EE measures are implemented as a single construction project and, therefore, they occur in the beginning of Year 1. No additional annual flows of investment and/or non-recurring costs are foreseen for the life of the project.

Table below shows a summary of the total investment cost estimated per building. These costs include only the incremental costs resulting from the implementation of the EE measures.

Table 4.4 Total investment costs.

| Buildings | Total measures Cost (US$) |
| --- | --- |
| Kingston Public and Victoria Jubilee Hospital | 4,800,000 |
| Cornwall Hospital | 1,988,338 |
| HEART Trust Ebony Park Academy | 624,994 |
| Falmouth Hospital | 206,996 |
| Mandeville Hospital | 696,891 |
| Marcus Garvey School | 198,323 |
| TOTAL | 8,515,542 |

As regards the operation and maintenance costs they are lower than the BAU scenario therefore negligible.

In the lighting project, the total costs of the retrofits are US$2,354,853 /8/.

4.3	Economic feasibility

Once costs and benefits have been identified and valued, the economic feasibility of the project (s) shall be discussed. There are three main criteria:

- Economic Net Present Value (ENPV) compares all costs with benefits, with all resource flows discounted by the agreed discount rate.

- The Benefit-Cost Ratio (BCR) is the ratio of the sum of benefits (in present value) to the sum of costs (in present value). Typically if this ratio is more than one, using this criterion the project would be approved.

- The Economic Internal Rate of Return (EIRR) is defined as the discount rate that causes the ENPV to be zero. Typically if the EIRR is larger than the recommended 12% discount rate the project would be approved.

For the project that deals with the EE retrofitting in six buildings, the economic indicators of the base case scenario are an ENPV of $3,014,513 and an EIRR of 19%.

Note that the base case scenario does not include a monetary valuation of the CO2 emissions reduction (4,407 tons/year). If it were included assuming US$37/ton as benefit – the assumption used in the fuel efficiency component, the EIRR would increase to 20.8% and the ENPV would increase to US$3,899,855.

For the project that deals with the lightning retrofitting, the economic indicators of the base case scenario are an ENPV of $5,005,159 and an EIRR of 67%.

Note that the base case scenario does not include a monetary valuation of the CO2 emissions reduction (2,220 tons/year). If it were included assuming $37/ton as benefit – the assumption used in the fuel efficiency component, the EIRR would increase to 70.4% and the ENPV would increase to US$5,350,278.

Figures below show the EIRR, ENPV and BCR calculation showing the benefit and costs streams used and the economic return estimations for the implementation of the EE measures in the 6 public buildings under investigation and the lighting project.

Figure 4.1 Six buildings investment and return graph.

For the lighting project, considering the base case, the investment and returns graph is as follows:

Figure 4.2 - Building lighting retrofit investment and return graph.

4.4	Sensitive analysis

This section contains a discussion of the feasibility analysis of the project. A sensitivity analysis was done for two changes: (i) increase/decrease 20% of the investment costs and (ii) increase/decrease 20% of the electricity price. EIRR and ENPV needs to be recalculated for changes in key variables that could affect project costs, benefits and assumptions. There have been considered scenarios of an increase or decrease of the capital costs of ± 20%, and the impacts of these have been evaluated with regards of the ENPV and IRR.

Considering the investments to be performed in the assets, the ENPV remains positive in all the scenarios considered.

For the EE retrofitting, the sensitivity analysis shows the following:

Table 4.5 Base case results and sensitivity analysis of 6 buildings.

|  | ENPV Sensitivity |  |  | EIRR Sensitivity |  |
| --- | --- | --- | --- | --- | --- |
|  | Discount rate US$ | Electricity price US$ | Investment costs US$ | Electricity price | Investment costs |
| Minimum (-20%) | 4,650,337 | 1,000,899 | 4,425,225 | 14% | 24% |
| Base value | 3,014,513 | 3,014,513 | 3,014,513 | 19% | 19% |
| Maximum (+20%) | 1,739,862 | 5,028,128 | 1,603,801 | 23% | 15% |

Figure 4.3 NPV Sensitivity analysis for 6 buildings.

Figure 4.4 Sensitivity analysis of EIRR 6 buildings

Regarding the lighting project, the result of the sensitivity analysis are the following:

Table 4.6 Results of the sensitivity analysis lighting buildings.

|  | ENPV Sensitivity |  |  | EIRR Sensitivity |  |
| --- | --- | --- | --- | --- | --- |
|  | Discount rate US$ | Electricity price US$ | Investment costs US$ | Electricity price | Investment costs |
| Minimum | 6,156,206 | 3,668,900 | 5,340,387 | 53% | 84% |
| Base value | 5,005,159 | 5,005,159 | 5,005,159 | 67% | 67% |
| Maximum | 4,090,288 | 6,341,419 | 4,669,932 | 80% | 56% |

Figure 4.5 Sensitivity analysis of EIRR for Lighting project

IRR rate is remaining positive in all the scenarios, and above the stated 12%, demonstrating robustness of the economic viability of the project.

V.	CONCLUSIONS

With respect to the buildings component, two economic evaluations were made. The first one comprise the energy technical assessment performed to a defined group of 6 public buildings (schools, hospitals) for improving their energy characteristics as a representation of the projects that will be finance. The benefits consisted of electricity savings (direct effect) and reduction of CO2 emissions (externality). The costs consist mainly on the investment cost of the different recommended measures (O&M costs are lower than business as usual therefore irrelevant). The more relevant assumptions of the analysis are: (i) the economic life of each measure vary from 5 to 20 years; the electricity prices are based on an average of historical costs ($USD/kWh) per building without considering inflation; annual escalation rate: 0.29% for the first 5 years; 2.18% for the remaining years (based on WTI crude forecast growth rate).

The results for the base case show an EIRR of 19% and the ENPV is US$3,014,513. A sensitivity analysis was performed to test the impact of significant changes to investment costs and the electricity price. The project showed to be economically robust.

The project is also characterized by environmental externalities such as reduction of CO2 emissions. It is expected that the implementation of the EE measures will reduce CO2 emissions in 9,214 Ton per year. If it were included assuming US$37/ton as benefit – the assumption used in the fuel efficiency component II of the loan, the EIRR would increase to 20.8% and the ENPV would increase to US$3,899,855.

The second economic evaluation was for the investment for lighting retrofit. The results for the base case show an EIRR of 67% and the ENPV is US$5,005,159. A sensitivity analysis was also performed to test the impact of significant changes to investment costs and the electricity price and the project showed to be economically robust.

VI.	REFERENCE

| /1/ | DNV GL 2016, Energy Sector Diagnosis Study Jamaica, Support for the Development of a comprehensive Energy Planning Program for Jamaica, report to the Inter-American Development Bank, Report No.: 10004931-ESZA-R-01, Rev E. |
| --- | --- |
| /2/ | DNV GL 2016, Kingston Public and Victoria Jubilee Hospitals Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /3/ | DNV GL 2016, Ebony Heart Trust Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /4/ | DNV GL 2016, Falmouth Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /5/ | DNV GL 2016, Marcus Garvey High School Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /6/ | DNV GL 2016, Mandeville Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /7/ | DNV GL 2016, Cornwall regional Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /8/ | FACILITY DATA LIGHTING LOTS Aug 24.xlsx agreed between MSET and DNV GL. |

ABOUT DNV GL

Driven by our purpose of safeguarding life, property and the environment, DNV GL enables organizations to advance the safety and sustainability of their business. We provide classification and technical assurance along with software and independent expert advisory services to the maritime, oil and gas, and energy industries. We also provide certification services to customers across a wide range of industries. Operating in more than 100 countries, our 15,000 professionals are dedicated to helping our customers make the world safer, smarter and greener.

CBA COMPONENT II – FUEL EFFICIENCY

Abbreviations

CAPEX		Capital Expenditure

CBA		Cost Benefit Analysis

CBR		Cost Benefit Ratio

CEA		Cost Effectiveness Analysis

CO		Carbon monoxide

CO2		Carbon dioxide

CVD		Clean Vehicles Directive

DK		Denmark

DOT		Department of Transportation

EFA		European Free Alliance

EREV		Electric Vehicles with a Range Extender

ERR		External Rate of Return

EV		Electric Vehicle

EXW		ExWorks

FEV		Full Electric Vehicle

FR		France

g		gram

gal		Gallon

GIZ		Deutsche Gesellschaft für Internationale Zusammenarbeit (GIZ) GmbH

GKA		Greater Kingston Area

GTZ		Deutsche Gesellschaft für Technische Zusammenarbeit (GTZ) GmbH

ICE		Internal Combustion Engine

IDB		Inter-American Development Bank

IT		Italy

ITS		Intelligent Transportation Systems

JMD		Jamaican Dollar

kg		Kilogram

lb		Pound

LCV		Light Commercial Vehicle

MCV		Medium Commercial Vehicle

NPV		Net Present Value

NL		The Netherlands

NOx		Nitric Oxide and Nitrogen Dioxide

NWA		National Works Agency

OLC		Operational Lifetime Cost

OPEX		Operational Expenditure

PC-S		Small Passenger Car

PC-M		Medium Passenger Car

PC-L		Large Passenger Car

PHEV		Plug-in Hybrid Electric Vehicles

PL		Poland

PM10		Particulate Matter 10 micrometers or less in diameter

PM2.5		Particulate Matter 2.5 micrometers or less in diameter

PrT		Private Transport

PuT		Public Transport

SOP		Standard Operational Procedures

SUTP		Sustainable Urban Transport Project

SUV		Sport Utility Vehicle

SWE		Sweden

THC		Totally Hydrocarbons

UK		United Kingdom

US$		United States Dollar

US$ M		Million United States Dollar

UTMS		Urban Traffic Management System

VOC		Volatile Organic Compounds

- INTRODUCTION

The objective of the present project component is the improvement of energy efficiency in urban mobility in the Greater Kingston Area. The goals will be achieved through the implementation and operation of Intelligent Transportation Systems (ITS) that will coordinate traffic flows over several corridors.

If the project is financed, the economic, social and environmental negative impact of private and public urban transport will decrease, benefiting the population of Kingston, Spanish Town, Portmore, as well as visitors from other parts of the country and abroad through improved travel times, less fuel consumption, and thereby less air pollution and CO2 emissions. In addition, the energy imports of Jamaica will reduce.

If the project is not financed a classic “do nothing scenario” will be created, meaning that the traffic congestion will further grow, as will costs of mobility and its environmental and social impact, leading to poorer life quality in the Kingston area.

The project will build on the existing traffic management infrastructure, institutions and personnel. The 3 main and most critical component are:

- Communication & centralization: The integration of currently deployed technologies through a communications network into one centralized urban traffic management and control systems (UTMS) with one centralized ITS management platform.

- Sensors & CCTV: Deployment of traffic sensors and cameras to allow permanent monitoring as well as continuous generation of traffic data for strategic and tactical traffic planning.

- Institutional strengthening: Training and coaching of personnel for planning and operation, development of operational procedures and provision of tools for maximum usage of all the technologies.

All three components are key for the achievement of project goals and also co-depend on each other.

For example:

- Without communications there is no central control and coordination of the traffic lights. Without coordination, the project goals will not be achieved.

- Without sensors there is no effective way of monitoring the traffic to take decisions in real time. Without traffic data there is no effective way to do traffic planning and the project goals will not be achieved.

- Without trained personnel there will be no effective use of existing and deployed technologies. Without methods and tools, planning processes will take long, will be inaccurate, and the project goals will not be achieved.

There are 2 reasons why this project the best alternative to meet the stated objective:

- Real time traffic coordination through the development of the typical 5-step traffic management system

- The highest impact per investment

The typical development of traffic management is as follows:

- Step 1: Fix time (local control)

- Step 2: Fix time with coordination (central control)

- Step 3: Coordination with traffic actuation and optional public transit priority (central control)

- Step 4: Traffic adaptive (central control)

- Step 5: Traffic predictive (central control & predictive model)

Kingston is currently at step 1 having only few centralized intersections. Not understanding that the technological and institutional elements are linked, taking too many steps at one time for one task, and not assuring synchronization between technology and institutions, often leads to discrepancies and future abandonment of the systems. (Figure 1)

Figure 1 - Outcome of technology projects in ITS

As of the benefits, the biggest benefits are always achieved between step 1 and step 2 of traffic management systems development, as traffic starts to move smoothly across many intersections. Later steps require much higher effort in investment as well as operation and planning and do not achieve the same ratio of benefit versus cost.

To minimize the risk of not obtaining and maintaining the project benefits due to discrepancies in technological and institutional developments as well as in search of an optimum benefit – cost ratio one step at the time will be taken, implementing a centralized and coordinated traffic control scheme.

All technology projects have technological and institutional risks. The technological risks are being addressed by:

- Design: Good and open technical specifications in the tender document, that assure key turn system implementation.

- Update: Update of the tender document shortly before tendering, taking into account that between the initial tender design and the actual tender process, there might be changes in the currently deployed systems, technology development and other starting conditions.

- Support: Qualified supervision of the contractors during installation, training and coaching.

There are also institutional risks which are mostly related with lack of training of the personnel of the future operation of the system. This is why these risks are being addressed by:

- Competences: Inclusion of training and coaching in planning and operation.

- Tools: Provision of state of the art modelling tools that integrate with currently existing planning tools to allow easy and fast response to all planning needs constructing and existing methodological experience.

- One step approach: Taking one step of the technological and methodological development ladder in urban traffic management at the time, avoiding an overload of learning and working at the same time.

The concept of coordinated traffic light management is well proven for decades and in hundreds of cities. Implementations are far spread in many countries around the globe. Each implementation has specific objectives seeking benefits, such as increase in traffic handling capacity of roads, reduction of collisions and wait time for both vehicular and pedestrian traffic , encouragement of travel within the speed limit to meet green lights in a green wave, reduction of unnecessary stopping and starting of traffic - this in turn reduces fuel consumption, air pollution, noise and vehicle wear and tear (present project), improvement of overall journey time and reduction of  driver frustration and 'road rage'; just to name some.

The present report is being delivered as an input to the POD, DEM and Result Matrix.

- ASSUMPTIONS AND METHODOLOGY

The alternatives for traffic management improvements are based on the above mentioned development steps, all starting with the current situation (Step 1 or do nothing), not taking into account a traffic predictive system at this time, as its successful deployment is still on the very high end of solutions for its extremely high cost and complexity.

Figure 2 - Traffic management change alternatives

Looking at an estimate per intersection budget, only focusing on intersection controller, intersection communications and intersection sensing (Figure 3), it is clear that a CAPEX analysis divides the development steps in 3 groups:

- Group 1: Steps 1 & 2

- Group 2: Steps 3 & 4

- Group 3: Step 5

Figure 3 - Per intersection telematics control hardware CAPEX

Doing a more detailed CAPEX analysis the difference between groups approximately doubles (Table 1), suggesting that Alternative 1 will have a much lower CAPEX than Alternatives 2 or 3.

Table 1 - Per intersection telematics control hardware CAPEX

The improvements are based on the implementation of a so called “progressive green wave” that coordinates the existing traffic lights in a manner that will reduce involuntary stops as is illustrated through time – space diagrams in Figure 4 and Figure 5.

Figure 4 - Uncoordinated traffic lights

Figure 4 shows that the lack of synchronization causes stops that either form congestion for following vehicles or make vehicles wait for the next green. This makes that the traffic flow is far below the theoretical capacity of the road.

Figure 5 - Coordinated traffic lights

Figure 5 shows how a simple progressive coordination in one direction reduces the number of stops, allowing the vehicles to go longer ways more fluidly. This makes that the traffic flow can approach the theoretical capacity of the road.

To enable this kind of traffic coordination, traffic controller hardware upgrades, a robust communications network and central monitoring software are being implemented, together with institutional strengthening in the areas of traffic planning. Sensors and cameras for real time traffic monitoring and variable message signs for route decision complement the solution.

The currency used is United States Dollars. This is because the ITS component 1) is designed to be an international bid and 2) consists nearly completely of technology (Table 2), which is traded internationally in US Dollars.

Table 2 - Current CAPEX division

Item prices for the project component CAPEX are based on international source values (manufacturer ExWorks outlet) and calculated like this:

- Technology components (hardware): Including acquisition, transport, nationalization, installation, connection and testing of the hardware in Jamaica. Most peripheral components items include the cost for minor materials and wirings.

- Technology components (software): Including acquisition and installation of the software in Jamaica.

- Fiber optic cable: Including acquisition, local transport and installation.

- Services: Fees for personnel/trainers, travel expenses and course materials.

- Minor provisions: Including acquisition, transport, nationalization and delivery of components.

Being all traffic lights low consumption LED technology, consuming between 5 and 8 Watts per light, for OPEX calculation an average power consumption of 0.5 kW per intersection was chosen. The cost of the kWh was taken from the Jamaica Public Service Limited Website at 40 JMD/kWh.

For fuel savings calculations, the current published (August 4th, 2016) price for Gasoline 87 (JMD 103.4128 per liter) from PETROJAM was used (Chapter , page 39).

For GHG savings calculation a cost of USD 37.00 per metric ton of CO2 was chosen.

The exchange rate from Jamaican Dollar to US Dollar (August 4th, 2016) was taken from OANDA (124.936 JMD / USD) (Chapter , page 42).

The present discount rate used is 12%.

As key aspect, the methodology is based on 3 logical blocks (Figure 6), taking only into account differences, meaning that everything that is maintained untouched by the project is not taken into the equation.

Things that will be maintained are organizational structure, costs for personnel, offices, and administration of NWA. The new lift truck will replace an existing one, so the monthly operational costs should be maintained or could even diminish, so not taking that into account will make all estimations more conservative.

Figure 6 - Typical 3 block ITS project analysis

Also two CAPEX are being calculated, one (CAPEX 1 or CAPEX new) only taking into account the budget of the IDB project, showing the benefits per US Dollar spent in the current project and another one (CAPEX 2 or CAPEX existing) taking into account the IDB project budget plus an estimate valuation of existing technological and civil infrastructure giving an overall impression of a wider cost benefit analysis, and putting CAPEX 1 into a more integrated perspective.

As a result, the economic analysis will be focused on the following components:

- CAPEX 1: Cost to put the project in place

- CAPEX 2: Cost to put the project in place plus an estimate value of the existing technological and civil infrastructure.

- OPEX: Power consumption of additional technological components.

- Benefits: Based on the sustainability triangle (Figure 7) the analysis includes savings in fuel consumption (economic impact), savings in pollutant emissions (environmental impact) and savings in travel time (social impact).

Figure 7 - Sustainability Triangle

All costs (CAPEX, OPEX) were calculated in Excel; the benefits were calculated based on the results of dynamic traffic model of the Greater Kingston Area, using the Aimsun EXPERT Version 8 dynamic multilevel traffic simulation model.

For the analysis the differences between the following scenarios were calculated:

- Baseline scenario: Scenario without project or “do nothing”.

- Future scenario: Scenario with project.

To calculate 1) travel times and 2) fuel consumption that were also the basis for emissions (GHG) calculations, a hybrid (mesoscopic – microscopic) dynamic traffic model of the Greater Kingston Area was developed and both scenarios were run and compared.

For the model a series of corridors were chosen based on 1) their relevance and 2) on the availability of reliable and up to date (2015, 2016) traffic data. Corridors that complied with both conditions were simulated with high detail on a microscopic level, while the rest of the network was simulated with minor resolution on a mesoscopic level. Both microscopic and mesoscopic portion of the model were taking into account driver behavior and vehicle dynamics such as acceleration and deceleration.

The Fuel Consumption Model (See Fuel consumption model, Page 44) assumes that each vehicle is either idling, or cruising at constant speed or accelerating or decelerating. The state of each vehicle in the network is determined and the model then uses the appropriate formula to calculate the fuel consumed for this state. As standard vehicle for all model calculation a sedan vehicle was chosen, consuming 0.333 ml/s in idle, 4.7 l/100km at 90 km/h, 6.5 l/100km at 120 km/h and 50 km/h as speed where the fuel consumption rate, in ml/s, is at a minimum for the vehicle cruising at constant speed.

Figure 8 - Logical development of traffic model

Being:

- Infrastructure: Road network, intersections, usage rules like speeds and traffic types

- Assignment: Current signal timing of the signalized intersections, turn relations.

- Demand: Traffic count data.

- Base line model: Model of current situation (Kingston without ITS Project).

- Optimization: Signal timing optimization, traffic light coordination,

- Future scenario: Modeling of the future scenario (Kingston with ITS Project)

- Impact analysis: Comparison of current and future scenario, fuel consumption, some environmental indicators.

For all model outputs, such as fuel consumption and travel time, the following expansion factors were used:

- Work day traffic equals 10 times peak hour traffic, making the expansion factor from peak hour to work day 10

- Work week equals 5 work days plus 2 weekend days, Weekend day traffic is calculated at 50% weekday traffic making the expansion factor from workday to week 6

- Every year has 52 weeks, making the expansion factor from week to year 52.

Figure 9 - Expansion factor for model outputs

For emissions calculations the expansion factors were calculated based on Average Annual Emissions and Fuel Consumption for Gasoline-Fueled Passenger Cars and Light Trucks (Table ), published by the United States Environmental Protection Agency, resulting in the following CO2 emissions for a standard passenger car per liter of fuel burned:

Table 3 - Emissions calculation conversion table

The analysis period is 10 years.

- ECONOMIC BENEFITS

The prime objectives of the present project component are:

- Economic benefits: Reduction of fuel consumption

- Environmental benefits: Reduction of greenhouse gases

- Social benefits: Reduction of travel time

Fuel consumption in peak hour is a direct model output in liters per hour [l/h], which is then expanded to daily, weekly and yearly fuel consumption. Using the above established conversion factors the yearly fuel consumption is converted from liters to United States Dollars. The savings are the difference between the two scenarios (Table 4, Table 5).

Table 4 - Fuel savings in liters

Table 5 - Fuel savings in USD

GHG emissions were calculated based on the fuel consumption using the above established conversion factors and then converted from kg to United States Dollars per year. The savings are the difference between the two scenarios (Table 6, Table 7).

Table 6 - GHG savings in kg

Table 7 - GHG savings in USD

Travel time in peak hour is a direct model output in liters per hour [h/h], which is then expanded to daily, weekly and yearly travel time. Using the above established conversion factors the yearly fuel consumption is converted from hours to United States Dollars. The savings are the difference between the two scenarios (Table 8, Table 9).

Table 8 - Travel time savings in hours

Table 9 - Travel time savings in USD

- ECONOMIC COSTS

The identified costs are:

- CAPEX 1: Cost to put the project in place

- CAPEX 2: Cost to put the project in place plus an estimate value of the existing technological and civil infrastructure.

- OPEX: Power consumption of additional technological components.

The CAPEX 2 analysis includes past infrastructure and technology investments that were included at a current approximate purchase value.

The total budget for present component is 3.5 M US$ (Table ).

The existing infrastructure and technology value estimate, itemized by control center, communications and intersections is approximately 9.97 M US$ (Table 10, Table 11, Table 12). Summing existing infrastructure and technology and project budget for the present component the CAPEX 2 value is approximately 13.47 M US$ (Table 13).

Table 10 - CAPEX 2 for existing control center infrastructure and technology

Table 11 - CAPEX 2 for existing communications infrastructure and technology

Table 12 - CAPEX 2 for existing intersection infrastructure and technology

Table 13 - CAPEX 2

As additional components include only media converters, interfaces, memory modules and sensors, the additional power consumption per intersection should be 100 Watts or less. Based on the above established conversion factors the per intersection power consumption is calculated for the total of 161 intersections and then converted from kw/h to US$ per year (Table 14, Table 15).

Table 14 - OPEX in kWh

Table 15 - OPEX in USD

- ECONOMIC RETURN

The economic return was calculated for both, CAPEX 1 and CAPEX 2 calculating the net present value (NPV) based on the following formula:

Equation 1 - Net Present Value (NPV) calculation

Where:

I0	initial investment for the proposed practices

Bi	benefits received during year i, measured as the revenue from selling the output at market prices and value added or avoided losses

Ci	production costs during year i, include expenses related labor, inputs and maintenance

r	discount rate (12% for this exercise)

As the implementation will take place over year 1 and 2 of the project, benefits are expected to behave in a typical ITS impact pattern representing the initial learning effect and behavior change that lead to an initial increase and traffic flow increase that diminishes the benefits over time.

Table 16 - Typical ITS project impact development

Figure 10 - Typical ITS project impact development

For CAPEX 1, only IDB investment the following table resumes the NPV / ERR analysis.

Table 17 - NPV / ERR calculation for 10-year period and CAPEX 1

For CAPEX 2, IDB investment and existing infrastructure and technology the following table resumes the NPV/ERR analysis.

Table 18 - NPV / ERR calculation for 6-year period and CAPEX 2

In either calculations, the project is economically feasible and implementation is recommended.

- SENSITIVITY ANALYSIS

A sensitivity analysis was conducted to assess the variation of benefits with regards to: a) including investments already made by the NWA crucial for the project (CAPEX 2), b) changes in fuel prices, c) changes in travel time savings, and d) a time delay to achieve the project benefits.

- By considering the investments made by the NWA, the CAPEX increases from US$3.5 million (CAPEX 1) to US$13.47 million (CAPEX 2). This increase reduces the ERR from 43% to 28%, respectively.

- The project benefits are marginally sensitive to fuel price changes. Reducing fuel prices by 20%, it decreases the benefits in about 4%. The ERR is 39% with CAPEX 1, and 24% with CAPEX 2. Increasing fuel prices by 20%, it produces an additional benefit of about 2%. The ERR is 45% with CAPEX 1, and 30% with CAPEX 2.

- Changing travel time savings by 20% give the same results as changing fuel prices 20%. That is, reducing travel time savings by 20% decrease the benefits in about 4% for both CAPEX, and increasing travel time savings by 20% increases the benefits by 2%, also for both CAPEX.

- Finally, if the benefits take longer to materialize, 1 or even 2 years, ERR with CAPEX 1 reduces to 39% and 36% respectively, whilst ERR with CAPEX 2 reduces to 25% and 23% respectively.

In summary, the sensitivity analysis performed to the cost-benefit analysis of the transport component, that considers changes in CAPEX, fuel prices, travel time savings, and a delay in materializing benefits, shows that the ERR varies between 23% and 45%, from the original estimation of 43%.

Below we present the tables with the results for each of the variables changed in the sensitivity analysis.

### CAPEX

- CBA projections with CAPEX 1 (only project investment)

- CBA projections with CAPEX 2 (project investment + NWA investment)

|  | ERR CAPEX 1 | ERR CAPEX 2 |
| --- | --- | --- |
| CBA | 43% | 28% |

Table 199 - ERR for CAPEX 1 and CAPEX 2 (CBA)

Sensitivity analysis to changes in CAPEX: By considering the investments already made by the NWA, the CAPEX increases from US$3.5 million (CAPEX 1) to US$13.47 million (CAPEX 2). This increase reduces the ERR from 43% to 28%.

### Fuel price changes

Since Jamaica imports all its fuel from abroad, the price of fuel responds to changes in international oil prices, which have varied greatly for the past 3 years, reducing from an average of US$100 per barrel in 2013, to US$45 dollars in July 2016. During the past year oil prices have somewhat stabilized, varying between US$35 and US$50 per barrel. It is therefore safe to assume that in the short and medium term oil prices could reduce up to 20%. The sensitivity analysis also considers an increase of 20% to be consistent with the reduction, however this variation is very conservative, as it is expected that in the medium to long run the price of oil will achieve a greater annual increase.

|  | ERR CAPEX 1 | ERR CAPEX 2 |
| --- | --- | --- |
| CBA | 43% | 28% |
| Fuel price -20% | 39% | 24% |
| Fuel price +20% | 45% | 30% |

Table 200 - ERR for fuel price changes

Sensitivity analysis to changes in fuel prices: The project benefits are marginally sensitive to fuel price changes. Reducing fuel prices by 20% decreases the benefits in about 4%. The ERR is 39% with CAPEX 1, and 24% with CAPEX 2. Increasing fuel prices by 20% produces an additional benefit of about 2%. The ERR is 45% with CAPEX 1, and 30% with CAPEX 2.

### Travel time savings changes

The impact of changes in travel time savings are equivalent to changes in fuel prices, since they have the same mathematical relationship in the model. Fuel consumption percentage changes are equivalent to fuel price percentage changes and currency exchange rate percentage fluctuations. The result of travel time changes will result in corresponding fuel consumption and cost changes.

|  | ERR CAPEX 1 | ERR CAPEX 2 |
| --- | --- | --- |
| CBA | 43% | 28% |
| Travel time savings -20% | 39% | 24% |
| Travel time savings +20% | 45% | 30% |

Table 211 - ERR for travel time savings changes

Sensitivity analysis to changes in travel time savings: The project benefits are marginally sensitive to travel time savings changes. Reducing travel time savings by 20% decreases the benefits in about 4%. The ERR is 39% with CAPEX 1, and 24% with CAPEX 2. Increasing travel time savings by 20% produces an additional benefit of about 2%. The ERR is 45% with CAPEX 1, and 30% with CAPEX 2.

### Impact delay changes

The project benefits could potentially take longer to materialize, for instance due to delays in the bidding process, the delivery of the imported equipment, training program, etc. Therefore a sensitivity analysis was conducted to assess the impact of a 1 and 2 year delay in the realization of the benefits.

|  | ERR CAPEX 1 | ERR CAPEX 2 |
| --- | --- | --- |
| CBA | 43% | 28% |
| 1-year benefit delay | 39% | 25% |
| 2-year benefit delay | 36% | 23% |

Table 222 - ERR for impact delay

Sensitivity analysis to changes in realization of project benefits: Finally, if the benefits take longer to materialize, 1 or even 2 years, ERR with CAPEX 1 reduces to 39% and 36% respectively, whilst ERR with CAPEX 2 reduces to 25% and 23% respectively.

- ADDITIONAL ANALYSIS

The analysis was based on two investment scenarios, CAPEX 1 and CAPEX 2.

- CONCLUSIONS

The Cost Benefit Ratio of the project is impressively high. This is due to the combination of a series of factors, such as: 1) the availability of an important amount of infrastructure allowing the present transaction to concentrate on the completion of missing links (communication, peripheral devices, software, and institutional strengthening) not having to build a project from cero, and 2) the fact that stepping up from a fix time uncoordinated traffic management scheme to a coordinated scheme is of low investment and high impact.

The sensitivity analysis performed, that considers changes in CAPEX, fuel prices, travel time savings, and a delay in materializing benefits, shows that the ERR varies between 23% and 45%, from the original estimation of 43%.

It is recommended to do the implementation of this operation.

- References

Figures

Figure 1 - Outcome of technology projects in ITS 	6

Figure 2 - Traffic management change alternatives	8

Figure 3 - Per intersection telematics control hardware CAPEX	8

Figure 4 - Uncoordinated traffic lights	9

Figure 5 - Coordinated traffic lights	10

Figure 6 - Typical 3 block ITS project analysis 	12

Figure 7 - Sustainability Triangle	12

Figure 8 - Logical development of traffic model	14

Figure 9 - Expansion factor for model outputs	15

Figure 10 - Typical ITS project impact development	19

Figure 11 -  USD / JMD exchange rate 	26

Tables

Table 1 - Per intersection telematics control hardware CAPEX	9

Table 2 - Current CAPEX division	10

Table 3 - Emissions calculation conversion table	15

Table 4 - Fuel savings in liters	15

Table 5 - Fuel savings in USD	16

Table 6 - GHG savings in kg	16

Table 7 - GHG savings in USD	16

Table 8 - Travel time savings in hours	16

Table 9 - Travel time savings in USD	16

Table 10 - CAPEX 2 for existing control center infrastructure and technology	17

Table 11 - CAPEX 2 for existing communications infrastructure and technology	17

Table 12 - CAPEX 2 for existing intersection infrastructure and technology	17

Table 13 - CAPEX 2	18

Table 14 - OPEX in kWh	18

Table 15 - OPEX in USD	18

Table 16 - Typical ITS project impact development	19

Table 17 - NPV / ERR calculation for 10-year period and CAPEX 1	20

Table 18 - NPV / ERR calculation for 6-year period and CAPEX 2	20

Table 19 - Jamaican petroleum product prices 	24

Table 20 - Jamaican petroleum product prices 	25

Table 21 - Average emissions and fuel consumption for passenger cars 	26

Table 22 - Current version of detailed project reference budget	28

Equations

Equation 1 - Net Present Value (NPV) calculation 	18

- ANNEXES

Petroleum product prices

Table 23 - Jamaican petroleum product prices

Table 24 - Jamaican petroleum product prices

Emissions calculations

Table 25 - Average emissions and fuel consumption for passenger cars

Currency exchange rate

Figure 11 -  USD / JMD exchange rate

Reference budget

Table 26 - Current version of detailed project reference budget

Fuel consumption model

XI	APPENDIX A.

# A.1	Analysis

## I.1	Scenarios

The following scenarios were analyzed:

- CBA projections with CAPEX 1 (only project investment)

- CBA projections with CAPEX 2 (project investment + NWA investment)

- Sensibility to fuel price changes

- Sensibility to travel time savings changes

- Sensibility to impact delay

## I.2	Results

### I.2.1	CAPEX

Table A. 23 - NPV for CAPEX 1 and CAPEX 2 (CBA)

### I.2.2	Fuel price change

Table A.24 - NPV for fuel price changes

### I.2.3	Travel time savings changes

The impact of changes in travel time savings are proportional to changes in fuel prices.

### I.2.4	Impact delay

Table A. 25 - NPV for impact delay

## I.3	Conclusion

The sensibility of calculating the project without and with considering the NWA investments (CAPEX 1, CAPEX 2) is great but NPV keeps being high.

The impact in changes of fuel prices and travel time projections are relatively insensible, as 20% change of each produces only around 2 – 4% change in NPV. The same would be the case for a static analysis of changes in travel time savings as both are proportional.

A delay in generating benefits has a higher impact. Though the NPV with a 1 or 2-year delay keeps being high, the impact of delays on the NPV are important. NPV change for CAPEX 1 is between 4 and 7% and for CAPEX 2 between 3 and 5%.

This leads to the conclusion that the delay has the greatest impact on NPV and that a high priority of the present component in the project schedule is recommended.

# A.2	Annex

## A.2.1	CBA calculations (fuel savings, travel time savings, CO2 emissions savings)

### Fuel consumption

Table A.26 - Fuel consumption in litres (CBA)

Table A.27 - Fuel consumption in US Dollars (CBA)

### CO2 emissions

Table A.28 - CO2 emissions in kg (CBA)

Table A.29 - CO2 emissions in US Dollars (CBA)

### Travel time

Table A.30 - Travel time in h (CBA)

Table A.31 - Travel time in US Dollars (CBA)

### ITS project impact development

Table A.32 - ITS project impact development (CBA)

### NPV CAPEX 1

Table A.33 - NPV CAPEX 1 (CBA)

### NPV CAPEX 2

Table A.34 - NPV CAPEX 2 (CBA)

## A.2.2	Fuel price -20%

### Fuel savings calculation

Table A.35 - Fuel consumption l (Fuel price -20%)

### NPV CAPEX 1

Table A.36 - NPV CAPEX 1 (Fuel price -20%)

### NPV CAPEX 2

Table A.37 - NPV CAPEX 2 (Fuel price -20%)

## A.2.3	Fuel price +20%

### Fuel savings calculation

Table A.38 - Fuel consumption (Fuel price +20%)

### NPV CAPEX 1

Table A.39 - NPV CAPEX 1 (Fuel price +20%)

### NPV CAPEX 2

Table A.40 - NPV CAPEX 2 (Fuel price +20%)

## A.2.4	1-year delay in impact generation

### Development pattern

Table A.41 - ITS project impact development (1-year delay)

### NPV CAPEX 1

Table A.42 - NPV CAPEX 1 (1-year delay)

### NPV CAPEX 2

Table A.43 - NPV CAPEX 2 (1-year delay)

## A.2.5	2-year delay in impact generation

### Development pattern

Table A.44 ITS project impact development (2-year delay)

### NPV CAPEX 1

Table A.45 - NPV CAPEX 1 (2-year delay)

### NPV CAPEX 2

Table A.46 - NPV CAPEX 2 (2-year delay)

John Weiss (1987) – “Approaches to estimating national economic parameters: Jamaica, Nepal and Ethiopia” estimates an average cost factor of 0.8 for Jamaica. Therefore the economic analysis using domestic prices is a conservative scenario and the use of shadow prices would improve the economic indicators. Article published in the Journal Project Appraisal – March 1987.

The investment for the retrofitting is assumed to take place in year 3 with retrofitting for each building completed in years 3 and 4 as shown in the Results Matrix

The ±20% change is a conservative assumption. A +20% cost overrun is not common in energy efficiency projects (contrary to projects that involve civil works). More information can be found in the “Energy Efficiency Guidelines Series – IDB publication (2012) The -20% case serves a double purpose: compensate for a possible overvaluation of economic costs (for a ratio of 0.08 as pointed before) and a potential decrease of cost due to technological improvements.

The ±20% change sensitivity to electricity price is a percent change to the projection obtained by escalating the price series following projections to the oil price. The objective is to account for the volatility associated to the prices of primary energy where oil plays a very significant role and long term fluctuations of 20% around the forecast have been observed (Commodity Markets Outlook – World Bank series).

The study “Energy Efficient: a compelling global resource” (McKinsey&Co,2010) defines a sensitivity analysis on energy efficiency technologies and show that even the most expensive opportunities selected are ENPV-positive over the lifetime of the measure and represent the least expensive way to provide for future energy requirements. The study also points out government buildings as one of the main cluster for efficiency potentials in the next decade.,

European Commission’s Clean Vehicle Portal: Directive 2009/33/EC

Source: Universidad de Medellin: Lecture on Intelligent Transportation Systems, Medellin, Colombia, 2016

Source: Xiao-Feng Xie, et al. Real-time traffic control for sustainable urban living. IEEE International Conference on Intelligent Transportation Systems (ITSC), Qingdao, China, 2014: 1863-1868

Source: Banse Klaus, Prof.-Univ.: Post degree lecture on Intelligent Transportation Systems Design, Universidad de Medellin, Medellin, Colombia, January 2015.

Source: IPCC, 2014.  Intergovernmental Panel on Climate Change Working Group II Assessment Report 5 (AR5) Summary for Policymakers.

Source: PETROJAM LIMITED, http://www.petrojam.com/price-index

Source: PETROJAM LIMITED, http://www.petrojam.com/price-index

Source: United States Environmental Protection Agency: Report EPA420-F-08-024, Average Annual Emissions and Fuel Consumption for Gasoline-Fueled Passenger Cars and Light Trucks, October 2008

Source: OANDA, https://www.oanda.com/currency/converter/

Source: Aimsun 8 User Manual, TSS, Barcelona, 2015