DOCUMENT OF THE INTER-AMERICAN DEVELOPMENT BANK

Jamaica

Energy Management and Efficiency Programme EU-CIF

(JA-G1003)

Cost Benefit Analysis

May, 2017

This CBA analysis is based on the report “Energy Conservation & Efficiency for Jamaica: Economic Assessment and Investment Plan prepared by: DNV-GL dated August 2016”

INDEX

CBA COMPONENT I – RETROFITTING BUILDINGS

I.	EXECUTIVE SUMMARY

II.	INTRODUCTION AND BACKGROUND

III.	ECONOMIC ASSESSMENT

3.1	Main assumptions and methodology

3.2	Base Case

IV.	INVESTMENT ASSESSMENTS

4.1	Economic benefits

4.2	Economic cost

4.3	Economic returns

4.4	Sensitive analysis

V.	CONCLUSIONS

VI.	REFERENCES

CBA – JA-G1003

Glossary

| Bls | Barrels |
| --- | --- |
| BOE | Barrels of Oil Equivalent |
| BoJ | Bank of Jamaica |
| DSM | Demand Side Management |
| EC | Energy Conservation |
| EE | Energy Efficiency |
| EIRR | Economic Internal Rate of Returt |
| ENPV | Economic Net Present Value |
| ESET | Electricity Sector Enterprise Team |
| EWP | East West Power |
| GCT | General Consumption Tax |
| GER | Government Electricity Regulator |
| GNI | Gross National Income |
| GoJ | Government of Jamaica |
| HF | Heavy Fuel |
| ICT | Information and Communication Technologies |
| IPP | Independent power producers |
| IRP | Integrated Resource Plan |
| JAMALCO | Jamaica Aluminum Company |
| JEC | Jamaica Energy Council |
| JEP | Jamaica Energy Partners |
| JPPC | Jamaica Private Power Company |
| JPS | Jamaica Public Service Company |
| LCE | Low Carbon Economy |
| LNG | Liquefied Natural Gas |
| MSET | Ministry of Science, Energy and Technology |
| NECEP | National Energy Conservation and Efficiency Policy |
| NEP | National Energy Policy |
| NGCC | Natural Gas Combined Cycle |
| OUR | Office of Utilities Regulation |
| PCJ | Petroleum Corporation of Jamaica |
| Petrojam | Petroleum of Jamaica - PDVSA/PCJ owned company |
| PIoJ | Planning Institute of Jamaica |
| PM | Project Manager |
| PPA | Power Purchase Agreement |
| PS | Project Sponsor |
| PSA | Production Sharing Agreement |
| PV | Photovoltaic |
| QoS | Quality of Service |
| RE | Renewable Energies |
| REP | Rural Electrification Program |
| RFP | Request for Proposal |
| SAIDI | System Average Interruption Duration Index |
| SAIFI | System Average Interruption Frequency Index |
| T&D | Transmission and Distribution |
| TFP | Total Factor Productivity |
| URA | Office of Utilities Regulation Act of 1995 |
| US$ | United States Dollars |
| VAT | Value Added Tax |
| VRE | variable renewable energy |
| WACC | weighted average cost of capital |
| WKPP | West Kingston Power Partners |
| WTI | West Texas Intermediate |

I.	EXECUTIVE SUMMARY

This Cost Benefit Analysis will be developed on the results of the Component I of the project JA-G1003 (Energy Management and Efficiency Program for Jamaica): retrofitting of public buildings. Energy efficiency implementation includes improvements to building envelope, lighting systems, mechanical systems, plumbing systems, controls, miscellaneous equipment, and renewable energy system implementation if applicable. In most cases, the facility includes more than one building; however, they are operated as a single facility and should be considered as such. The seven hospitals included in this scope of work include:

- University Hospital West Indies

- Bustamante Children Hospital

- St Anns Bay Hospital

- Burke RD Hospital

- Annotto Bay Public Hospital

- Port Antonio Hospital

- Port Maria Hospital

In 2016, DNV-GL executed investment grade audits over six different buildings in Jamaica, including 4 hospitals. The information resulting from those audits has been used to project and assess the investment costs and potential improvement measures that could lead to savings during the following years in the seven hospitals targeted under this project.

After the technical inspections, DNV-GL run a series of financial tests considering the potential loan interest rates and the investments that were needed to be carried out.

As a result, excellent financial results which have been Economic Internal Rate of Return (EIRR) values between 17% and 27% for the buildings project, and extremely positive Economic Net Present Value (ENPV), both depicted in section 4.1 have been obtained considering that, the implementation of the energy efficiency measures shall be accepted on economic merit.

II.	INTRODUCTION AND BACKGROUND

Jamaica has been one of the most active countries in the Caribbean from an energy policy perspective. Some of the main issues addressed include renewable energy penetration, energy efficiency and conservation, and system losses. Despite these efforts, dependence on energy imports and electricity costs for the consumer remains high; volatile oil markets create uncertainty in prices and deliveries; and adoption of renewable energy to reduce emissions requires planning for these unique resources. Planning for these efforts requires an integrated resource plan to:

- Communicate strategy and implementation thereof

- Coordinate infrastructure requirements

- Notify all interested parties of changes to allow said parties adapting to the changes

- Determine the scope of potential impacts

The National Energy Conservation and Efficiency Policy 2010-2030 (NECEP) provides the overarching framework for energy efficiency (EE) in Jamaica with a national level target for a reduction in energy intensity and consumption across all sectors of the economy from 21,152 BTU in 2009 to produce US$1 of output to 6,000 BTU/US$1 of output by 2030.

Table 2.1. Goals of Jamaica National Plan.

Source: Jamaica Energy Policy 2009-2030.

Of particular highlight in NECEP for this Program are Goal 1 and Goal 6.

An IDB-funded study of 6 walk through and investment grade audits, carried out in 2016, investigated the potential of deep energy retrofits in government facilities. The findings revealed that it is possible to achieve a reduction in electricity consumption of up to 30% within selected public sector (named as Health, Education and Public Agencies – HEPA) facilities, producing saving the GOJ electricity bills and therefore in imported oil barrels for those public facilities. The audited buildings were 4 hospitals and 2 schools:

- Kingston Public Hospital and Victoria Jubilee Hospital

- Cornwall Western Regional Hospital

- Mandeville Public Hospital

- Marcus Garvey High School

- Heart Trust Ebony Park Academy

- Falmouth Hospital

Previous to EMEP - The Energy Efficiency and Conservation Technical Assistance (EECTA) report which was completed in 2011 and reviewed 2009-2010, was based on 22 Walk through audits and 14 detailed audits. The EECTA indicated that an investment of approximately US$113 million had the potential of saving 101GWh/year, or 25% of the electricity consumption of the Jamaican public sector at the time (estimated at 411kWh/year).

Based on discussions with MSET, for a projected spend of US$20 million, it was determined that a 4-5% reduction in electricity consumption in public sector facilities could be realized.

This project is using the above mention information to project the costs a potential savings in the 7 hospitals that will be retrofitted. Based on the electricity consumption and the number of beds of each of them, it is possible to make an analogy with the existing Investment Grade Audits in other similar hospitals and project of the investment costs and potential resulting savings. In this project, the total investment costs to retrofit the 7 hospitals are estimated over US$9 million.

The analysis includes a detailed description of assumptions and methodology, economic benefits and costs, economic return and a sensitivity analysis of key variables.

III. ECONOMIC ASSESSMENT

3.1	Main assumptions and methodology

The following are the assumptions and methodology for the economic evaluation of the recommended energy efficiency (EE) measures:

- The economic analysis is based on the energy technical assessment performed to 6 public buildings in 2016, 4 hospitals and 2 schools, in Jamaica to improve their energy characteristics. These results have been projected to the 7 hospitals that will be retrofitted under this project based on the current electricity consumption and on the number of beds, investment needs and resulting savings can be inferred.  Energy efficiency implementation includes improvements to building envelope, lighting systems, mechanical systems, plumbing systems, controls, miscellaneous equipment, and renewable energy system, if applicable. In most cases, the facility includes more than one building; however, they are operated as a single facility and should be considered as such.

- The economic analysis follows a Cost Benefit Analysis (CBA):

- Economic Benefits consist of electricity savings (direct effect);

- Economic Costs consist mainly on the investment cost of the different recommended measures (O&M are lower than BAU therefore with very little impact on the figures);

- Other externalities such as reduction of CO2 emissions have been considered with an assessment purpose, but have not been considered as a benefit for the base case scenario for the project as there is no CO2 market where to sell the emissions. However, a scenario including a monetary valuation of CO2 emissions reduction was also run.

- For each public building under investigation, the study recommended a number of energy efficiency investment measures. The economic life of each measure varies from 5 to 20 years. The stream of economic benefits takes into account the economic life of the different measures.

- The analysis is performed using a 20-year reference period to be able to capture the overall economic life of the project assets. This considers that cooling systems have an average lifespan of 18-25 years with little or some maintenance and average LED lightings have a lifespan of approximately 15 years.

- The reductions of CO2 emissions were determined based on an emission factor (ton CO2 per kWh): 0.000637 (source: IEA).

- The economic discount rate used was the standard 12% typically used by the IDB.

- Electricity tariff is based on an average of historical costs for the last 5 years (US$/kWh).

- Jamaica has high energy rates and a large dependence upon oil. Therefore, the escalation rates were set based on the WTI real crude forecast from the Long Term Energy Outlook published by the US Department of Energy, Energy Information Agency (DNV-GL 2016). For the annual escalation rate we have used a rate of 0.29% for the first 5 years and 2.18% for the remaining years (based on WTI crude forecast growth rate).

This assessment is not using efficient (shadow) pricing but market prices. This is because there are no recent estimates of shadow prices that could be used. Shadow prices were not addressed in the CBA analysis because of (i) limited recent data for key variable that are required for a shadow price assessment and, (ii) an ex-ante assessment suggests that applying shadow prices would increase the project's ENPV and EIRR. Shadow prices could be applied to the Jamaica dollar exchange rate (with the US dollar), to the price of imported fuels and materials, and to the cost of labor (both skilled and unskilled). An analysis of Jamaica shadow prices that was performed in 1987 estimated national economic parameters.

3.2	Base Case

The energy savings are based on the energy technical assessments performed by DNV-GL Energy Engineers in 6 public buildings in Jamaica in 2016 on the framework of the Energy Management and Efficiency Program (EMEP, JA-L1056). The goal of the work was to provide a path to widespread energy savings across municipal and federal buildings throughout Jamaica. The expected results of the project are:

- Reduced electricity consumption within government facilities;

- Decreased oil imports through improved EE;

- Reduced GHG emissions which can contribute to Jamaica’s INDC commitment;

- An increase capacity to promote and supervise electricity planning in Jamaica.

Having defined the objective of the intervention, the next step is to present in detail the proposed project to be implemented. Information and data about the project’s engineering features, technical characteristics and expected effects were provided in separate reports for each of the 6 buildings analyzed with Investment Grade Audits in Jamaica. For further details please refer to the Assessment Reports submitted to the Ministry of Science, Energy and Technology (MSET).

The following table provides a summary of the recommended EE measures for recommended in the 4 Hospitals audited in Jamaica in 2016 and that will be taken as reference for the current analysis.

Table 3.1. Summary of the recommended measures per building.

| Measures/Building | Kingston Public & Victoria Jubilee Hospital | Cornwall Hospital | Falmouth Hospital | Mandeville Hospital |
| --- | --- | --- | --- | --- |
| Window Replacements with Insulated Glazing | X | X |  |  |
| Automatic Door Closers | X | X | X | X |
| Air Seals on Doors | X | X | X | X |
| Cool Roof Installation |  |  | X |  |
| Variable Refrigerant Flow (VRF) Systems | X |  |  |  |
| Proper Spacing between Split Units | X | X |  | X |
| Replace split systems with high efficiency, inverter-driven units | X |  |  | X |
| VFD Installation on Kitchen Fans | X |  |  | X |
| VFD Installation on Chiller Pumps | X |  |  | X |
| Fluorescent to LED Fixture Retrofit | X | X | X | X |
| Occupancy Sensor for Interior Lights | X | X | X | X |
| Compressed Air Leakage | X | X |  | X |
| Centralized Chiller Controls | X | X |  | X |
| Solar PV | X | X | X | X |
| VFD on supply and return fans |  | X | X |  |
| VFD on Exhaust Fans |  | X |  |  |
| VFD on constant volume pumps |  | X |  |  |
| BAS upgrades |  | X | X | X |
| Installment of Solar Tubes |  |  | X |  |
| Exterior Light Retrofit to LED |  |  |  | X |
| Replace Broken Windows |  |  | X |  |
| Proper Spacing Behind Condenser Units |  |  | X |  |

IV.	INVESTMENT ASSESSMENTS

4.1	Economic benefits

4.1.1	Energy savings

Projects involving a refurbishment of public buildings or works to improve building envelope, lighting systems, mechanical systems, controls, miscellaneous equipment, and renewable energy system are associated with an increase of energy efficiency, which is reflected in a reduction of energy consumption costs. The benefit is valued via the decrease of energy costs incurred when attaining the same final useful effect as in the without-the-project scenario. The saved costs compared to the counterfactual (BAU) scenario represent the project’s benefit.

Table 4.1 Energy savings in each building.

| Buildings | Current Consumption (kWh) | Annual Energy Savings (kWh) | Number of beds |
| --- | --- | --- | --- |
| UH West Indies | 10,212,000 | 4,288,390 | 579 |
| Bustamante Children Hospital | 1,718,496 | 987,710 | 280 |
| St Anns Bay Hospital | 2,054,232 | 1,042,559 | 271 |
| Burke RD | 1,387,296 | 1,107,503 | 386 |
| Annotto Bay Public Hospital | 484,992 | 342,837 | 117 |
| Port Antonio Hospital | 300,680 | 242,287 | 92 |
| Port Maria Hospital | 578,186 | 240,476 | 60 |

These figures of electricity consumption were obtained based on the consumption of the 4 hospitals analyzed during the investment grade auditing work of 2016 for the Energy Management and Efficiency Program (EMEP, JA-L1056). Based on these audits, average consumption per bed in was obtained and therefore extended to the 7 hospitals targeted in this project.

4.1.2	CO2 emissions reductions

As with any other energy project, the projects aimed at improving consumption EE are also characterized by environmental externalities, such as reduction of CO2 emissions.

The annual GHG emissions reduction were determined based on the estimated annual energy savings (expressed in kWh) and converted into ton CO2 eq (using an emission factor of 1.09 ton CO2/MWh). As shown in table below it is expected that the implementation of the EE measures will reduce CO2 in 9,003 Ton CO2eq per year.

Table 4.3 CO2 net emissions avoided in each building.

| Buildings | Annual net CO2eq emissions (ton CO2eq) |
| --- | --- |
| UH West Indies | 4,679 |
| Bustamante Children Hospital | 1,078 |
| St Anns Bay Hospital | 1,137 |
| Burke RD | 1,208 |
| Annotto Bay Public Hospital | 374 |
| Port Antonio Hospital | 264 |
| Port Maria Hospital | 262 |
| TOTAL | 9,003 |

Please note that a monetary valuation of CO2 emission reductions results were not quantified for the base case scenario and used in the EIRR and ENPV calculation. Also, it is not expected the installation of any emission monitoring device, been emissions calculated based on the electricity savings and therefore in fuel combustion avoided.

4.1.3	Other savings/benefits

It is worth mentioning that job retention/creation can be seen as an indirect benefit of the recommended EE measures. However, such co-benefit is beyond the scope of this analysis.

4.2	Economic cost

The economic costs include all investments or non-recurring costs as well as all operation and maintenance costs needed to implement the EE measures during the life of the project.

The investment cost of each measure was estimated including fixed and variable costs i.e. based on quantities and respective cost per unit and labor per unit (expressed in US$), as applicable. It is assumed that all EE measures are implemented as a single construction project and, therefore, they occur in the beginning of Year 1. No additional annual flows of investment and/or non-recurring costs are foreseen for the life of the project.

Table below shows a summary of the total investment cost estimated per building. These costs include only the incremental costs resulting from the implementation of the EE measures.

Table 4.4 Total investment costs.

| Buildings | Total measures Cost (US$) |
| --- | --- |
| UH West Indies | $6,357,736 |
| Bustamante Children Hospital | $728,603 |
| St Anns Bay Hospital | $868,987 |
| Burke RD | $523,741 |
| Annotto Bay Public Hospital | $172,262 |
| Port Antonio Hospital | $83,825 |
| Port Maria Hospital | $210,651 |
| TOTAL | $8,945,805 |

As regards the operation and maintenance costs they are lower than the BAU scenario therefore negligible.

4.3	Economic feasibility

Once costs and benefits have been identified and valued, the economic feasibility of the project (s) shall be discussed. There are three main criteria:

- Economic Net Present Value (ENPV) compares all costs with benefits, with all resource flows discounted by the agreed discount rate.

- The Benefit-Cost Ratio (BCR) is the ratio of the sum of benefits (in present value) to the sum of costs (in present value). Typically, if this ratio is more than one, using this criterion the project would be approved.

- The Economic Internal Rate of Return (EIRR) is defined as the discount rate that causes the ENPV to be zero. Typically, if the EIRR is larger than the recommended 12% discount rate the project would be approved.

For this project that deals with the EE retrofitting in seven hospitals, the economic indicators of the base case scenario are an ENPV of $5,353,832 and an EIRR of 23.4%.

Note that the base case scenario does not include a monetary valuation of the CO2 emissions reduction (9,003 tons/year). If it were included assuming US$37/ton as benefit – the assumption used in the fuel efficiency component, the EIRR would increase to 24,1% and the ENPV would increase to US$5,590,934.

Figures below show the EIRR, ENPV and BCR calculation showing the benefit and costs streams used and the economic return estimations for the implementation of the EE measures in the 7 hospitals under investigation and the lighting project.

Figure 4.1 & Hospitals investment and return graph.

4.4	Sensitive analysis

This section contains a discussion of the feasibility analysis of the project. A sensitivity analysis was done for two changes: (i) increase/decrease 20% of the investment costs and (ii) increase/decrease 20% of the electricity price. EIRR and ENPV needs to be recalculated for changes in key variables that could affect project costs, benefits and assumptions. There have been considered scenarios of an increase or decrease of the capital costs of ± 20%, and the impacts of these have been evaluated with regards of the ENPV and IRR.

Considering the investments to be performed in the assets, the ENPV remains positive in all the scenarios considered.

For the EE retrofitting, the sensitivity analysis shows the following:

Table 4.5 Base case results and sensitivity analysis of 6 buildings.

|  | ENPV Sensitivity |  |  | EIRR Sensitivity |  |
| --- | --- | --- | --- | --- | --- |
|  | Discount rate US$ | Electricity price US$ | Investment costs US$ | Electricity price | Investment costs |
| Minimum (-20%) | $7,456,914 | $2,226,439 | $7,120,937 | 17% | 31% |
| Base value | $5,353,832 | $4,635,537 | $5,710,225 | 22% | 25% |
| Maximum (+20%) | $3,708,098 | $7,044,635 | $4,299,513 | 27% | 20% |

Figure 4.3 NPV Sensitivity analysis for7 Hospitals.

Figure 4.4 Sensitivity analysis of EIRR 7 Hospitals

IRR rate is remaining positive in all the scenarios, and above the stated 12%, demonstrating robustness of the economic viability of the project.

V.	CONCLUSIONS

With respect to the retrofitting of 7 hospitals, two economic evaluations were made. The first one comprises the energy assessment performed to a defined group of 7 hospitals for improving their energy characteristics as a representation of the projects that will be financed. The benefits consisted of electricity savings (direct effect) and reduction of CO2 emissions (externality). The costs consist mainly on the investment cost of the different recommended measures (O&M costs are lower than business as usual therefore irrelevant). The more relevant assumptions of the analysis are: (i) the economic life of each measure vary from 5 to 20 years; the electricity prices are based on an average of historical costs ($USD/kWh) per building without considering inflation; annual escalation rate: 0.29% for the first 5 years; 2.18% for the remaining years (based on WTI crude forecast growth rate).

The results for the base case show an EIRR of 23.4% and the ENPV is US$5,353,832. A sensitivity analysis was performed to test the impact of significant changes to investment costs and the electricity price. The project showed to be economically robust.

The project is also characterized by environmental externalities such as reduction of CO2 emissions. It is expected that the implementation of the EE measures will reduce CO2 emissions in 9,003 Ton per year. If it were included assuming US$37/ton as benefit – the assumption used in the fuel efficiency component II of the loan, the EIRR would increase to 24,1% and the ENPV would increase to US$5,590,934.

VI.	REFERENCE

| /1/ | DNV GL 2016, Energy Sector Diagnosis Study Jamaica, Support for the Development of a comprehensive Energy Planning Program for Jamaica, report to the Inter-American Development Bank, Report No.: 10004931-ESZA-R-01, Rev E. |
| --- | --- |
| /2/ | DNV GL 2016, Kingston Public and Victoria Jubilee Hospitals Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /3/ | DNV GL 2016, Ebony Heart Trust Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /4/ | DNV GL 2016, Falmouth Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /5/ | DNV GL 2016, Marcus Garvey High School Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /6/ | DNV GL 2016, Mandeville Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /7/ | DNV GL 2016, Cornwall regional Hospital Assessment Report, Jamaica Ministry of Science, Energy and Technology. |
| /8/ | FACILITY DATA LIGHTING LOTS Aug 24.xlsx agreed between MSET and DNV GL. |

ABOUT DNV GL

Driven by our purpose of safeguarding life, property and the environment, DNV GL enables organizations to advance the safety and sustainability of their business. We provide classification and technical assurance along with software and independent expert advisory services to the maritime, oil and gas, and energy industries. We also provide certification services to customers across a wide range of industries. Operating in more than 100 countries, our 15,000 professionals are dedicated to helping our customers make the world safer, smarter and greener.

http://www.iea.org/bookshop/729-CO2_Emissions_from_Fuel_Combustion

John Weiss (1987) – “Approaches to estimating national economic parameters: Jamaica, Nepal and Ethiopia” estimates an average cost factor of 0.8 for Jamaica. Therefore, the economic analysis using domestic prices is a conservative scenario and the use of shadow prices would improve the economic indicators. Article published in the Journal Project Appraisal – March 1987.

Source of current consumption and number of beds: provided by MSET.  The calculation of savings obtained are based in the audits carried out in 2016 by DNV to Cornwall Regional Hospital and Mandeville Public Hospital. Based on similar number of beds, and therefore size, electricity reduction per bed was calculated the retrofitting costs and results projected to the other hospitals.

Electricity savings include the contribution of PV generation installed.

IDB calculations. Based on kWh expected savings per buildings, and considering an emission factor of 1.091 ton CO2eq/MWh in Jamaica (Second National Communication to the UNFCCC, December 2011).

IDB calculations based on Cornwall Regional Hospital audits carried out by DNV in 2016. Due to similar building characteristics, cost per bed were calculated and then projected to the hospitals.

The ±20% change is a conservative assumption. A +20% cost overrun is not common in energy efficiency projects (contrary to projects that involve civil works). More information can be found in the “Energy Efficiency Guidelines Series – IDB publication (2012) The -20% case serves a double purpose: compensate for a possible overvaluation of economic costs (for a ratio of 0.08 as pointed before) and a potential decrease of cost due to technological improvements.

The ±20% change sensitivity to electricity price is a percent change to the projection obtained by escalating the price series following projections to the oil price. The objective is to account for the volatility associated to the prices of primary energy where oil plays a very significant role and long-term fluctuations of 20% around the forecast have been observed (Commodity Markets Outlook – World Bank series).

The study “Energy Efficient: a compelling global resource” (McKinsey&Co,2010) defines a sensitivity analysis on energy efficiency technologies and show that even the most expensive opportunities selected are ENPV-positive over the lifetime of the measure and represent the least expensive way to provide for future energy requirements. The study also points out government buildings as one of the main cluster for efficiency potentials in the next decade.,