TRANSPORT INFRASTRUCTURE REHABILITATION PROGRAM

ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

JAMAICA
July/2008

TRANSPORT INFRASTRUCTURE REHABILITATION PROGRAM ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

I.	Introduction	3

II.	Objectives	3

III.	Project Description	3

IV.	Environmental Policy and Strategies	3

V.	Roles and Responsibilities	3

VI.	Environmental Management Structure	3

Environmental Management Implementation Plan	3

Emergency Preparedness	3

Monitoring and Recording	3

Documentation	3

VII.	Environmental Requirements	3

Construction Camps and Storage Facilities	3

Water Pollution Control	3

Erosion and Sediment Control	3

Drainage and Water Resources Protection	3

Dust and Air Pollution Control	3

Noise and Vibration Control	3

Waste Management	3

Vegetation and Fauna Protection	3

Health and Safety	3

Community relations	3

Protection of Cultural and Natural Heritage Features	3

Relocations	3

## Introduction

  - The Environmental and Social Management Plan (ESMP) aims to ensure that mitigation measures for the negative impacts indentified are properly undertaken in order to prevent or minimize adverse effects. This ESMP was prepared to assess all mitigation measures necessary during the Transport Infrastructure Rehabilitation Program construction phase.

  - The Transport Infrastructure Rehabilitation Program, to be funded by IDB, aims to support the restoration of Jamaica’s transportation system through the rehabilitation of vital portions of the primary, secondary and tertiary road network which have been damaged due to storms and to help fillip economic activity and improve the quality of life of the Jamaican population by the delivery of transportation services on a continuous, safe and reliable basis.

  - This ESMP was prepared based on the Environmental Management Guidelines prepared by the Technical Service Department of the National Works Agency (Revision 1 - April 2006) and Bank’s Policies. Accordingly to these guidelines, a project-specific ESMP must be prepared and responded to by the Contractor in an Environmental and Social Management Implementation Plan (EsMIP). This ESMIP corresponds to the project-specific ESMP. The ESMIP provides the framework for satisfying the Contracting Agency that the contractor is aware of the environmental issues and required mitigation measures to be applied to the project.

  - The bidding documents must include Environmental Clauses indicated in the Environmental Management Guidelines (2006), as works requirements that the Contractor must comply with. These environmental clauses are specified and detailed in the present ESMP. The purpose of this ESMP is to provide a controlled source of mandatory environmental clauses that establish the minimum requirements for the contractor.

  - Revision of the ESMP will be appropriate as the works progress and the nature and the extent of the specific environmental problems becomes clear. For this reason, the ESMIP also should be subject to periodic audits to refocus the plans in light of experience gained and issues that have emerged.

## Objectives

  - The broad objectives of the ESMP are to ensure that protection measures are followed during construction.

  - Specific objectives of the ESMP are:

- Define the organizational and administrative arrangements for environmental monitoring including definition of responsibilities of staff and the coordination, liaison and reporting procedure.

- Define requirements and procedures for environmental monitoring including equipment needs, frequency of monitoring, parameters to be monitored, analytical services requirements, data management and presentations, etc.

- Discuss procedures for pro-active environmental management with the site supervision staff, so that potential problems can be identified, and mitigation measures adopted prior to works being initiated.

- Define procedures for environmental control, in the event of pollution, fire or similar incidents.

## Project Description

  - The general objectives of the Transport Infrastructure Rehabilitation Program (TIRP) include: (i) restoring essential portions of the road system and ensuring continuity of the primary network; (ii) active public investment in construction to create jobs; (iii) help decrease travel time for transportation system users, which will in turn benefit access to social services and economic spaces.

  - Program Components: The program will be implemented over a five-year term and will include three components (i) Civil works will support the rehabilitation and improvement of selected primary roads on the main road network (ii) Project Implementation Support will center on supporting staff for the project execution unit, and in providing ancillary (environmental and engineering) professional support (iii) Technical studies will provide support for technical assistance and studies to inform future supplementary interventions. Each component is described in detail below.

  - Component 1 – Civil Works (US$17.2 million). Under this component roads under the purview of the NWA will be selected for rehabilitation using HDM for the economic analysis and prioritization based on the surface condition and traffic volume. In addition projects must:

      - be part of the national main road network system comprising 5006km

      - have a budget allocation provided for the corresponding year

      - be economically feasible, i.e., the economic rate of return shall be at least 12%

      - be environmentally feasible thereby complying with the country’s environmental legislation and Bank Policies

      - have final designs prepared  and have incorporated mitigation measures to prevent correct or compensate direct environmental impacts.

      - have clearance of the right of way” before signing the contract for civil works

  - The scope of work in this component is expected to fall into two groups (i) routine maintenance comprising simple works consisting of patching, milling, sealing cracks, repairs to longitudinal drains, road markings and medians. It will include repair of surface distress, pavement strengthening and reduction of roughness, reconstruction of inadequate drainage facilities, and retaining walls which will only require simple standard designs and the implementation of the Environmental and Social Management Plan and (ii) periodic maintenance comprising bridges restoration, river training,  embankment stabilization and sea defense works as well as improvement of geometrical and structural characteristics requiring detailed designs and more important engineering and environmental studies prior to construction.

  - Component 2 – Project Implementation Support (US$ 2.7 million) This component will directly support the hiring of the project manager, the project accountant and an environmental expert to manage the day to day technical, socio-environmental, financial and administrative aspects of the project and the hiring of an engineering and supervisory consultant firms on a competitive basis to monitor and oversee the implementation of the proposed construction of the civil works. This component will also provide resources for minor office equipment and the hiring of financial and environmental auditors and a monitoring and evaluation consultant to respectively conduct the audits and to track and monitor the results of the operation.

  - Component 3 – Technical Studies (US$ 2.1 million). Provides resources for studies to strategically address key issues that will assist in continuing the works to improve road transport policy and regulation covering inter alia: the RMMS, and alternative sources of funding to augment the road maintenance fund. Funding will support the hiring of a consultant to assist the Project coordinator in determining, managing and implementing strategic interventions to strengthen the governance of road maintenance. Also at the request of Government this component provides resources to undertake engineering and environmental feasibility studies pertinent to the rehabilitation of future restoration or improvement of road sections. Amongst the possible studies Jamaican authorities have considered developing feasibility studies for the Southern Highway Corridor focusing on continuing the main road-network circumscribing the island in keeping with GOJ’s strategic development plan for the south coast of the islandNWA prepared a list of proposed works for both the emergency operation and the TIRP including 23 road sections, including river training, repavement, bridges rehabilitation, etc. The emergency operation (JA-L1015) is directed to support immediate rehabilitation of basic road infrastructure affected by hurricane Dean; the reconstructive work will be such as to reduce the impact of future hurricane conditions and flooding.

  - The eligibility criteria for road rehabilitation selection will consider HDM for the economic analysis; and prioritization based on the surface condition and traffic volume. In addition projects must: (i) be part of the national main road network system; (ii) have a budget allocation provided for the corresponding year; (iii) be economically feasible and sustainable assuring the investments will last for a long term with no further major investments; (iv) be environmentally feasible thereby complying with the country’s environmental legislation, and environmentally sustainable assuring that no additional impacts will be imposed to the environment; (v) have final designs prepared  and have incorporated mitigation measures to prevent correct or compensate direct environmental impacts including removal of utilities; (vi) have clearance of the right-of-way before signing the contract for civil works; (vii) ensure that all residents or vendors, if any, are properly relocated accordingly to a resettlement plan approved by the Bank.

  - Given the characteristics of the proposed works, as well as the fact they will take place mostly within the existing road right-of-way, the potential negative impacts are expected to be: (i) generation of dust, noise, vibrations, and air emissions by construction activities; (ii) soil erosion with possible sedimentation of nearby water bodies; (iii) traffic congestion, creation of hazardous driving conditions and tESMPorary obstruction of access to community services; (iv) accidental rupture of utilities pipes and lines (electricity, telephone, and water), and tESMPorary interruption of services during the excavation and removal of pavement; (v) risk of accidents for workers or road users (pedestrian or vehicles) around construction sites; (vi) soil and water contamination due to wastes and effluents (garbage, used waters, oil, grease, fuel, paint, etc.) coming from work areas or equipment yards.

## Environmental Policy and Strategies

- The Ministry of Transport and Works (MTW) is responsible for establishing and implementing transport policy, regulating land transport services and for the construction, maintenance and operation of all road networks. The policy and sector planning role is under the responsibility of the MTW, while the role of infrastructure management under the National Works Agency (NWA).

- The National Roads Services Improvement Program (NRSIP) gave special mphasis to institutional capacity building and sustainability in strategic leadership, planning, contracting, project management and supervising of road works, within the NWA. In this process of building its institutional strength for effectively manage the environmental implications of its main road network construction and maintenance programs, NWA had developed: (i) an Agency Environmental Policy Statement; (ii) Environmental Procedures; and (iii) Environmental Specifications for Construction and Maintenance Projects.

- It is the National Works Agency Environmental Policy and Strategies to protect the environment from adverse effects resulting from all alterations on the national road network. NWA is committed to:

- Implementing best environmental management practices, in order to meet or exceed applicable legal and other requirements;

- Strive for continual improvement in our Planning, design and implementation procedures to minimize the creation of wastes, depletion of the environment and pollution.

- Manage our processes, our materials and our people in order to reduce the environmental impacts associated with our work;

- Implementing and operating an Environmental Management System to further improve environmental performance and enhance the environment.

- The National Works Agency’s Environmental Action Plan has five aims which provide a policy framework for the long term, fundamental business and environmental benefits:

- Integrate Environmental Concerns in Operation. For an effective, environmentally sound road network, the NWA will incorporate environmental considerations and community values and goals in every aspects of its decision-making from early planning to operation and maintenance. The NWA will strive to become a leader among government agencies in carrying out an environmental ethic that encompasses the consequences of all activities, internal as well as external.

- Achieve Full Involvement of Staff, Customers and Stakeholders Consistent with NWA’s Vision. Effective communication is critical to successful implementation of the NWA’s mission and environmental policy. The agency will continually strive to communicate our commitment to protect and enhance the environment.

- Development and Strengthen Environmental Ethic by the Incorporation Active Protection and Enhancement Measures for the Natural and Built Environment. To be an environmental leader in the transportation field, the NWA must go beyond compliance and strive for environmental excellence. The widest possible range of both traditional and innovative measures to protect and enhance the environment must be pursued.

- Develop State-of-the-Art Research, Technology Transfer and Training Capabilities. Innovation through research and development is a major theme of the agency mandate and a priority for the NWA. The agency is committed to expanding environmental research and improving the transfer of new environmental technology to partners and stakeholders in the transportation community.

- Develop Environmental Expertise Commensurate with Organizational Mission, Goals and Responsibilities. If the NWA is to fulfill its mandate and this policy statement, it must have well-trained and professional personnel skilled not only in the traditional highway program related disciplines, but also on environmental, social and cultural sciences. These personnel must possess the expertise to provide the sound environmental and socio-economic advice needed as part of an effective and responsive decision-making process.

- Accordingly to NWA’s environmental policy environmental management requirements during construction shall be documented in this project specific Environmental and Social Management Plan (ESPM), which will be responded to by the construction contractor in its Environmental Management Implementation Plan (ESMIP).

## Roles and Responsibilities

- Environmental Management within the Government of Jamaica is under the responsibility of the National Environmental and Planning Agency (NEPA). NEPA has a statutory mandate for conservation, protection and proper management of the natural resources of Jamaica. The Natural Resources Conservation Authority (NRCA) Act stipulates that new developments and discharges might require a permit or a license that will be issued by NEPA.

- The NWA of the Ministry of Transport and Works (MTW) is the executing agency for this program. Within the Technical Services Department an Environmental Unit was created, and is being staffed through the NRSIP. For the TIRP implementation, the Environmental Unit will work in coordination with the Project Execution Team, in order to enforce the environmental management system.

- The responsibility for Environmental Management of the TIRP is the duty of the MTW and the NWA, although it is envisioned that monitoring will take place at four levels:

- Contractor. The Contractor will carry out specific mitigation activities to comply with the Environmental Clauses of the Contract, utilizing the methodology outlined in its Environmental Management Implementation Plan (ESMIP). These activities will be coordinated by the Contractor’s Environmental Manager.

- Supervision firms will provide environmental engineer to oversee contractors’ compliance with the ESMIP, provide training to NWA/EU and Contractors’ staff and prepare monthly reports and any other reports prepared in relation to environmental, including emergencies that may occur (e.g. spills of hazardous substances). These environmental reports should be presented to NWA/EU, NEPA and the Bank.

- National Works Agency. The Environmental Unit at the NWA will have the responsibility for environmental integrity of the project. The activities to be undertaken, in coordination with Project Executing Team, will ensure that Contractor’s activities are in compliance with the Contract Environmental Clauses and this ESMP. The NWA will take the necessary steps to ensure that sanctions are applied in cases where there are violations of the Environmental Clauses, the ESMP, and the ESMIP.

- National Environmental and Planning Agency. NEPA will ensure that the mitigation measures outlined in the ESMP and ESMIP are implemented in accordance with such requirements. It is also envioned that NEPA will exercise its authority to issue warnings, citations and cessation orders to the Contractor and the NWA whenever breaches have been observed.

- The overall policy with respect to environmental management of the road improvements lies with the NWA, through the recently created Environmental Unit. Specific standards, methods and procedures might be developed during project implementation, based on lessons learned and specific issues that might rise up.

- The role of NWA’s Environmental Unit is:

- Liaise with NEPA and agencies with jurisdiction over Jamaica’s natural resources;

- Stay informed regarding changes in laws and policies affecting the NWA;

- Keep agencies informed regarding program implementation and NWA’s mitigation activities;

- Develop and disseminate environmental guidance documents for NWA’s management and field personnel and for contractors;

- In cooperation with Human Resources and Administration, provide or coordinate appropriate environmental training for NWA’s Construction and Maintenance management and field personnel and for contractors;

- Implement, maintain, update environmental commitment tracking system;

- Develop and maintain an environmental actions tracking system to facilitate implementation of the NWA’s environmental program;

- Develop and maintain appropriate checklists to be used in the review of Environmental Impact Assessments prepared by consultants;

- Develop and update, in consultation with directors and project managers and appropriate management officials, performance measures to evaluate the environmental process;

- Conduct inspections and meetings to resolve problems, evaluate progress, and to identify systemic or recurring issues that my need special attention from NWA’s management. Provide status reports, advice and recommendations to NWA’s management.

- Specifically for the TIRP, the responsibilities of the NWA/EU’s Environmental Engineer are:

- Review all construction and maintenance contracts to ensure that they include environmental criteria for prioritizing works activities;

- Review and approve Contractor’s ESMIP;

- Ensure that all required environmental permits and licenses are in place;

- Ensure compliance with all environmental standards;

- Carry out all environmental supervision activities, including weekly inspections of construction sites, equipment and materials storage areas;

- Propose correction actions and ensure that they are implemented;

- Coordinate with Project Executing Team regarding program implementation and mitigation activities, as necessary;

- Prepare an environmental compliance report for each road section/works completed.

- The NWA/EU Environmental Engineer will prepare a Quarterly Inspections Report, based on the Environmental Monitoring Form filled in on a weekly basis (see Appendix 1). The inspections shall assess the environmental status of all construction sites and proposed structures; identify non-conformities and violation to this ESMP; notifications and corrective actions results. The reports must be submitted to the PET, MWT and IDB.

- The NWA/EU Environmental Engineer shall perform periodically environmental audits (at least 2 audits a year), to establish the effectiveness of environmental management on the project. The audit shall include: assessing the degree of compliance with the ESMIP achieved; reviewing the continuing relevance of the plan in the light of experience gained and the instigation of changes where appropriate; reviewing the organizational and administrative frameworks described in the ESMIP; reviewing environmental monitoring data and reporting; identification of environmental aspects and impacts and the mitigation measures proposed to mitigate these; and identification of any deficiencies in environmental performance and information on measures to address these.

- The Supervisor Environmental Engineer, provided by the Supervisor firm, will be responsible for overseeing environmental activities and performance, and each site manager will have environmental responsibilities. The Supervisor Environmental Engineer will give overall advice and define the general procedures, which will include environmental reports, and will establish day-to-day monitoring procedures. The Supervisory Environmental Engineer will provide training to the following groups: (i) NWA/EU Environmental Engineer and senior staff: training on basic environmental management procedures; and (ii) Construction Contractors’ personnel will be trained on: environmental management, environmental emergency response, materials handling, waste material management and application of environmental and social management specifications. The above training will be included in the supervision firms’ budgets

- The Supervisor Environmental Engineer will be responsible for: identify and develop the Environmental Management Implementation Plan; provide advice on environmental issues to the Contractor’s and site engineers; establish routine management, liaison and reporting systems with the NWA’s Environmental Engineer; interpret the results of the monitoring program and advise the responsible supervisor of action required; train and support the members of the Contractor’s staff, in particular during the initial period and during the occasional visits to sites.

- The ESMIP shall include an organizational chart clearly showing Contractor’s Environmental Manager position within the structure of personnel on the project, including lines of communication and responsibility. The contractor shall use this ESMP to develop procedures at specific level.

- All personnel on the project whose work may, directly or indirectly, cause a significant impact on the environment must be made aware of this and be given sufficient know-how to manage their activities accordingly. The Contractor shall ensure the ESMIP identifies how environmental awareness is to be achieved, how environmental competence training needs are identified and how training will be undertaken.

- The Contractor is responsible to bring all environmental management requirements to the attention of appointed personnel, as applicable, to their work, and to ensure that these requirements are complied with. The Contractor shall ensure that the terms and conditions of any sub-contractor placed by them shall include the requirement for the sub-contractor to comply with this specification.

- The Contractor shall be aware of all environmental legislation and regulations that may or do impinge on the project. It is the responsibility of the Contractor to ensure that relevant legislation and regulations have been taken into account.

- The Contractor will be responsible for any permits or licenses required. Expected permits are:

- Construction Camps are subject to the provision of a license by NWA and NEPA, which will be granted on the basis of a detailed drawing and a plan of operations submitted by the Contractor. Such plan of operations shall show clearly how negative impacts will be mitigated, including erosion and pollution controls, sanitation measures and restoration of camp sites.

- Borrow Areas and Quarries. In accordance with Jamaican law, the use of any borrow areas is subject to the provision of a license by the Department of Mines and Geology, which will be granted on the basis of a quarry or borrow area management plan submitted by the Contractor. Such plan must clearly show how environmental impacts will be mitigated, including erosion prevention and after-use restoration. Borrow sites and quarries must be assessed for environmental impact before use and a report should be submitted to NWA and NEPA.

- River Training. Watershed developments might require an environmental permit based on project detailed drawings to be submitted by the Contractor to NEPA. The information provided must address project description, importance for the road network protection, expected impacts and mitigation measures proposed.

- Sea Defenses. Works along the coastal zone, such as sea defenses, will require a beach license, by the Beach Control Authority, which will be granted based on detailed drawings and project information to be submitted by the Contractor. Such information shall show clearly project layout, detailed designs, potential negative impacts, and how they will be mitigated.

- Cultural and Historical Sites. No archeological remains are expected to found in this project as works will remain within the right-of-way. Though there is one historical site that might be directly affected by proposed works which is the Flat Bridge, at Bog Walk. Interventions on this site might require a previous permission from the Jamaica National Heritage Trust.

## Environmental Management Structure

- This Environmental and Social Management Plan (ESMP) is a method of ensuring that the measures and commitments are properly undertaken during construction phase of the project. Environmental Management requirements for construction are documented in this ESMP. These will then be responded to by the construction Contractor in an Environmental Management Implementation Plan (ESMIP).

- The ESMIP is a document that will be produced by the Contractor, in accordance with ESMP guidelines. It will detail how the Contractor will implement and manage environmental aspects of the project. The Contractor will outline mitigation measures that are required to be put in place for the duration of the Contract. The ESMIP is expected to be a “live document” and is updated throughout the course of the contract.

### Environmental Management Implementation Plan

- The contractor shall prepare an Environmental Management Implementation Plan (ESMIP) and submit it to the NWA’s Environmental Engineer for approval. The ESMIP shall define the Contractor’s program and procedures regarding environmental conservation and impact mitigation measures.

- The contractor shall act proactively to manage potential environmental impacts and develop appropriate mitigation measures from an early stage in the Project.

- The ESMIP must include: mitigation and pollution prevention strategies; organizational framework and specific responsibilities; principal pollution control facilities, and proposed contingency plans; waste management; environmental management procedures to ensure that facilities are operating satisfactorily and that operations are not causing undue stress; environmental awareness training program proposed for the contractor’s workforce; work camps, borrow pits, fill requirements including the overall analysis of cut and fill quantities; site clearance; health and safety; and storage and handling of hazardous materials.

- The Contractor shall describe how effective and positive communication with external, interested parties shall be conducted. The ESMIP shall also outline the how communication on environmental matters is to be undertaken by the contractor and the NWA / Environmental Unit.

- The ESMIP must comply with this ESMP, the Environmental Clauses of the contract, and with the NWA Environmental Procedures (2006) and Bank’s policies. The contractor shall not commence any work until the ESMIP has been approved by the Supervisor Environmental Engineer and NWA/EU Environmental Engineer.

### Emergency Preparedness

- The Contractor shall anticipate what emergency situations might arise and how these will be addressed. The ESMIP shall identify the different emergency situations or pollution incidents, and include emergency control measures for these. A process for the reporting of environmental incidents to the NWA’s Environmental Engineer and, where appropriate, relevant agencies, shall be included.

- To ensure preparedness for any spills on soil, groundwater and surface water, the Contractor shall submit plans for the containment and rapid clean-up and disposal of any spills. The Contractor shall ensure clean-up materials are stored in a readily accessible and useable state on site and in sufficient quantities to handle the maximum volume of liquid that may be spilled. Small quantities of waste from plant/equipment servicing shall be stored separately from general waste streams and disposed of at sites designated for such materials.

### Monitoring and Recording

- The Contractor shall have means of measuring and monitoring its environmental impacts in order to establish whether it is complying with legal limits and adequately controlling environmental impacts in order to achieve sound environmental performance. Monitoring and site inspections shall be required to ensure compliance with the ESMIP and to assess the overall performance and effectiveness of the required environmental mitigation measures.

- Monitoring shall include at minimum, but not limited to: erosion and siltation; water pollution; air pollution, dust, and noise control; drainage and water resources; extraction and restoration works at borrow areas; vegetation and wildlife protection measures; waste management and disposal operations; safety; and assurance that construction sites are cleaned up and restored after construction activities cease.

### Documentation

- The contractor shall keep copies of all relevant documentation required to ensure sound environmental management. Main environmental documentation will be ESMIP and other referenced documentation, which is essential to delivering sound environmental management. This will include: work instructions, and environmental procedures; permits and licenses; monitoring records; relocation of houses or vending; and incident records and corrective action undertaken.

- Reporting to NWA Environmental Unit shall be a formal monthly written report that summarizes the actions from previous period and progresses achieved by all parties, as well as provide commentary on all aspects of environmental management in the project.

## Environmental Requirements

- The mitigation measures herein described include the Best Management Practices and Measures identified in the Environmental Procedures (April 2006) prepared by NWA.

### Construction Camps and Storage Facilities

- Licenses. Construction Camps are subject to the provision of a license by MTW and NEPA. The following mitigation measures shall be used to control pollution, erosion and other direct impacts at construction camps and storage facilities. Asphalt, Concrete and Crushing plants are required to obtain a license by NEPA. Borrow Areas and Quarries are subject to the provision of a license by the Department of Mines and Geology and NEPA. The Contractor shall be responsible for obtaining approvals or consents as required by any utility company, in connection with the works or any method of working.

- Sanitary Conveniences. The contractor shall supply proper and efficient sanitary conveniences for staff and ESMPloyees. The contractor shall equip the convenience with water closets of approved pattern and shall maintain them in a proper and sanitary condition. TESMPorary toilet facilities, if not connected to main drainage, shall be of the sealed cess tank type. Tanks shall be monitored for level and ESMPtied regularly. Septic tanks shall not be used. On removal of the convenience the sites shall be properly disinfected and the ground restored.

- Waste water. Sewage and waste water discharges shall be collected and treated in an approved system installed on site. Waste water to be discharged from construction or maintenance facilities must first be filtered or otherwise treated to eliminate obnoxious material likely to cause pollution into natural watercourses or aquifers or endanger indigenous flora and fauna. No waste water or other liquid or obnoxious material will be dumped on wetlands or breeding grounds of aquatic birds.

- Tidiness of the Site. The Contractor shall be responsible for the cleanliness of the site and all public roads, highways and footpaths adjacent to, and affected by, the Contractor's operations at all times. Particular attention shall be given to road cleanliness during the transporting of materials. The Contractor shall provide and maintain neat and sanitary conditions on work sites in accordance with the provisions of the National Public Health Standard. The Contractor shall take every possible precaution against contamination of work site or surrounding areas.

- Equipment Maintenance, Fuelling and Storage Facilities. The Contractor shall ensure that all equipment maintenance, fuelling and storage is performed in such a manner that no fuels, oils, lubricants, chemicals or other toxic materials can gain access to the soil, groundwater and surface water. All tESMPorary and permanent fuel and oil storage areas shall be above ground and isolated within an impermeable bunded area. The volume of the bund shall not be less than the total stored volume capacity. Drainage from such bunded areas shall incorporate oil separators before discharge to existing watercourses. All workshop and equipment storage areas shall be drained such that all surface water runoff passes through an oil separator prior to discharge into watercourses. Suitable fuelling and maintenance areas shall be established at all construction sites. No vehicles or maintenance/re-fuelling areas can be stored or located adjacent to water bodies.

- Spill Contingency Plan. A spill contingency plan shall be prepared before the commencement of site activities and the necessary clean-up agents and equipment in place. All site workers shall be made aware of the emergency planning measures and selected staff should be trained in the handling, storing and disposal of hazardous and polluting substances. Stockpiles of fine materials (e.g. topsoil, cement, sand) shall be covered to prevent the wind erosion of the stockpiles. Such coverings shall be used during periods of inactivity, when access to the stockpile is not required. This includes covering at the end of each working day.

- Waste Management. Solid wastes shall be stored in proper containers, collected and disposed of using public or private companies entrusted with solid waste collection in the communities / parishes where such wastes are collected (camp sites) and disposed in landfills/dumpsites approved by NEPA.

- Asphalt, Concrete and Crushing Plant. Where applicable, the Contractor shall install the asphalt, concrete and crushing plants in flat land, cleared of organic topsoil and with suitable access for vehicles. The locations shall not be near urban or other settlements. The installations shall provide adequate storage space for raw and surplus materials, and ample space for traffic circulation to prevent hindrance to loading and unloading operations. The production plants shall be adjusted to operate in such a manner that, to the extent possible, dust and fumes discharge are kept to a minimum. The plant shall be kept free of human and construction debris and storm water runoff shall not carry contaminants to adjacent watercourses, wetlands or sensitive vegetation. Special measures should be taken to ensure that storm water runoff is adequately treated to prevent the export of contaminants. Such measures may include stilling ponds, silt traps and oil interceptors. In transportation from the plants, trucks shall be fitted with adequate devices to prevent material spillage.

- Site Compound and Stock Piles. The contractor shall ensure that the locations of site compounds and stockpiles, that have the potential to cause the siltation of water bodies, ESMPloy containment devices, perimeter ditching or contour ploughing to prevent erosion or runoff. Compounds and stockpile shall be restored after construction; topsoil removed from the sites shall be stored and used in the restoration/landscaping processes.

- Borrow Areas, Quarries and Haul Roads. The contractor shall be environmentally responsible when making choices relating to the procurement and use of construction materials. Materials shall only be purchased from an approved and licensed quarry. The contractor shall ensure materials are sourced from non-sensitive areas or from sustainable sources. The contractor shall prioritize the use of areas near the road alignment in order to minimize the transportation of materials, preferably use areas which have already been cleared and are of a degraded nature and not use areas within floodplains. Land used for borrow pits and haul roads shall be restored after construction. This shall include final cleaning of the area, restoration of soil horizons, and cover with top soil, contour shape embankments to slow the rate of water run-off, restoration/fencing proposals and re-vegetation plans, including the planting of native and arboreal species. The Contractor shall not extract river stone, gravel, sand or any other construction material from watercourses without approval. The dredging of wetlands and coral reefs for source materials is prohibited. The contractor shall ensure materials to be removed from sites are sent to licensed disposal sites.

- Fire Control Measures. The contractor shall ensure all incineration is banned along road corridors; tESMPorary storage of vegetation occurs only in open areas; and that an emergency fire-fighting plan is drawn up, including the provision of fire extinguishers on site. The burning of materials where unavoidable in open air shall be done under strict supervision of the contractor’s qualified staff. The Contractor shall not burn tires, asphaltic materials, oil or any materials that will produce dense smoke, either for the purpose of incineration or to augment the burning power of other matter. Extreme caution shall be taken to dispose of materials so as to avoid damage or destruction to private property or to cause excessive air pollution.

- Site Restoration. Upon completion of the Contract, all elements of the worksites shall be removed and the site, as far as possible, be returned to its original condition unless designated for alternative uses with approval of the NWA and NEPA.

### Water Pollution Control

- The Contractor will take all precautionary measures to prevent the contamination of water courses, aquifers and the marine environment by pollutant wastes such as chemical products, fuel, lubricants, sewerage water, silt or other obnoxious material.

- Waste water to be discharged from construction facilities must first be filtered or otherwise treated to eliminate obnoxious material likely to cause pollution into natural watercourses or endanger flora and fauna. No waste water or other liquid or obnoxious material will be dumped on wetlands.

- If by accident the contractor dumps, discharges or spills any fuel or chemical product likely to contaminate any watercourse or aquifer, the contractor will immediately take action to arrest and eliminate the pollution caused thereby. The Contractor shall maintain written records of any such incident and of the mitigation measures implemented.

- Construction and maintenance activities on water courses or over aquifers or aquifer recharge areas will be confined to the minimum practicable areas. Activities for construction projects shall be limited to areas shown in the final design drawings and where the erection of temporary or permanent structures has been so designated. temporary fills and scaffolding or false works erected for the undertaking of construction works will be removed from river or watercourse beds immediately upon termination of the construction activities at such locations.

- Frequent crossing by construction equipment over watercourse beds during construction or maintenance activities shall not be permitted. If and when needed, temporary structures will be erected to allow crossings. The use of silt, sand or similar fine graded material which could cause turbidity or sedimentation in the stream shall not be permitted in the construction of stream crossings.

- The contractor shall ensure that where there is a high risk of pollution from construction materials and activities, measures are undertaken to prevent sediment or hydrocarbons entering these areas.

- The contractor shall ensure all bare embankments are planted to create a vegetative ground cover to minimize soil erosion and silt runoff. Provision shall be made to intercept and divert site drainage at short distance intervals, into natural channels, infiltration ditches, settling ponds or areas of suitable and stable ground cover. The contractor shall ensure that there is no siltation of water bodies through direct drainage to water bodies, wetlands or marine areas.

### Erosion and Sediment Control

- To avoid erosion and gullying of banks, the Contractor will schedule his operations so that earthworks are conducted, as much as possible, outside the peak of the rainy season; use gabions and miter drains; and will avoid angle termination at the borders of cuts and fills.

- The work program shall be phased to minimize land disturbance and exposure. Areas of land cleared must be kept to a minimum, and the period of time areas remained cleared to a minimum. Ground cover shall be restored as soon as earthworks permit, and the necessary measures to promote regeneration of native vegetation shall be undertaken.

- The Contractor shall limit the surface area of erodible earth material exposed by clearing, grubbing, excavation, borrow and embankment construction operations in progress commensurate with his capability and progress in keeping tESMPorary erosion control measures and the finish grading, mulching, seeding, and other such permanent erosion and pollution control measures current in accordance with the accepted schedule.

- The Contractor shall provide, install, maintain, remove and dispose of erosion control measures such as gravel filter berms, dykes, catch basin inlet protection, end of pipe filtering devices, silt fences, dams, sediment basins, netting, bale barriers, slope drains and other erosion control devices or methods. The Contractor shall construct riprap aprons to slow down flow and prevent scouring at the outlet of permanent and tESMPorary erosion control measures.

- The Contractor shall incorporate appropriate tESMPorary or permanent erosion control measures (such as mulching, sandbagging, slope drains, silt fences, brush barriers, tESMPorary diversions, etc.) into the work at the earliest possible time. One of the first construction activities shall be the placement of appropriate permanent and tESMPorary erosion and sediment control measures around the perimeter of the work site to protect the work site, adjacent work, adjacent properties, and water resources.

- The Contractor will build tESMPorary and permanent silt traps where the road crosses streams and rivers or where the road traverses close to water courses, using appropriate materials and techniques, including settling ponds, silt fences, filter cloths and small scale wetlands, even if such measures may not be immediately clear from the final designs.

- The Contractor will minimize earthwork in the vicinity of water bodies; work from banks rather than from stream beds and in general reduce contact with watercourses to the minimum. On all bridge approaches the Contractor will sow the faces exposed to inclement weather with local grass or with other local species, especially those that are adapted to local conditions and which grow rapidly and have deep roots.

- Where it is necessary to remove vegetation cover, the topsoil shall be striped immediately after clearing and, if possible, used to rehabilitate the area or an adjacent area that has been disturbed. If this is not possible, soil stockpiles must be located in cleared areas away from drainage lines. Protection of exposed embankments (via silt control devices including silt fences, mulch or tESMPorary plant covers) should be a priority.

- Construction operations in streams and other watercourses shall be restricted to: areas where channel modifications are specified; areas which must be entered into to construct or repair structures or erosion and sediment control measures; and areas where waters must be forded occasionally during construction or maintenance operations. Frequent fording will not be permitted.

- The Contractor shall inspect the erosion control measures daily and after heavy rain episodes and implement corrective action immediately when existing measures are not working. He shall maintain erosion control measures in working condition until they are no longer needed or, for permanent measures, until the project is completed.

- The contractor will take all necessary actions to maintain the available silt holding capacity of silt traps and barriers until no longer needed. The Contractor shall remove and properly dispose of the sediments or other materials collected by silt fences and check barriers. Prior to removal of the barriers, the contractor shall request concurrence from the Engineer. The contractor shall remove and replace hay bales which have deteriorated and filter stone or fabric which has become dislodged.

### Drainage and Water Resources Protection

- Natural drainage shall be maintained wherever possible. The Contractor shall conduct the work in such a way that adequate drainage is provided at all times. At no time shall existing functioning storm drains, gutters, ditches and other runoff facilities be obstructed unless appropriate measures are taken to manage the runoff.

- Construction activities on watercourses or over aquifer recharge areas will be confined to the areas shown in the final design drawings and where the erection of tESMPorary or permanent structures has been so designated. Crossing by construction equipment over watercourse beds during construction activities will not be permitted.

- In the demolition of existing structures and the construction of new ones, the contractor shall ensure that no rubble or other surplus construction material is dumped into existing watercourses or aquifer recharge areas.

- TESMPorary fills and scaffolding or false works erected for the undertaking of construction works will be removed from river or watercourse beds immediately upon termination of the construction activities. The use of silt, sand or similar fine graded material which could cause turbidity or sedimentation in the stream shall not be permitted in the construction of stream crossings.

- A buffer zone of 10-20 m shall be kept between the edge of ditches and all watercourses. Where space restrictions exist, the maximum separation practicable shall be effected. An erosion check dam, constructed with hay or straw bales or as specified, shall be installed at the end of the ditch, with appropriate erosion control measures and structures installed further up the ditch.

- Ditches should be directed into the surrounding vegetation (where possible), rather than discharging directly into natural watercourses.

- Where construction of a ditch would result in the modification of the hydrology of a wetland, consultation with NEPA representatives will be undertaken before any action that would affect the wetland is taken.

- The contractor shall not excavate borrow ditches or pits on flat lands subject to water stagnation and/or with slow runoff drainage, and in proximity of villages or urban settlements.

- Removed materials and sediments shall be deposited at a location at least 30 meters away from any watercourse and such that it cannot wash into a watercourse. Suitable material shall be used when needed to fill washouts, depressions and the like on fore slopes and back slopes. These materials can also be used, in conjunction with seeding, to construct berms to manage runoff and to flatten slopes.

- Grading shall be done so that no ridges are left along the edge of the road that would prevent sheet runoff.

- The Contractor shall remove any debris or materials blocking the flow of water through drainage structures, and dispose of such material in an environmentally safe manner at an approved disposal site. All suitable measures must be undertaken to prevent the material from entering any watercourses or sensitive resources.

### Dust and Air Pollution Control

- All appropriate measures to minimize the generation of dust as a result of works, operations and activities must be undertaken. Such measures shall include regular and effective treatment of gravel roads and working areas, use of dust extractors on drilling equipment or wet drilling equipment or wet drilling, water sprays on aggregate crushing and screening plants, etc.

- The Contractor shall control dust primarily by watering all dust emitting surfaces within the limits of the works, including the worksite, materials sites, and unpaved roads used by construction equipment. The Contractor shall use potable water, or acceptable water of quality, for dampening dust-emitting surfaces, such that application to such surfaces will not cause contamination of the surrounding environment or pose a bigger threat from contaminated fugitive dust after drying of the surfaces.

- The contractor shall ensure public roadways are kept free of mud. The contractor shall ensure construction activities in close proximity to commercial and residential areas are carefully monitored to minimize impacts from vehicle emissions and dust.

- The Contractor shall minimize dust generation during grading, gravelling and marling by wetting the road and shoulders surface at appropriate intervals.

- The Contractor shall minimize dust emissions by programming his work to minimize the size of dust-emitting surface areas exposed at any given time, particularly areas of excavation and embankment and other areas cleared of vegetation. Disturbed areas must be restored as soon as practicable.

- The Contractor shall propose, and adopt, methods for reducing dust arising from the operation of any batching plant or crusher plant. Construction activities, including blasting, shall be limited on windy days.

- The Contractor shall control dust within the maintenance, construction and associated works sites at all hours, including nights, week-ends and other non-work periods.

- The dump trucks to be utilized in the transportation of surplus material shall be fitted with tarpaulins to prevent dust or other material spilling while being hauled. Construction materials shall be watered prior to loading and transport. Dust control measures may also include the application speed restrictions on site.

- Coating and road (traffic) marking materials used by the Contractor must be from an approved list of suppliers and comply with technical specifications and with the requirements related to volatile organic compounds and lead. These materials must not exude fumes which are toxic or detrimental to persons, livestock, wildlife and property and the environment in general.

- The contractor shall undertake measures to minimize the production of emissions through good practice techniques and ensuring equipment is maintained and functioning correctly. Equipment and vehicles shall be fitted with appropriate emission controls; and all equipment and vehicles shall be serviced regularly to minimize emissions.

- The contractor shall monitor air quality to ensure that activities are not causing a statutory nuisance or breathing hazard.

### Noise and Vibration Control

- The contractor shall undertake measures to minimize the production of noise and vibration through good practice techniques and ensuring equipment is maintained and functioning correctly and is performing within the permitted noise levels for the device.

- The contractor will comply with the following construction noise standards (with the exclusion for blasting) and assure that noise increment does not exceed background noise levels in more than 10%.

| Construction Noise Standard |  |  |  |
| --- | --- | --- | --- |
| Time | Noise levels in dBA L10 | Noise levels in dBA L95 | Noise levels in dBA Lmax |
| 0530-0730 | 60 | 45 | 70 |
| 0730-2000 | 70 | 55 | 85 |
| 2000-0530 | 55 | 40 | 75 |

- The Contractor will keep equipment in such a condition that they comply with the following maximum criteria:

| Noise From Construction Equipment |  |  |
| --- | --- | --- |
| Equipment | Typical Noise (dBA) | Range (dBA) |
| Portable Air Compressor | 81 | 74-87 |
| Concrete Mixer | 85 | 81-89 |
| Jackhammer | 88 | 81-89 |
| Pneumatic Tools | 85 | 82-89 |
| Generator | 76 | 71-82 |
| Pile Driver | 101 | 95-106 |

- The Contractor will use the correct equipment for each job in order to minimize noise intrusion. The Contractor shall provide working machinery and equipment designed to operate with the least possible noise, including the use of shields, soundproof housings, enclosures, or other physical barriers to restrict the transmission of noise, and when gearing is used, such gearing shall be a type designed to reduce noise to a minimum. Compressors shall be equipped with silencers on intake lines. Gas or oil operated equipment shall be equipped with silencers or mufflers on intake and exhaust lines.

- In populated or urban areas all noisy operations such as piling, transportation of material, earthworks and bulldozing will be carried out during daylight hours (730-2000) only, Monday through Saturday. Construction activities in close proximity to residential areas shall be carefully monitored to minimize impacts. Residents and business owners shall be notified of construction activities, hours of operation and the potential noise levels prior to construction.

- Blasting will be conducted in a safe manner with charges large enough to break up material, but not so large as to damage adjacent structures and threaten human safety from shock and debris. Blasting will take place only during daylight hours, once a day, and at a time set in consultation with the inhabitants of the nearest by community.

- The Contractor shall notify users of facilities of restrictions prior to blasting; design shot-plans to minimize environmental impact and impact on marine fauna; secure areas prior to blasting and inspect the area immediately afterwards to visually monitor for any incidents; monitor and analyze noise and vibration results and adjust construction practices as required.

### Waste Management

- The contractor has title to all waste material for disposal. All waste spoil and materials shall be stored in a safe manner and disposed of at an approved landfill site. The disposal of materials within the working areas shall comply with all environmental constraints.

- Waste management measures include: minimize the production of waste materials; reuse and recycle materials (for example, mulch vegetation, stabilize and reuse pavement materials), recycle oils; separate and recycle solid wastes generated by construction activities, from offices and mess rooms; ensure all fuel and chemicals are appropriately contained and bunded in safe locations; site storage depots away from watercourses and danger areas, areas prone to flooding, or tidal areas.

- Surplus or waste materials shall not be deposited in watercourses or open areas, or on private land. To the extent possible, such surplus material shall be used in the backfilling of tESMPorary quarries or in embankment construction if the quality so warrants.

- The Contractor shall promptly remove accumulated soils and other materials resulting from construction and maintenance operations from roadways and other areas of the ROW to ensure safety of the road users and prevent the pollution of natural waterways and wildlife habitats. He shall dispose of these materials at an approved location.

- The Contractor shall abstain from dumping any material, resulting from maintenance activities (ditching, patching, grading, gravelling, clearing of land-slips, etc.) along the roadside or onto adjacent properties. Used pavement material shall not be disposed on the road side, in watercourses or wetlands. Asphaltic cement removed from the site should be re-used or disposed of in a controlled manner.Suitable material can be used in conjunction with seeding for slope flattening or berm construction.

- Whenever large amounts of asphaltic material are to be removed from a road or highway, the material shall be reused, hauled to an approved facility for processing and future reuse, or disposed of by burial, at least at one meter depth, outside the project site and urban settlements at a approved location.

- The Contractor shall collect and dispose of all waste generated in the removal or repair of deteriorated components.

- The Contractor shall take all necessary precautions to prevent discharge or loss of any harmful material or substance into the watercourse, including but not limited to lead-based paints, hydrocarbons, fresh concrete, and preservatives.

- The Contractor shall collect in a leak-proof container for recycling or safe disposal all solvents used in the cleaning of painting equipment. Disposal of this material must be done in accordance with NEPA Guidelines for disposal of industrial waste.

- The Contractor shall exercise due care to minimize disturbance to watercourses and wetlands. The Contractor shall exercise due care when resurfacing bridges located over watercourses to prevent asphalt from entering the watercourse. Where the danger exists of liquid asphalt seeping through the deck into the watercourse and drain pipes, etc., some measure such as draping with plastic, polyethylene or other suitable material shall be implemented.

- The Contractor shall make the necessary arrangements for the timely disposal of sewage and solid wastes resulting from his operations. Where necessary, depending on the duration of work and the nature of the waste material in a particular site, he shall provide garbage receptacles (bags or bins) at convenient locations within the worksite.

- The Contractor shall collect materials removed (scrap metals, etc.) or replaced and dispose of them in an environmentally safe manner at an approved site.

- The Contractor shall exercise extreme care to minimize the spillage of preservatives used in the treatment of wood or metals. He shall dispose of ESMPty containers (can, bottles) in accordance with environmentally responsible industry practices.

### Vegetation and Fauna Protection

- The Contractor shall reduce loss of vegetative cover, erosion and sedimentation to the minimum possible by: (i) avoiding any off-the road driving outside the immediate construction areas; (ii) keeping process areas as small as possible; (iii) reducing damage to vegetation through environmentally sound construction practices; (iv) where roadside banks are cut and exposed, protecting the banks from gully erosion and sedimentation into nearby streams; (v) providing sediment traps and screens where needed; (vi) excavating bulk fill from areas so as to minimize erosion, reduce visual impact of scarring and allow for rapid revegetation following excavation.

- All topsoil shall be stripped and stored correctly and then used as topsoil in re-vegetation and landscaping schemes. Soils for use in landscape planting may need to be imported due to the limited amount of topsoil available on the island. The Contractor shall ensure preservation of topsoil by not stripping under wet conditions. Topsoil and subsoil shall be piled and stored separately to avoid mixing.

- In areas of native vegetation, return topsoil and mulched vegetation to approximately the same area of the road it came from; reuse cleared vegetation, where possible, for seed collection, milling, chipping or mulching for use on roadside batters, or firewood; take special care with works within protected areas, for example, National Parks or Marine Protected Areas; mark the areas to be cleared.

- The Contractor shall not damage any vegetation designated to remain after road works or vegetation outside the approved construction zone; keep access tracks to marked areas; confine vehicle movements to the old or new alignment where feasible; place site depots, equipment compounds and stockpile areas on previously cleared areas away from trees, bushes and native grasses; avoid work within the drip-line of trees to prevent damage to the tree roots; store fill, materials and equipment away from trees to avoid compacting the soil and preventing air and water reaching the tree roots; limit the removal of topsoil and cleared vegetation from the site to reduce the risk of spreading weeds and diseases.

- The contractor shall ensure that the existing vegetation outside the right of way remains undamaged by the works. Specifically the contractor shall not: stockpile material within drip line; allow traffic, vehicles or equipment to compact the soil within the drip line; cut major tree roots. The contractor shall: tunnel under or around roots by hand digging so as to minimize damage to them; prune interfering branches and treat with a suitable dressing; treat any damaged roots over 2,5 cm in diameter, immediately, with a suitable tree paint.

- The Contractor shall make all reasonable efforts to preserve all vegetation adjacent to wetlands and other water resources. The Contractor shall avoid the removal of vegetation that would result in the creation of erosion-prone areas. Where vegetation removal is necessary, the Contractor shall implement all applicable erosion control provisions.

- Existing vegetation within the right of way shall be retained as much as possible, and shall be aided by: erecting site fencing around drip line of trees within the working area; erecting tESMPorary fencing around broadleaf coppice and woodland edges to protect from encroachment, illegal dumping and damage from machinery; avoiding stockpiling material within drip line; avoiding traffic, vehicles or equipment to compact the soil within the drip line; avoiding cut major tree roots; tunnelling under or around roots by hand digging; pruning interfering branches and treat with a suitable dressing; treating any damaged roots over 2,5 cm in diameter, immediately, with suitable tree paint.

- The Contractor shall restore ground cover as soon as earthworks permit, and will take the necessary measures to promote regeneration of native vegetation, even if such measures may not be immediately clear from the final designs. These measures may include the use of degradable geotextile, the use of seeds or live stakes of native grasses, revegetating with local shrub and tree species (especially those characterized by rapid growth and deep roots) and the reuse of top soil and mulch. The Contractor will replant shade trees and screening vegetation, as designated in the specific environmental mitigation plan.

- The contractor shall ensure that cleared vegetation is chipped and used in landscaping schemes. Body wood shall be cut in appropriate lengths and stacked along the ROW. Cleared vegetation that cannot be used in landscaping schemes shall be disposed of at an approved site. The contractor shall ensure that landscaping and planting occurs as soon as possible after the completion of each section of the road.

- The Contractor shall dispose of all the vegetation removed during construction or maintenance operations (trees, branches, stumps, roots, buried logs, shrubs, brush, and other vegetation in an approved disposal site. Burning of these materials and other roadside debris shall not be permitted because it would jeopardize health and safety. The Contractor shall perform all vegetation control or removal by manual or mechanical means. The use of herbicides will not be permitted.

- The Contractor shall protect fauna living within the site and ensure that hunting, trapping, shooting, bird-nesting or egg-collecting does not occur. Certain species of birds are known to be rare and endangered. The Environmental Engineer will identify such species to the Contractor, who shall take steps to inform his ESMPloyees that such species of birds are to be protected. Only licensed fishermen will be allowed to fish and then only in approved locations.

### Health and Safety

- The Contractor will provide medical examinations and emergency medical care for the construction crew and will provide suitable and clean sanitary facilities and necessary safety equipment, including safety (dust) masks covering nose and mouth, ear plugs, hard boots, safety vests, hard hats, goggles and raincoats for ESMPloyees when applicable and take measures to assure that this safety equipment is used at all appropriate times.

- The Contractor shall provide first-aid facilities and supplies and emergency medical care for the construction crew and where work will extend more than 30 days he shall arrange for “on call” medical services and facilities.

- The Contractor shall at all times have transportation on site or readily available to the workers in case of emergencies.

- The Contractor shall take all necessary precautions for the protection of life, health and the general occupational welfare of the persons authorized to be in the site. Safety measures must be taken to prevent accidents involving workers and members of the community, such as speed control, work areas signaling, safety devices (barriers).

- The Contractor shall place road markers, warning devices and appropriate delineation and fencing along active worksites to safeguard the safety of road users and workers. Barricades or tESMPorary fencing shall be provided in order to guarantee public safety and the security of the works.

- The contractor will implement adequate pedestrian safety measures to reduce accident risks, install signboards and other safety devices, and provide a clear and unrestricted pedestrian path for the movement of pedestrians around the works.

- The Contractor shall provide trucks and earth moving equipment, as necessary, equipped with functioning back-up warning devices. Vehicle loads must comply with their capacity and shall not exceed it under any circumstances.

- Drivers assisting the construction process that demonstrate a lack of safety while driving shall be subject to warnings. Drivers assisting the construction process shall be trained to perform spill reporting and clean-up procedures for minor spills.

- The Contractor shall not store or allow the storage of materials in any way that impedes movement along the roadway, that present safety hazards to the users or pedestrians or prevents movement of pedestrians. When this is totally unavoidable, the Contractor shall obtain authorization from the Environmental Engineer, install warning signs and if necessary, in conjunction with NWA, provide an alternative safe route.

- The contractor’s operations shall offer the least possible obstruction and inconvenience to the public and shall have under construction no greater length or amount of work that they can properly manage at a time due regard to the rights of the public in maintaining a free flow of traffic.

- The contractor shall comply with special requirements of all Utility Corporations and Companies imparted via communications during the period of the Contract. The contractor shall ensure that proprietary service detection equipment is constantly available on site. Where any service is found on exposure to be damaged or where damage is done by the contractor to any drain, land drain, pipe or any other service as a result of the contractor's operations, he shall immediately notify the PET and the utility concerned. The contractor shall make sure all damage done to public or privately owned services meet the cost of repairing such damage. The approval of the owner must be obtained before any repair work, whether tESMPorary or permanent, is commenced.

### Community relations

- The Contractor shall follow a work schedule, procedures and rules of conduct conducive to good community relations during construction. This will entail adequate notification of work schedules for local communities, and hiring practices which provide opportunities for local labor. Communities must be given adequate notice of intended construction and potential for dust and blockage of access to roads or community facilities during construction.

### Protection of Cultural and Natural Heritage Features

- Although no archeological sites are expected to be found in this Project, the Contractor shall allow archaeological and cultural inspections during construction to facilitate the protection of cultural and historical sites. If, during construction, an indigenous heritage or burial site is discovered, the Contractor must stop work immediately and notify the Environmental Engineer.

- Historical and cultural sites are also protected and must have the approval from the Jamaica National Heritage Trust before commencement of works.

### Relocations

- Although all works will be performed within the right-of-way, and will not affect private properties, the Contractor must be aware that there might be some encroachments along the road sections, as it is common to see along road network. Most encroachments are fences or some small business constructions, but there might also be some dwellings. If such encroachments are found within the right-of-way of the road sections/works to be rehabilitated, these constructions must be relocated.

- Where people or structures must be relocated, a consultation and negotiation process that leads to a contractual agreement and relocation or cash compensation must be undertaken. This process must offer a fair and timely compensation for loss of goods and dwellings, based on legal regulations.

- As the number of families estimated to possibly undergo relocation might be very small it is not necessary to prepare a Resettlement Plan in order to comply with the Bank’s resettlement safeguard requirements, but the basic criteria has to be applied. This criterion defines two compensation options: resettlement of these families in new properties, served with basic infrastructure services and nearby the current locations of residence; or financial compensation through indemnification.

- In addition, a social monitoring must be performed throughout the resettlement process, in order to mitigate any negative social impacts. These activities shall be implemented by the NWA and the local governments.

- Besides the criteria above described, basic good practices will be adopted, such as: (i) works in such areas will only initiate after affected families have been relocated; (ii) communities' concerns and requirements will be consider; (iii) negotiations terms that prevent the families to return to their daily activities will not be imposed; (iv) population will not be pressure by works schedule in case some delays occur in the resettlement process.

APPENDIX 1

ENVIRONMENTAL MONITORING FORM

| Project Site | Date: |
| --- | --- |
| Brief description of construction activities at the site |  |
| Monitored by: | Verified by |

| ENVIRONMENTAL SPECIFICATIONS & GUIDELINES | COMPLIANCE RATINGS |  |  |  |  | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |  |
| BORROW PITS AND QUARRIES |  |  |  |  |  |  |
| Acceptable site selection |  |  |  |  |  |  |
| Working plan (top soil, trees retained) |  |  |  |  |  |  |
| Restoration plan (agreed/implemented) |  |  |  |  |  |  |
| No pollution /siltation of adjacent watercourse |  |  |  |  |  |  |
| Erosion control measures |  |  |  |  |  |  |
| No avoidable nuisance (dust, noise, traffic) |  |  |  |  |  |  |
| FUEL & MATERIALS STORAGE |  |  |  |  |  |  |
| Protected and bunded hard standing |  |  |  |  |  |  |
| Fire precautious in place |  |  |  |  |  |  |
| Runoff collection drain/oil interceptor |  |  |  |  |  |  |
| Acceptable tESMPorary materials storage |  |  |  |  |  |  |
| Secure fencing around and within site |  |  |  |  |  |  |
| REHABILITATION WORKS |  |  |  |  |  |  |
| Maximum working corridor of 25m |  |  |  |  |  |  |
| Adequate roadside drainage |  |  |  |  |  |  |
| Spoil tip slopes less then 1:4 |  |  |  |  |  |  |
| Road safety signs in place |  |  |  |  |  |  |
| DITCHING AND CHANNELING |  |  |  |  |  |  |
| Buffer zone of 10-20m or max. allowable maintained between ditch edges and all watercourses |  |  |  |  |  |  |
| Erosion check dam at end of ditch with erosion control measures and structures further up ditch |  |  |  |  |  |  |
| Natural drainage where possible |  |  |  |  |  |  |
| Ditches directed to surrounding vegetation where possible |  |  |  |  |  |  |
| Ditches cleared routinely or when channel capacity is reduced by 33% |  |  |  |  |  |  |
| Discards deposited materials at least 30m from any watercourse |  |  |  |  |  |  |
| EROSION CONTROL |  |  |  |  |  |  |
| Minimize loss of vegetative cover, erosion, and sedimentation |  |  |  |  |  |  |
| Accumulated soils and materials removed from roadways and other areas of the right-of-way |  |  |  |  |  |  |
| Avoids that which causes deposition/spillage of undesirable or potentially harmful wastes into streams or watercourses |  |  |  |  |  |  |
| Construction materials, fuels, lubricants and other chemicals appropriately stored without access to roadway and adjacent waters |  |  |  |  |  |  |
| Prevents sediment transport and keeps hazardous/polluting liquids from entering highway & waterways |  |  |  |  |  |  |
| Avoids erosion/gullying of banks |  |  |  |  |  |  |
| Earthwork minimized by river/stream |  |  |  |  |  |  |
| Abstains from dumping material along roadside or adjacent properties |  |  |  |  |  |  |
| For excavation or fill, uses appropriate erosion control measures as soon as possible |  |  |  |  |  |  |
| Sediment retention structures at places of high runoff concentrations |  |  |  |  |  |  |
| Riprap aprons present at outlet of erosion control measures |  |  |  |  |  |  |
| Erosion control measures inspected and maintained daily, after heavy rain falls and when necessary |  |  |  |  |  |  |
| Proper disposal of materials collected by silt fences and check barriers |  |  |  |  |  |  |
| Limits surface area of erodible earth material exposed by construction |  |  |  |  |  |  |
| HABITAT & LANDSCAPE PROTECTION |  |  |  |  |  |  |
| Access through sensitive areas agreed |  |  |  |  |  |  |
| No alteration of river courses |  |  |  |  |  |  |
| No undue clearance/drainage impact |  |  |  |  |  |  |
| Trees, valued landscape features retained |  |  |  |  |  |  |
| Native vegetation maintained |  |  |  |  |  |  |
| No rubbish, un-restored soil tips |  |  |  |  |  |  |
| Vegetation preserved or appropriately disposed of when necessary |  |  |  |  |  |  |
| No waste water/liquid or obnoxious material dumped on wetlands, breeding grounds of aquatic birds, natural watercourses, aquifers, or endanger indigenous flora and fauna |  |  |  |  |  |  |
| AIR POLLUTION & DUST CONTROL |  |  |  |  |  |  |
| Watering of unpaved areas to avoid dust generation |  |  |  |  |  |  |
| Uses only potable water or water of acceptable quality for watering |  |  |  |  |  |  |
| Dump trucks fitted with tarpaulins during transport |  |  |  |  |  |  |
| Phasing of vegetation removal to minimize exposure of soil |  |  |  |  |  |  |
| Appropriate coating and road marking that do not exude fumes harmful to life and environment |  |  |  |  |  |  |
| NOISE |  |  |  |  |  |  |
| Notifying community on work activities schedule |  |  |  |  |  |  |
| Breaks in noisy activity to reduce stress levels |  |  |  |  |  |  |
| Work done during normal working hours |  |  |  |  |  |  |
| Noise impact minimized |  |  |  |  |  |  |
| WATTER POLLUTION |  |  |  |  |  |  |
| Prevents contamination of water courses, aquifers, and marine environment |  |  |  |  |  |  |
| Installation of tESMPorary drainage system |  |  |  |  |  |  |
| Maintenance of drainage system |  |  |  |  |  |  |
| Stock piles protected during rain to prevent erosion |  |  |  |  |  |  |
| Installation of sediment control measures |  |  |  |  |  |  |
| Maintenance of sediment control measures |  |  |  |  |  |  |
| No rubble or surplus material dumped in watercourses/aquifer recharge area |  |  |  |  |  |  |
| Minimize construction/maintenance over watercourses |  |  |  |  |  |  |
| Avoid frequent crossing of equipment over watercourses |  |  |  |  |  |  |
| WASTE MANAGEMENT |  |  |  |  |  |  |
| No pollution risk to surface/groundwater |  |  |  |  |  |  |
| Appropriate hazardous waste disposal |  |  |  |  |  |  |
| Adequate refuse/scrap disposal/ clean-up |  |  |  |  |  |  |
| Prompt and environmentally safe disposal of waste materials |  |  |  |  |  |  |
| Proper disposal of used pavement material |  |  |  |  |  |  |
| Large amounts of asphaltic material properly reused or disposed of in approved landfill site. |  |  |  |  |  |  |
| Garbage receptacles at worksite |  |  |  |  |  |  |
| Toilet and sanitation facilities |  |  |  |  |  |  |
| CULVERTS AND DRAINS |  |  |  |  |  |  |
| Adequate drainage maintained |  |  |  |  |  |  |
| Approved disposal of debris or materials blocking water flow |  |  |  |  |  |  |
| Appropriate erosion control installed |  |  |  |  |  |  |
| Prevents obstruction of existing storm drains, gutters, ditches, and runoffs |  |  |  |  |  |  |
| MINOR BRIDGE MAINTENANCE |  |  |  |  |  |  |
| Appropriate disposal of wastes |  |  |  |  |  |  |
| Prevents discharge/loss of harmful material/substances into watercourse |  |  |  |  |  |  |
| Collects solvents in leak-proof container for recycling/safe disposal |  |  |  |  |  |  |
| OCCUPATIONAL HEALTH & SAFETY |  |  |  |  |  |  |
| Roadway maintained free of obstruction and safety hazards |  |  |  |  |  |  |
| Emergency and accident plan (safety officer) |  |  |  |  |  |  |
| Immediate actions taken to arrest and eliminate incurred pollution |  |  |  |  |  |  |
| Organization and tidiness of site |  |  |  |  |  |  |
| Adequate street lighting and warning signs |  |  |  |  |  |  |
| Protects life, health & general welfare |  |  |  |  |  |  |
| Workers appropriately provided with safety equipment |  |  |  |  |  |  |
| Suitable and clean sanitary facilities and clean potable water |  |  |  |  |  |  |
| First-aid, supplies, and access to emergency medical care |  |  |  |  |  |  |
| Road markers, warning devices and appropriate delineation and fencing |  |  |  |  |  |  |
| Additional Comments: |  |  |  |  |  |  |

Key: 1= Non Compliance; 2= Poor Compliance; 3= Fair Compliance; 4 = Good Compliance; 5=Full Compliance

Environment and Safeguards Compliance Policy (OP-703); Involuntary Resettlement (OP-710); Natural Disaster (OP-704); Indigenous People (OP-765); Disclosure of Information (OP-102)

Environment and Safeguards Compliance Policy (OP-703); Involuntary Resettlement (OP-710); Natural Disaster (OP-704); Indigenous People (OP-765); Disclosure of Information (OP-102)

Monitoring requirements are listed in the Environmental Monitoring Form (Appendix 1).