JAMAICA 
TRANSPORT INFRASTRUCTURE REHABILITATION PROGRAM

ENVIRONMENTAL AND SOCIAL IMPACT REPORT

I. INTRODUCTION	3

II. PROGRAM BACKGROUND	3

Transport Sector	3

III. PROGRAM DESCRIPTION	4

Program Execution	6

Environmental Management	6

IV. LEGAL AND INSTITUTIONAL FRAMEWORK	7

Transport Sector	7

Environmental Sector	9

V. ENVIRONMENTAL AND SOCIAL CONDITIONS	12

Transport and Road Network	15

VI. ENVIRONMENTAL AND SOCIAL IMPACTS	17

Potential Impacts during Construction Phase	18

Potential Impacts during Operation Phase	20

VII. MITIGATING MEASURES AND ENVIRONMENTAL MANAGEMENT PLAN	20

Mitigation Measures	20

Environmental and Social Management Plan	24

Traffic Management Plan	25

Environmental Management Implementation Plan	26

VIII. MONITORING AND SUPERVISION	27

IX. RECOMMENDATIONS	27

Abbreviations

GoJ	- Government of Jamaica

ITA	- Island Traffic Authority

JUTC	- Jamaica Urban Transit Company Limited

MBM	- Montego Bay Metro Limited

MLG	- Ministry of Local Government, Community Development & Sport

MOFPS	- Ministry of Finance and Public Services

MTW	- Ministry of Transport and Works

NEPA	- National Environment and Planning Agency

NROCC	- National Road Operating and Constructing Company

NRSIP	- National Road Service Improvement Program

NWA	- National Works Agency

PEU	- Project Executing Unit

PIOJ	- Planning Institute of Jamaica

RMFB	- Road Maintenance Fund Board

TA	- Transport Authority

## I. INTRODUCTION

- This report was prepared based on information provided by the Ministry of Transport and Works, the National Works Authority, the National Environmental Protection Agency, website information and other related documents, as well as other IDB Project Documents.

- During the period of April 8 to 11, 2008, The IDB team - Ms. Ione Müller, the environmental consultant, Mr. Brian McNish, Mr. Alejandro Taddia, Mr. Dennis Chong, and Mr. German Silva –held meetings with: (i) Ms. Barbara Scott and Mr. Renaldo Thompson, of the Planning Institute of Jamaica (PIOJ), and Ms. Jacqueline Brown, and Mr. Howard Duval from the Ministry of Finance and Public Service (MoF&PS); Mr. Joseph Hubbert, Minister of State; Minister Michael Henry, Ms. Elsa Biuns, Ms. Valerie Simpson, Mr. Shawn Grey, and Dr. Janine Darkins from the Ministry of Transport and Works (MTW); Mr. Milton Hodelin, Mr. Patrick Rose, and Mark Richards from the National Works Authority (NWA); and Mr. Victor Brown, National Road Service Improvement Program (NRSIP) Coordinator.

- A site visit was performed on April 10, to the following roads: Yallahs bridge, under construction; stretch from Poor Man’s Corner to Easington bridge, where NRSIP road maintenance activities are in progress, and where there are flood damage river training works proposed to be performed under the emergency loan; Roselle area, where flood damage sea defense project is proposed, also under the emergency loan. It was not under the consultant´s scope of work to review or evaluate results of NRSIP implementation.

- Also on April 10, a meeting was held with National Environmental Protection Agency (NEPA) representatives – Winsome Townsend, Janet Hyde, Michelle Grant, Marc Rammechere, and Frances Blair – in order to clarify environmental permitting and licensing procedures applicable to the Project.

## II. PROGRAM BACKGROUND

- Jamaica is island country in the Caribbean Sea, which has a dense road network, but in poor conditions due to the lack of adequate maintenance, exacerbated by natural disasters such as hurricanes and heavy torrential rains and flooding.

- IDB has funded two road construction and rehabilitation projects in Jamaica: National Road Service Improvement Program - NRSIP (Loan 1363/OC-JA) and the Northern Coastal Highway Improvement Program - NCHIP (Loan 972/OC-JA). Since January 2008, the Government of Jamaica negotiated with IDB a new rehabilitation program, due to the damages caused by Hurricane Dean, in 2007. This new program was divided in two parts: a first emergency loan for immediate repairs to the road network; and the second part which consists of this Transport Infrastructure Rehabilitation Program.

### Transport Sector

- The road network in Jamaica consists of some 22,000 km, including all the paved main roads, the parochial roads and the farm and housing scheme roads (paved and un-paved).

- The National Works Agency (NWA) is responsible for just under 4,780 km of main roads and a further 220km of urban roads. The Ministry of Local Government, Community Development & Sport (MLG) being responsible for the remaining 12,000 km of Parochial roads and a further 5,000 km of farm and estate roads.

- The lack of appropriate and timely maintenance has resulted in the continued degradation of Jamaica's road network and subsequent increase in cost of the transportation of goods and people. This associated to the impact of heavy rains and hurricanes, and the impact of overloaded commercial vehicles, have resulted in the extensive degradation of 50% of roads in Jamaica.

- The majority of road failures are due to poor drainage of the pavement. Inadequate attention is currently given to resolving this problem of avoiding the ingress of water into the subgrade which is fundamental to accelerating the deterioration of the pavement structure. Any maintenance plan, and its consequent funding, needs to include sufficient money to maintain all the elements noted above, roads, culverts, bridges, drainage channels and rivers.

- Current estimates of condition of the road network is that of the total main and parochial roads, between 30%-60% are currently in poor or very poor condition to the extent that the road network in part the extent that the road network in part has been rendered unreliable. Some roads are closed which result in long journey times and high transport costs, effectively limiting the mobility of the nation. The attendant dislocation and social difficulties manifest in traffic congestion, difficulty for children to reach to school, difficulty accessing other basic social services such as health, public administration and other economic spaces such as markets and jobs.

- The high density of roads associated to steep slopes, fragile soils and sensitive areas, hurricanes and heavy torrential rains, followed by floods, requires a large sum of funds for road maintenance and rehabilitation. A balanced maintenance program must include both routine and periodic maintenance of roads, culverts and bridges, and the drainage channels and rivers. Periodic maintenance will extend the life of the road network and result in a reduction of whole life costs and ultimately save the country money.

## III. PROGRAM DESCRIPTION

- Objective. The general objective of the Transport Infrastructure Rehabilitation Program (TIRP) is to support the restoration of Jamaica’s transportation system through the rehabilitation of vital portions of the primary, secondary and tertiary road network which have been damaged due to storms and to help fillip economic activity and improve the quality of life of the Jamaican population by the delivery of transportation services on a continuous, safe and reliable basis. The specific objectives include: (i) restoring essential portions of the road system and ensuring continuity of the primary network; (ii) active public investment in construction to create jobs; (iii) help decrease travel time for transportation system users, which will in turn benefit access to social services and economic spaces.

- Program Components: The program will be implemented over a five-year term and will include three components (i) Civil works will support the rehabilitation and improvement of selected primary roads on the main road network (ii) Project Implementation Support will center on supporting staff for the project execution unit, and in providing ancillary (environmental and engineering) professional support (iii) Technical studies will provide support for technical assistance and studies to inform future supplementary interventions. Each component is described in detail below.

- Component 1 – Civil Works (US$17.2 million). Under this component roads under the purview of the NWA will be selected for rehabilitation using HDM for the economic analysis and prioritization based on the surface condition and traffic volume. In addition projects must:

- be part of the national main road network system comprising 5006km

- have a budget allocation provided for the corresponding year

- be economically feasible, i.e., the economic rate of return shall be at least 12%

- be environmentally feasible thereby complying with the country’s environmental legislation and Bank Policies

- have final designs prepared  and have incorporated mitigation measures to prevent correct or compensate direct environmental impacts.

- have clearance of the right of way” before signing the contract for civil works

- The scope of work in this component is expected to fall into two groups (i) routine maintenance comprising simple works consisting of patching, milling, sealing cracks, repairs to longitudinal drains, road markings and medians. It will include repair of surface distress, pavement strengthening and reduction of roughness, reconstruction of inadequate drainage facilities, and retaining walls which will only require simple standard designs and the implementation of the Environmental and Social Management Plan; and (ii) periodic maintenance comprising bridges restoration, river training,  embankment stabilization and sea defense works as well as improvement of geometrical and structural characteristics requiring detailed designs and more important engineering and environmental studies prior to construction.

- Component 2 – Project Implementation Support (US$ 2.7 million) This component will directly support the hiring of the project manager, the project accountant and an environmental expert to manage the day to day technical, socio-environmental, financial and administrative aspects of the project and the hiring of an engineering and supervisory consultant firms on a competitive basis to monitor and oversee the implementation of the proposed construction of the civil works. This component will also provide resources for minor office equipment and the hiring of financial and technical and environmental auditors and a monitoring and evaluation consultant to respectively conduct the audits and to track and monitor the results of the operation.

- Component 3 – Technical Studies (US$ 2.1 million). Provides resources for studies to strategically address key issues that will assist in continuing the works to improve road transport policy and regulation covering inter alia: the RMMS, and alternative sources of funding to augment the road maintenance fund. Funding will support the hiring of a consultant to assist the Project coordinator in determining, managing and implementing strategic interventions to strengthen the governance of road maintenance. Also at the request of Government this component provides resources to undertake engineering and environmental feasibility studies pertinent to the rehabilitation of future restoration or improvement of road sections. Amongst the possible studies Jamaican authorities have considered developing feasibility studies for the Southern Highway Corridor focusing on continuing the main road-network circumscribing the island in keeping with GOJ’s strategic development plan for the south coast of the island. NWA prepared a list of proposed works for both the emergency operation and the TIRP including 23 road sections, including river training, repavement, bridges rehabilitation, etc. The emergency operation (JA-L1015) is directed to support immediate rehabilitation of basic road infrastructure affected by hurricane Dean; the reconstructive work will be such as to reduce the impact of future hurricane conditions and flooding.

- The eligibility criteria for road rehabilitation selection will consider HDM for the economic analysis; and prioritization based on the surface condition and traffic volume. In addition projects must: (i) be part of the national main road network system; (ii) have a budget allocation provided for the corresponding year; (iii) be economically feasible and sustainable assuring the investments will last for a long term with no further major investments; (iv) be environmentally feasible thereby complying with the country’s environmental legislation and Bank’s Policies and environmentally sustainable assuring that no additional impacts will be imposed to the environment; (v) have final designs prepared  and have incorporated mitigation measures to prevent correct or compensate direct environmental impacts including removal of utilities; (vi) have clearance of the right-of-way before signing the contract for civil works; (vii) ensure that all residents or vendors, if any, are properly relocated accordingly to a resettlement plan that should comply with the Bank Policy OP-710 an d be approved by the Bank prior to the beginning of the works.

### Program Execution

- The borrower will be the Government of Jamaica (GoJ). The executing agency will be the Ministry of Transport and Works (MTW), through the National Works Agency (NWA). The Project Executing Unit (PEU) implemented to execute the NRSIP will be responsible for loan administration, including financial management and accounting, reports, bank accounts, technical and environmental monitoring and supervision, and coordination with other international lenders.

- NWA currently performs in-house Quality Assurance monitoring on a monthly basis, reviewing on-going maintenance works. Quality Assurance monitors the following maintenance programs: Drain Mitigation; Emergency Flood Damage; Force Account; KMR Patching/Milling/Overlay; National Road Services Improvement Project (NRSIP); Northern Coastal Improvement Project; Road Maintenance Fund; Routine Maintenance; Special Bushing & Drain Cleaning; Special Flood Damage; Special Routine Maintenance; Spray Patching. Works under the proposed TIRP shall also be monitored by Quality Assurance.

- A comprehensive monitoring and evaluation system will be designed into the program to continually access and refine the program’s impact. The expected outcomes of the project would be: (i) reduction in vehicle travel times on the roads to be improved; (ii) proportion of the road network in good/fair conditions; and (iii) more efficient and effective road management by NWA. The key outputs of the project would include: (a) kilometer of road improved or rehabilitated, (b) number of structures improved, and (c) the conduct of specific capacity building activities in the NWA.

### Environmental Management

- Environmental Management of the Program will be conducted by the Environmental Unit (EU) at the National Works Agency, which will have the responsibility for environmental integrity of the project. Thus, all contracts will have a special section named Environmental Clauses, with which the contractors must comply. The Environmental Clauses had been prepared under the NRSIP and must be continuously overviewed and updated to specific conditions and to incorporate lessons learned during project implementation. The Environmental Clauses must be indicated in the Contract as integrant part of it enforcing Contractors obligation to comply with.

- The EU will assure that the contractor’s activities are in compliance with the Environmental Clauses and the NEPA requirements. EU will take the necessary steps to ensure that actions are applied in cases where there are violations of the Environmental Clauses and requirements.

- The EU will be composed by one Environmental Engineer (already in house), and two technical assistants that are being hired under NRSIP as part of the institutional strengthening component.

- The Contractor will carry out specific mitigation activities, to comply with all Environmental Clauses of the Contract, utilizing the methodology outlined in this Environmental and Social Management Plan and their Environmental and Social implementation Plan - ESMIP as well as Traffic Control Plan and Project Management Manual. In order to better manage environmental mitigation measures/activities, the Contractor shall have an Environmental Specialist that will coordinate directly with the EU.

- Some projects might require environmental permits or beach licenses, such as sea defenses, river training and major road works. In these cases the EU will assist PEU on preparing the environmental documents required for obtaining such permits or licenses. The NEPA will ensure that the mitigation measures that are outlined in the EMP and ESMIP are implemented in accordance with those, throughout periodical site inspections. NEPA will take the necessary actions with regards to violations of the Environmental Permits. The Contractor shall not initiate any works until the required permits or licenses are obtained.

- The Program will finance annual technical and environmental auditing in order to ensure that adequate preventive, mitigation and compensation measures were implemented and  to recommend some  improvements  on the ESMP and ESIMP.

## IV. LEGAL AND INSTITUTIONAL FRAMEWORK

### Transport Sector

- The Ministry of Transport and Works (MTW) is responsible for establishing and implementing transport policy, regulating land transport services and for the construction, maintenance and operation of all public buildings and transport infrastructure, including roads. The National Works Agency (NWA) was created in 2001, and is the main executing entity within the ministry responsible for the construction, maintenance and operation of all public buildings and transport infrastructure, including roads.

- There are nine entities under the purview of the MTW that are involved in road transportation in Jamaica: the Island Traffic Authority (ITA), Jamaica Ultimate Tyre Company, Jamaica Urban Transit Company (JUTC) Limited, Montego Bay Metro Limited (MBM), National Road Operating and Constructing Company (NROCC), National Works Agency (NWA), Road Maintenance Fund Board (RMFB), Toll Authority and the Transport Authority (TA). The RMFB, NROCC and the NWA are involved in road construction and the development and maintenance of road infrastructure.

- There are other entities that operate within the realm of road transportation that are not portfolio entities of the MTW. These entities include Jamaica Social Investment Fund (JSIF), Urban Development Corporation (UDC), Rural Agricultural Development Authority (RADA), Trade Board Limited, Inland Revenue Department (IRD), Parish Councils and the Kingston and St. Andrew Corporation (KSAC).

- The RMFB was established in 2002 under the Road Maintenance Fund Act, to manage a pool of funds for the specific maintenance of main roads and structures of main roads island wide. The Fund receives one third (1/3) of the motor vehicle licensing fees collected by the Inland Revenue Department. The RMFB finances a number of roadwork projects which are implemented by the NWA.

- The responsibility for maintenance and management of roads and bridges, island wide is shared between the NWA, which has jurisdiction over main roads, and Parish Councils along with the Kingston and St. Andrew Corporation (KSAC), which carry out works related to parochial roads. The NWA designated as an executive agency is therefore responsible for the island’s main road network, and as such, oversees the implementation of all main road works projects.

- The NWA is also responsible for the implementation of a number of roadwork programs such as the Kuwait/OPEC Fund Assisted Road Rehabilitation Project (KFAED), the Caribbean Development Bank (CDB) program, and the National Road Services Improvement Program (NARSIP), funded by IDB; Spray Patching Program for Main Roads; Routine Maintenance Program; Jamaica Social Investment Fund (JSIF) - Roadwork Projects; Urban Development Corporation (UDC) - Roadwork Projects.

- The NWA has also been carrying out work in the area of overloaded heavy-duty vehicles. Given the varied responsibilities of the NWA, the successful implementation of the ongoing programs and activities will lead to greater efficiency in addressing the growing demands on the main road network.

- A Project Executing UNIT (PEU) was created within the NWA, directly under the Department of Project Implementation for the NRSIP implementation. This same team will be responsible for both the emergency operation and the TIRP implementation, taking advantage of lessons learned and institutional capacity gained during this process.

- The National Transport Policy covers all aspects of transport in Jamaica: roads, railways, air, water, infrastructure and services. Within the scope of the National Indicative Program (NIP) funded by the European Commission a draft Road Sector Policy (Land Transport – Roads/Infrastructure) has been developed, to be included in the National Transport Policy; and a Road Maintenance Master Plan, to determine investment priorities in the road sector.

- The Land Transport Road Sector Policy comprehends eleven Strategic Policy Statements, addressing several issues, including environmental and social concerns related to land transport, including an environment related policy: “To minimize any negative environmental and social impacts resulting from the provision and maintenance of roads and roads infrastructure.”

- According to this policy, all new roads and major improvements, tolled or otherwise, will be subjected to an Environmental Impact Assessment (EIA) and a Transportation Impact Assessment (TIA) by the relevant agency, in order that appropriate measures can be identified to mitigate any adverse effects of road construction and use. Environmental standards for new roads and road improvements will be promulgated, including social and re-settlement issues.

- The Road Maintenance Master Plan identifies a framework which will comprehensively identify those roads, in the period 2006-2016, which are in need of urgent periodic maintenance. The Road Master Plan recommends that consideration shall “be given to formulating a National Environmental Road Sector Mitigation Plan which would complete the framework for environmental protection measures in the Road Sector Master Plan.” Road Master Plan is to be approved shortly.

- The Road Traffic Act (amended in 2005) establishes a Road Traffic Control Authority and several regulations related to motor vehicles, public passenger vehicles, commercial motor cars, roads and vehicles generally. Among these regulations, the latest amendment  provided power to the Traffic Authority for weighing vehicles and to require that goods are unloaded if maximum laden weight is exceeded, as well as instituted appropriate fines for these cases. This has a very important effect on road maintenance, as the Public Authority had identified that most vehicles circulate with weight above the maximum allowed. Preparation is also being made to implement the commercial vehicle safety and weight limit enforcement Program.

- The Main Roads Act (amended in 1985) establishes procedures for main roads management, laying out roads, taking of lands, encroachments, offences, and other issues. This act establishes that “the boundary of any main road shall be the fence running along it on either side, and where there is no fence the boundary shall be a line at all points twenty-five feet from the center of the roadway measuring in a direction at right angles to the road.” Accordingly to this act, no person shall erect any building five feet (1,52m) of the boundary of the main road without the consent of the Chief Technical Director. Any building erected without consent shall be deemed to be an encroachment. If there is an encroachment, the Director will issue a notice to the owner or occupier, and this one will have to remove the encroachment at his own expenses.

### Environmental Sector

- In 1991, Jamaica promulgated the Natural Resources Conservation Authority Act by which a Natural Resources Conservation Authority (NRCA) was established to provide for the management, conservation and protection of the natural resources of Jamaica. The NRCA was also charged with administering the Beach Control Authority Act (1956), the Watershed Protection Act (1963) and the Wildlife Protection Act (1945).

- In 2001, the NRCA merged with the Town Planning Department (TPD) and the Land Development and Utilization Commission (LDUC) to form the National Environment and Planning Agency (NEPA), as part of the Ministry of Local Government and Environment. As a regulatory agency, NEPA therefore now has responsibility for the management, conservation and protection of the natural resources of Jamaica.

- Environmental protection in Jamaica is supported by several laws and regulations that control activities in the physical and biological environment. The legal and institutional framework of the environmental assessment process in Jamaica is governed by the National Resources Conservation Authority (NRCA) Act (1991). This Act requires that people or entities must not undertake an activity that pollutes, or might pollute, the environment unless all reasonable and practical measures are undertaken to prevent or minimize any resulting environmental harm. All individuals and organizations have a duty of care to prevent or minimize environmental harm.

- Section 9 of the NRCA Act creates a Ministerial discretion to declare parts of or the entire island a ‘prescribed area’, in which specified activities require a permit, and for which activities an environmental impact assessment may be required. The Natural Resources (Prescribed Areas) (Prohibition of Categories of Enterprise, Construction and Development) Order, 1996 and the Permits & Licensing Regulations (1996, amended 2004) were passed pursuant to section 9.  The Order provides that the entire island of Jamaica is a prescribed area and lists specified categories of enterprise, construction or development that require a permit. The Act also addresses sewage and trade effluent discharges as well as air emissions.

- The Permit and License System (P&L) is a mechanism to ensure that all development within Jamaica meets required standards in order to minimize negative environmental effects. The P&L is framed by provisions of the NRCA Act (1991), the NRCA Order and the NRCA Regulations (1996, amended 2004) which seek to effectively manage development activities and their deleterious effects in Jamaica and its Territorial Sea as it relates to the environment and human health.

- A permit is required to undertake any construction, enterprise or development of prescribed nature anywhere in the island and the Territorial Sea. The permit is intended to safeguard the various environmental/natural resources from direct damage due to physical development. Permits are applicable to new activities. Permit validity is indefinite. Permits might be required for asphalt & concrete belching plant, major road improvements, river basin improvement, land reclamation and drainage projects, watershed development and river training, among others.

- A license is required for discharging of any sewage or trade effluent or other pollution matter to air, ground or water, or the construction, reconstruction or alteration works, in order to safeguard the various environmental media from contamination. Licenses are applicable to both existing and new facilities. Any activity on beaches, foreshore, floor of the sea, and water column require a beach license. The validity of the license is 5 years, and shall be renewed before expiry. Works such as sea walls and other sea defenses will require a beach license.

- The Wildlife Protection Act (1945) is primarily concerned with the protection of specified species of fauna, and establishes fines and the number of animals enjoying protected status. Further amendments are being undertaken to address a variety of other issues relating to the management and conservation of these natural resources, and the inclusion of flora.

- The Beach Control Act (1956) was enacted to ensure the proper management of Jamaica’s coastal and marine resources by a system of licensing of activities on the foreshore and the floor of the sea; amendment in 2004 included jurisdiction over the water column. The Act also addresses issues such as access to the shoreline, rights associated with fishing and public recreation, as well as the establishment of marine protected areas.

- The Watersheds Protection Act (1963) provides for the protection of watersheds and promotes the conservation of water resources. The entire island is considered to be one watershed, but for management purposes, is divided into 26 watershed units. The Act makes provision for conservation of watersheds through the implementation of provisional improvement schemes whereby soil conservation practices are carried out on land. A National Integrated Watershed Management Council was established in 2001 with the mandate to implement a National Integrated Watershed Management Framework including the coordination of watershed activities of public and private sector, NGOs and CBOs.

- The Clean Air Act (1964) and the Natural Resources Conservation Authority (Air Quality) Regulations (2006) establishes rates of emissions and maximum ambient concentrations for priority air pollutants such as carbon monoxide, nitrogen oxides, sulphur dioxide, particulate matter and fine particulate matter. These regulations though do not apply to any air pollutant emitted from a combustion engine in a truck, car, bus, train, ship, airplane or any other mode of transportation.

- The Jamaica National Heritage Trust Act (1985) was enacted to ensure  the protection and preservation of the reminders of Jamaican heritage. The Jamaica National Heritage Trust, which replaced the Jamaica National Trust Commission, is responsible for declariong or designating places and objects of historical value, such as National Monuments or Protected National Heritage. When a site is declared, designated or has a preservation notice placed on it, the JNHT has to give written approval for development, or any alteration of such site.

- The Forest Act (1996) addresses the sustainable management of forests on lands in the possession of the crown and vests management responsibility in the Conservator of Forests.  The Act provides for the establishment of forests reserves, the establishment of protected areas, the promotion of forestry research areas, reforestation initiatives  and the preparation of a forestry management plan. The latter has been prepared and is being implemented.

- The Endangered Species (Conservation and Regulation of Trade) Act, 2000. This Act was promulgated to ensure the codification of Jamaica’s obligations under the Convention for the International Trade in Endangered Species of Wild Fauna and Flora. The Endangered Species Act governs international and domestic trade in endangered species in and from Jamaica. The Act defines the functions of a Management Authority and Scientific Authority. The Natural Resources Conservation Authority is the Management Authority. The functions of the Management Authority include the grant of permits and certificates for the purpose of international trade, the determination of national quotas and the monitoring the trade in endangered species.  A Scientific Authority is appointed under the Act to determine whether a species is at risk, vulnerable or threatened, to advise on trade matters and to monitor the grant of permits and certificates in respect of limitations to maintain species and to ensure sustained survival.

- The National Solid Waste Management Act, 2002. This recent enactment creates the National Solid Waste Management Authority and mandates inter alia that the Authority take such steps as are necessary for the effective management of solid waste in Jamaica in order to safeguard public health as well as the collection, transportation, re-use and re-cycling of waste in an environmentally sound manner. The Act establishes a licensing regime for operators of solid waste management facilities, and the operators of collection and transfer services.

## V. ENVIRONMENTAL AND SOCIAL CONDITIONS

  - Jamaica, an island nation situated in Caribbean Sea in Central America, covers a land of 10,990.5 square kilometers, 234 km in length and 80 km in width. Adjacent territorial seas (12 miles radius) and Exclusive Economic Zone (EEZ) were declared in 1996 and 1991 respectively. The island accommodates a population of 2,680,100 inhabitants (52% in urban areas) with an annual population grown rate of 0.47% (Statistical Institute of Jamaica, 2005), an employment base of 171,426 jobs and is visited by nearly 2.5 million tourists annually.

  - Jamaica is the third largest island in the Caribbean, and the most populous English-speaking Island in that region. The island of Jamaica is home to the Blue Mountains inland, and is surrounded by a narrow coastal plain. Most major towns and cities are located on the coast. Chief towns and cities include the capital Kingston, Portmore, Spanish Town, Mandeville, Ocho Rios, Port Antonio, and Montego Bay.

  - Jamaica has experienced negative to moderate economic growth in the past years, mainly due to the effects of the financial crisis of 1995/96, which created a serious fiscal imbalance that constrained expenditures. The Government’s goal is to reduce the public sector debt burden that currently exceeds 120 percent of GDP. The high level of debt has an impact on government cash flows as interest payments consume 40% of all central government revenue. In this context, the availability of funds for investment in transportation and infrastructure has been severely limited in the past years.

  - The Jamaican economy is heavily dependent on services, which now account for more than 60% of GDP. The country continues to derive most of its foreign exchange from tourism, remittances, and bauxite/alumina. Remittances account for nearly 20% of GDP and are equivalent to tourism revenues. Jamaica's economy, already saddled with a record of sluggish growth, will suffer an economic setback from damages caused by hurricane Dean in August 2007.

  - The economy faces serious long-term problems: high but declining interest rates, increased foreign competition, exchange rate instability, a sizable merchandise trade deficit, large-scale unemployment and underemployment. Jamaica's onerous debt burden - the fourth highest per capita - is the result of government bailouts to ailing sectors of the economy, most notably the financial sector in the mid-to-late 1990s. Inflation also has declined, standing at about 7% at the end of 2007. High unemployment exacerbates the serious crime problem, including gang violence that is fueled by the drug trade.

  - The climate in Jamaica is tropical, with hot and humid weather, although higher inland regions have a more temperate climate. Some regions on the south coast, such as the Liguanea Plain and the Pedro Plains are relatively dry rain-shadow areas. Jamaica lies in the hurricane belt of the Atlantic Ocean; as a result, the island sometimes experiences significant storm damage. Hurricanes Charlie and Gilbert hit Jamaica directly in 1951 and 1988, respectively, causing major damage, destruction, and many deaths. In the 2000s, hurricanes Ivan and Dean also brought severe weather to the island.

  - Jamaica’s environment is under threat from various sources: squatting, pressures exerted by development; improper waste disposal including solid waste, the treatment of sewage and hazardous waste; air pollution; poor land use including degrading forests, strained coastal areas and improper mineral exploitation; the overexploitation of resources; and threats to biodiversity.

  - The main productive sectors of tourism, agriculture, manufacturing, mining and quarrying are heavily dependent on the island’s natural resources - beaches, sea, scenic beauty, land, mountains, freshwater and air.

  - There is a strong correlation between the state of the environment and the island’s vulnerability to natural hazards. Poor environmental practices such as improper disposal of solid waste, indiscriminate removal of forest cover, poor land use practices and squatting, coastal waters polluted by industrial waste, sewage, and oil spills, damage to coral reefs, air pollution in Kingston results from vehicle emissions, tend to exacerbate the effects of natural disasters. This has become a concern given the increased frequency and intensity of tropical storms associated with climate change.

  - Forest Cover. In 2004, the National Forestry Inventory Report indicated that forests cover 31% of Jamaica’s surface. There have been slight variations in the percentage of forest cover during the last decade: while natural forest cover has decreased over the period, forest plantation cover has increased. Net degradation of forest cover has been attributed to an increase in mixed land use and agriculture.

  - Water Quality. NEPA routinely monitors 27 river sites and 28 beach sites for water quality. The parameters typically measured are nitrates, phosphates, biochemical oxygen demand and faecal coliform. As expected, the rivers that drain major settlement areas show on average significantly higher levels of faecal coliform.

  - Air Quality. NEPA has begun monitoring for air quality but confined to the corporate area. Levels of total suspended particulate (TSP) for sites at Cross Roads and Harbour View in Kingston exceeded the annual standard and indicate the potential for impairment of human health. PM10 concentrations are not at levels that are detrimental to human health. Gases (SO2, NO2 and CO) are below ambient standards and are not causes for concern at this time.

  - A vulnerability index for the natural environment (EVI) was developed by the South Pacific Applied Geosciences Commission (SOPAC), and the United Nations Environment Program (UNEP) and presented in 2005. The index was developed through consultation and collaboration with countries, institutions and experts across the globe and is based on 50 indicators. The index is designed for use with economic and social vulnerability indices to provide insights into the processes that can negatively influence the sustainable development of countries.

  - Jamaica participated in this initiative and based on the application of the index, received a score of 381. This means that Jamaica falls in the category of countries that are characterized as being extremely vulnerable.

  - Kingston, which is the capital city, and the surrounding parish of St Andrew, whilst totaling 452.5 km2 (4.1% of total land mass), is highly urbanized. This population density has major implications for the transportation system within the Kingston and St. Andrew areas and surrounding environs, which constitute the Kingston Metropolitan Transport Region (KMTR).

  - Archaeology and Historical Heritage. Sites of significance are identified in the Jamaica National Heritage Trust Survey, in compliance with the requirements of the Jamaica National Heritage Trust (JNHT) Act to protect any chance finds of cultural property. In the present TIRP there is no likelihood that any archeological finds will be made since it does not involve any new excavations for construction but only reconstruction at the same site. The only cultural heritage found was the Flat Bridge, at Bog Walk, which might be included in the emergency operation or on this TIRP.

  - Flat Bridge - St. Catherine. This bridge was built somewhere after 1724. This bridge is flat and composed of planks on frame of timberwork which rests upon two piers and two buttresses projecting from the banks, constructed with piles and braces interlaced with masonry. When this bridge was being constructed, the sixteen plantations in the Bog Walk area were obliged to send one slave in every fifty to work on the River Road, sometimes called Sixteen Mile Walk. Gravel, marl, lime, sand and stone had to be dug. Slaves often lost their lives as they performed dangerous tasks in the Gorge. Contracts for timber and for masons to work on the bridge were authorized at vestry meetings. Between 1881 and 1915, the floor of the bridge was washed away and later re-floored with iron girders and buckle plates taken from the original flooring of the May Pen Bridge. Today, the bridge of three spans is supported by two piers and two abutments. In the 1930s it had metal handrails and later wooden ones, but these were devoured by the river at different times. Semi-circular spheres are now the only protection on the bridge itself. The Flat Bridge is one of Jamaica’s oldest bridges.

### Transport and Road Network

  - The transportation infrastructure in Jamaica consists of roadway, railway, maritime and air transportation. There are 34 airports in Jamaica, being international two of them. Jamaica has 370 km of railways, but only 57 km remain in operation, currently used to transport bauxite. The most important ports and terminals are in Kingston, Port Esquivel, Port Kaiser, Port Rhoades, and Rocky Point.

  - The bulk of the commuting public in Jamaica travel via land transportation which encompasses two areas: road and railway transportation. Road transportation encompasses the movements of pedestrians, automobiles and cycles. The road network is the most important mode of transport and mobility in Jamaica that facilitates access to social and economic life for the island. Transport infrastructure has an influence in many aspects of economic activity such as agriculture, manufacturing, tourism and international trade.

  - Land transportation has been significantly affected by natural disasters which cause extensive damage to infrastructure and in effect result in a significant reduction in activities of Jamaica’s transportation system. As there is interdependence among the transportation sub-sectors of air, land and maritime transportation, any activity that negatively impacts on the land transportation sector, is immediately felt in the other transportation sub-sectors, and vice versa.

  - Jamaica has one of the densest road networks in the world, which demands for resources for road maintenance in excess of the country’s ability to adequately finance it. Accordingly to the World Bank Development Reports, Jamaica has the highest normalized road index of all the countries (752). The normalized road index is the length of total roads in a country compared with the expected length of roads, which is a function of population density, income and region specific characteristics. A value of 100 is considered “normal”.

  - Jamaica’s road network consists of 20,996 km of road, being 15.386 km paved (including 33 km of the expressway Highway 2000) and 5,610 km non-paved. The main road network in Jamaica is administered by the National Works Agency (NWA). There are some 12,000 km of parochial roads and 5,000 km of estate and farm roads distributed throughout 13 parishes and the urban conurbations – Kingston and Portmore. Only 27% of the parochial road network is considered to be in a maintainable condition.

  - Traffic volume has been steadily increasing over the last years. This has led to congestion problems in major towns and capitals across the island. Traffic management initiatives have been implemented in the Kingston Metropolitan Transport Region (KMTR) and proposals have been made to address congestion issues in other parishes.

  - In 1999, the GOJ conducted road inventory and condition surveys, mostly for the main or national road network. Roads were classified into three types according to their function and importance: Class A arterial roads are those of national importance which carry daily traffic volumes in excess of 1,000 vehicles and link one or more major areas of the island with other major national regions; Class B secondary roads are roads of national importance which connect with the arterial roads and normally carry average daily traffic volumes of 500-1,000 vehicles; Class C tertiary roads are normally of local importance and have the function of allowing communications and contact within and between districts, and normally serve traffic volumes of less than 500 vehicles.

  - Results of this road condition survey indicate that Class A and B roads are in ‘quite good to fair’ condition while the paved C roads are in ‘poor to bad’ surface condition. The majority of road failures are due to poor drainage of the pavement. The practice of overloading heavy-duty vehicles is also a major factor that has lead to excessive damage to road surfaces in Jamaica’s main road network. Spot checks done by the NWA, at various points across the island, found that the majority of heavy vehicles were carrying nearly twice their designated axle-bearing load.

  - Jamaica’s road network poor or bad conditions have been exacerbated by hurricanes, attendant floods, and significant storm damage. In 2002 there was heavy torrential rains and flooding, followed by floods in the wake of hurricane Ivan in 2004 (the sixth most intense Atlantic hurricane on record) and hurricane Dean (the seventh most intense Atlantic hurricane ever recorded) most recently in 2007.

  - The lack of timely maintenance, the impact of heavy rains and hurricanes, and the impact of overloaded commercial vehicles have resulted in the extensive degradation of Jamaica's road network. Routine maintenance includes cleaning of drains, cutting back trees and vegetation, pothole and crack repairs. Periodic maintenance includes some form of overlay as well as repairs of drainage and kerbs and help to prolong the life of a road.  If both forms of maintenance are not carried out when required the road will reach a state where the only option is to reconstruct the road to improve the riding surface raising the whole life cost. The Road Management Master Plan considers the periodic maintenance of only 6,000 km of maintainable roads.

## VI. ENVIRONMENTAL AND SOCIAL IMPACTS

- The net environmental and social impacts are likely to be positive for residents and businesses. No major negative impacts are expected since works will include mainly rehabilitation of primary and parochial roads, including road pavement and bridge restoration, river training, embankment stabilization and sea defenses. Minor negative impacts are likely to be restricted to the immediate area of influence of the road, and limited to construction phase. Although some encroachments in the right-of-way may occur, no relevant resettlement action is expected.

- By improving physical, operational and safety conditions of roads, the Program will bring positive environmental and social impacts. The Program will benefit the community living in the project area as well as in the whole region. Road users will experience reduced vehicle-operating costs and reduction in travel time and accidents. This should result in greater competition among transport firms, lower prices, increase the capacity of local contractors and ultimately encourage new investment in such areas as tourism and export agriculture. Economic expansion and diversification would provide sources of employment and increase revenues.

- Potential positive impacts are: (i) increased safety and comfort for road users due to improvement of road surface; (ii) reduction in travel time and accidents; (iii) improved drainage and storm-water management; (iv) reduction in road deterioration and erosion and sedimentation processes due to flooding; (v) employment generation during construction.

- Given the characteristics of the proposed works, as well as the fact they will take place mostly within the existing road right-of-way, and will not affect resident population, the potential negative impacts are expected to be: (i) generation of dust, noise, vibrations and air emissions by construction activities; (ii) soil erosion with possible sedimentation of nearby water bodies; (iii) traffic congestion, creation of hazardous driving conditions and temporary obstruction of access to community services; (iv) accidental rupture of utilities pipes and lines (electricity, telephone, and water), and temporary interruption of services during the excavation and removal of pavement; (v) risk of accidents for workers or road users (pedestrian or vehicles) around construction sites; (vi) soil and water contamination due to wastes and effluents (garbage, used waters, oil, grease, fuel, paint, etc.) coming from work areas or equipment yards.

### Potential Impacts during Construction Phase

- Air Emissions, Dust, Noise and Vibrations. There will be construction related traffic along the roads. Vehicle air emissions will be an impact during peak period. Dust will be an issue mostly on the construction site. Mobile units to be placed at the construction fronts will be critical. This impact will occur at campsites, construction yards, right-of-way and surroundings, in different degrees of importance. The main noise and atmospheric emissions sources will be the equipment used during the works, which will reach both the workers in activity, and the resident population in the proximities. Noise emissions represent a temporary impact about the individuals of the local fauna that will move off this discomfort during the construction activities.

- Erosion and Sedimentation of Water bodies. Works might result in the removal of the existing surface soil. If not controlled, sediment may be washed from the exposed cut and into nearby wetlands and water bodies resulting in loss of aquatic flora and fauna. Soil movement by wind, water and site activities provides the greatest threat to water quality downstream from or in adjoining marine areas to a construction site. Soil and pavement materials will erode if they are not protected. The best form of protection is to prevent water flowing over the site and protected drainage lines.This adverse effect should be mitigated through erosion control protection associated with construction, and rapid revegetation of the exposed flat areas.

- Traffic Congestion and Accidents. All proposed works will have a direct impact on traffic, as they will be developed within roads boundaries. Traffic congestion might bring increased risks to people and property, especially nearby urbanized areas or along main roads. By-passes and traffic diversions will increase traffic slowdown and the risk of accidents involving both vehicles and pedestrians. Existing alternatives roads might also be used, increasing the number of vehicles in a calm area. Special arrangements will need to be made in order to minimize conflicts resulting from shared use of some segments of the road. Congestion will also be due to the limited space for materials and equipment during construction: rubble from the demolished structures and pavements will be deposited, at least temporarily, at the construction site on shoulders of the road, as well as timbers, boards, sand and gravel must be stockpiled at locations convenient to the work site. Also to be accommodated is the heavy equipment, including dredges, pile drivers, trucks, and others.

- Accidental Rupture of Utility Pipes and Lines. Although designs must show existing utilities, the actual location of these utilities might not be exactly as shown. Works can accidentally cause rupture of utility pipes and lines, resulting in temporary interruption of services with severe consequences on life quality of the residents of the affected neighborhood.

- On-the-Job Accidents and Health Issues. Construction works along roadways can impose some risk of on-the-job accidents, due to excessive velocity of vehicles and equipment, poor signalization, or spillage of materials transported. The increase of vehicles and equipment traffic on the roads can also increase the risk of accidents with road users, posing additional risks to workers.

- Soil Contamination. Removal of old structures and pavements will produce waste materials: broken concrete from existing abutments and wing walls, timbers and boards, sediments dredged from canal bottoms and banks. Debris could cause nuisances and hazards if handled carelessly. Significant volumes of fuels and lubricants will be used by vehicles and equipment during works, and waste petroleum products, in the form of used engine oil, and solvents will be generated. Several other potentially hazardous chemicals, reagents and industrial compounds may be used during construction, which if accidentally spilled or improperly handled can impose adverse environmental impacts on soil and water. Construction support sites can generate erosive processes and, consequently, siltation of the drainage system. The abandonment of campsites and construction yards after works completion can deteriorate local landscape, as well as the inadequate discharge of soil residues and waste materials.

- Water Pollution. Water quality is not likely to be substantially lessened during construction, except when works take place along water bodies, such as river training. These activities will temporarily add to turbidity. Engine oil and transmission or hydraulic fluids will be changed, possibly entering waterways. Trash, such as cement bags, pieces of concrete from old abutments or discarded positions of concrete forms, may be discarded in rivers. Campsite effluents may be transported for disposal at existing treatment plants. During paving procedures and other activities involving intensive use of concrete, uncontrolled runoff may occur resulting in alteration of water acidity levels (pH).

- Vegetation and Fauna. When construction sites are located in areas less occupied, impacts on surrounding vegetation and fauna might occur, due to improper vegetation removal, waste and materials dumped into vegetated areas, disturbance to fauna, etc.

- Removal of Encroachments. All works will be performed within the right-of-way of the roads, not affecting private properties. However, it is possible that along work sites some encroachments might occur, either residents or vendors installed very close to the road limits. If such buildings are found, NWA must properly relocate them prior to works commencement according to Bank Policy OP-710. Where people or structures must be relocated, a consultation and negotiation process that leads to a contractual agreement and relocation or cash compensation must be undertaken, in compliance with the OP-710.

Potential Impacts during Operation Phase

- The most significant impacts of road rehabilitation occur during the construction phase. Once the roads are operational, less significant impacts are observed, such as blocking drainage systems with small amounts of material, waterways pollution by petroleum products and other chemicals spillages, erosion and watercourses siltation.

- Water Pollution. Water pollution is associated to rainwater that washes off the roads, picking up dirt and dust, rubber and metal deposits from tire wear, engine oil that has dripped onto the pavement, pesticides and fertilizers, and discarded cups, plastic bags, cigarette butts, pet waste, and other litter. These contaminants are carried into rivers and ocean. Potential contamination of watercourses is also verified along roadside occupation, where sanitary effluents and solid wastes are disposed improperly, as well as from accidents with hazardous materials transportation.

- Erosion around Structures and Roads. The stability of structures – such as culverts, bridges and river banks – depends on proper maintenance of vegetation of the areas around structures and road sections, as well as adequate flow through drainage systems.  Thus, natural and man induced erosion at and around structures can become a major problem. Also inappropriate waste disposal along roads and drainage can pose additional risks to structures and road safety as it can induce new flooding events.

## VII. MITIGATING MEASURES AND ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

### Mitigation Measures

- The mitigation measures herein described include the Best Management Practices and Measures identified in the Environmental Procedures (April 2006) prepared by NWA.

- Construction Camps. Construction Camps are subject to the provision of a license by MTW and NEPA. The following mitigation measures shall be used to control pollution, erosion and other direct impacts at construction camps and storage facilities: (i) sewage and waste water discharges shall be collected and treated in an approved system installed on site; (ii) solid wastes shall be stored in proper containers, collected and disposed of using public or private companies entrusted with solid waste collection in the communities / parishes where such wastes are collected (camp sites) and disposed in landfills/dumpsites approved by NEPA; (iii) erosion shall be minimized and sedimentation prevented through a drainage system with sedimentation traps and/or screens; (iv) hydrocarbon storage facilities shall be provided with collection traps and spillage berms; (v) refueling and servicing areas shall be situated on impermeable surfaces served by an oil trap and run-off collection facilities; (vi) trucks and earth moving equipment shall be equipped with functioning back-up warning devices. The Contractor shall provide clean potable water, food and housing when necessary. The burning of materials where unavoidable in open air shall be done under strict supervision of the contractor’s qualified staff and in accordance with the countries fires act. The Contractor shall not burn tires, asphaltic materials, oil or any materials that will produce dense smoke, either for the purpose of incineration or to augment the burning power of other matter. Extreme caution shall be taken to dispose of materials so as to avoid damage or destruction to private property or to cause excessive air pollution.

- Storage Facilities. The Contractor shall take every possible precaution against contamination of work site or surrounding areas. All temporary and permanent fuel and oil storage areas shall be above ground and isolated within an impermeable bunded area. The volume of the bund shall not be less than the total stored volume capacity. Drainage from such bunded areas shall incorporate oil separators before discharge to existing watercourses. All workshop and equipment storage areas shall be drained such that all surface water runoff passes through an oil separator prior to discharge into watercourses. A spill contingency plan shall be prepared before the commencement of site activities and the necessary clean-up agents and equipment in place. All site workers shall be made aware of the emergency planning measures and selected staff should be trained in the handling, storing and disposal of hazardous and polluting substances. Stockpiles of fine materials (e.g. topsoil, cement, sand) shall be covered to prevent the wind erosion of the stockpiles. Such coverings shall be used during periods of inactivity, when access to the stockpile is not required. This includes covering at the end of each working day. The Contractor shall propose, and adopt, methods for reducing dust arising from the operation of any batching plant or crusher plant.

- Asphalt, Concrete and Crushing Plant. Asphalt, Concrete and Crushing plants are required to obtain a license by NEPA. Where applicable, the Contractor shall install the asphalt, concrete and crushing plants in flat land, cleared of organic topsoil and with suitable access for vehicles. The locations shall not be near urban or other settlements. The installations shall provide adequate storage space for raw and surplus materials, and ample space for traffic circulation to prevent hindrance to loading and unloading operations. The production plants shall be adjusted to operate in such a manner that, to the extent possible, dust and fumes discharge are kept to a minimum. The plant shall be kept free of human and construction debris and storm-water run-off shall not carry contaminants to adjacent watercourses, wetlands or sensitive vegetation. Special measures should be taken to ensure that storm water run-off is adequately treated to prevent the export of contaminants. Such measures may include stilling ponds, silt traps and oil interceptors. In transportation from the plants, trucks shall be fitted with adequate devices to prevent material spillage.

- Borrow Areas and Quarries. The use of any borrow areas is subject to the provision of a license by the Department of Mines and Geology and NEPA.Cover gently sloping or flat borrow areas with top soil after termination of the use of the site; contour shape embankments to slow the rate of water run-off; provide for good borrow site management practices in contracts with private site operators. The Contractor shall not extract river stone, gravel, sand or any other construction material from watercourses without approval.

- Site Restoration. Upon completion of the Contract, all elements of the camp shall be removed and the site, as far as possible, be returned to its original condition unless designated for alternative uses with approval of the MTW and NRCA.

- Erosion Control and Storm-water Management. The following actions must be undertaken to minimize soil loss: (i) phase the work program to minimize land disturbance in the planning and design phase; (ii) keep areas of land cleared to a minimum, and the period of time areas remained cleared to a minimum; (iii) provide, install, maintain, remove and dispose of erosion control measures such as gravel filter berms, dykes, catch basin inlet protection, end of pipe filtering devices, silt fences, dams, sediment basins, netting, bale barriers, slope drains and other erosion control devices or methods; (iv) minimize earthwork where possible at bridges, work from banks rather than from stream beds and reduce contact with watercourses to the minimum; (v) sow the faces of bridge approaches exposed to inclement weather with local grass or with other local species, especially those that are adapted to local conditions and which grow rapidly and have deep roots; (vi) restore ground cover as soon as earthworks permit, and take the necessary measures to promote regeneration of native vegetation; (vii) replant shade trees and screening vegetation; (viii) to avoid erosion and gullying of banks, refrain from earthworks during the peak of the rainy season, use gabions and miter drains, and avoid angle termination at the borders of cuts and fills; (ix) build temporary and permanent silt traps where there are perennial streams and the likelihood of other usage downstream, using appropriate materials and techniques, including settling ponds, silt fences, filter cloths and detention ponds, even if such measures may not be immediately clear from the final designs.

- Water pollution control. Precautionary measures shall be taken to prevent the contamination of water courses, aquifers and the marine environment by pollutant wastes such as chemical products, fuel, lubricants, sewerage water, silt or other obnoxious material. Waste water to be discharged from construction facilities must first be filtered or otherwise treated to eliminate obnoxious material likely to cause pollution into natural watercourses or endanger flora and fauna. No waste water or other liquid or obnoxious material will be dumped on wetlands.

- Waterways, river banks and floodplains protection. In the demolition of existing structures and the construction of new ones, the contractor shall ensure that no rubble or other surplus construction material is dumped into existing watercourses or aquifer recharge areas. Construction activities on watercourses or over aquifer recharge areas will be confined to the areas shown in the final design drawings and where the erection of temporary or permanent structures has been so designated. Temporary fills and scaffolding or false works erected for the undertaking of construction works will be removed from river or watercourse beds immediately upon termination of the construction activities. Crossing by construction equipment over watercourse beds during construction activities will not be permitted. The use of silt, sand or similar fine graded material which could cause turbidity or sedimentation in the stream shall not be permitted in the construction of stream crossings.

- Dust Control. All appropriate measures to minimize the generation of dust as a result of works, operations and activities must be undertaken. Such measures shall include regular and effective treatment of gravel roads and working areas, use of dust extractors on drilling equipment or wet drilling equipment or wet drilling, water sprays on aggregate crushing and screening plants, etc. Dump trucks to be used in the transportation of materials are to be fitted with tarpaulins to prevent dust or other material spilling during operation.

- Noise and Vibrations. Minimize noises caused by operations. Provide working machinery and equipment designed to operate with the least possible noise, including the use of shields, soundproof housings, enclosures, or other physical barriers to restrict the transmission of noise, and when gearing is used, such gearing shall be a type designed to reduce noise to a minimum. Compressors shall be equipped with silencers on intake lines. Gas or oil operated equipment shall be equipped with silencers or mufflers on intake and exhaust lines. Allowable sound levels for mobile construction equipment shall not exceed 85 dBA. For stationary construction equipment, the level allowed shall not exceed 70 dBA. Operation shall be restricted to the hours of 7 a.m. to 10 p.m., Monday through Saturday. Use the correct equipment for each job in order to minimize noise intrusion. In populated or urban areas all noisy operations such as piling, transportation of material, earthworks and bulldozing will be carried out during daylight hours only. Blasting will only be carried out once a day, at a time set in consultation with the residents of the nearest by community.

- Blasting. Blasting will be conducted in a safe manner with charges large enough to break up material, but not so large as to damage adjacent structures and threaten human safety from shock and debris. Blasting will take place only during daylight hours and at a time acceptable to affected adjacent parties.

- Waste Management. Waste material shall be disposed in approved areas. Surplus or waste materials shall not be deposited in watercourses or open areas, or on private land. The dump trucks to be used in the transportation of surplus material should be fitted with tarpaulins to prevent dust or other material spilling during the haul. Used pavement material shall not be disposed on the road side, in watercourses or wetlands. Asphaltic cement removed from the site should be re-used or disposed of in a controlled manner.

- Vegetation Protection. Reduce loss of vegetative cover, erosion and sedimentation to the minimum possible by: (i) avoiding any off-the road driving outside the immediate construction areas; (ii) keeping process areas as small as possible; (iii) reducing damage to vegetation through environmentally sound construction practices; (iv) giving special importance to the preservation of trees, particularly large trees of genetic and landscape value; (v) where roadside banks are cut and exposed, protecting the banks from gully erosion and sedimentation into nearby streams; (vi) providing sediment traps and screens where needed; (vii) excavating bulk fill from areas so as to minimize erosion, reduce visual impact of scarring and allow for rapid revegetation following excavation; and  (viii) reusing organic topsoil.

- Fauna Protection. Protect fauna living within the site and ensure that hunting, trapping, shooting, bird-nesting or egg-collecting does not occur. Certain species of birds are known to be rare and endangered. The EU will identify such species to the Contractor, who shall take steps to inform his employees that such species of birds are to be protected. Only licensed fishermen will be allowed to fish and then only in approved locations.

- Archaeological and Cultural Resources: The Contractor shall allow archaeological and cultural inspections during construction to facilitate the protection of cultural and historical sites. Jamaica National Heritage Trust (JNHT) archaeologists will periodically monitor work and specific areas. The Contractor shall assume that the unearthing of archaeological features will result in a work suspension in that area until features are recovered and the site is released by the JNHT.

- Community Relations. The Contractor shall follow a work schedule, procedures and rules of conduct conducive to good community relations during construction. This will entail adequate notification of work schedules for local communities and villages, and hiring practices which provide opportunities for local labor. Communities must be given adequate notice of intended construction and potential for dust and blockage of access to roads or community facilities during construction.

- Health and Safety. The Contractor will provide medical examinations and emergency medical care for the construction crew and will provide suitable and clean sanitary facilities and necessary safety equipment, including special masks covering nose and mouth, for employees when applicable. Safety measures must be taken to prevent accidents involving workers and members of the community, such as speed control, work areas signaling, safety devices (barriers). Other health and safety procedures are: (i) vehicle loads must comply with their capacity and shall not exceed it under any circumstances; (ii) special care should be given in the transport of materials, such as coverage of the buckets; (iii) all material tumbled about the way or about the highway shoulders should be removed, as soon as possible; (iv) work areas and facilities should be properly drained, avoiding the formation of puddles; (v) effluents and waste management shall be properly conducted; (vi) ensure that workers at all times wear protective gear such as ear plugs, dust masks, boots, etc.; (vii) additional measures to ensure the continued safety of pedestrians, drivers and workers, shall be provided, as necessary; (viii) barricades or temporary fencing shall be provided in order to guarantee public safety and the security of the works; (ix) drivers assisting the construction process that demonstrate a lack of safety while driving shall be subject to warnings; (x) drivers assisting the construction process shall be trained to perform spill reporting and clean-up procedures for minor spills.

### Environmental and Social Management Plan

- A project environmental and social management plan (ESMP) has been prepared during the Program preparation Sage, based on NWA’s Environmental Procedures (April 2006) and Bank’s policies. The ESMP contains: NWA’s environmental policy and strategies; best practices and measures; roles and responsibilities; and environmental specifications for civil works. The ESMP will be part of the civil works  bidding documents.

- Monitoring of direct impacts will be carried out by the Supervisor Environmental Manager and NWA’s Environmental Engineer (EU/NWA) and shall include, but not be restricted to, the following concerns: assessment of dust control measures implemented by the contractor; measurement of noise levels arising from construction equipment and construction methods; measurement of air quality in general and pollution levels from construction equipment (exhaust in particular); assessment of surface water treatment implemented by the contractor; verification that proper waste disposal at construction sites and road camps is carried out; verification that the construction sites and camps are cleaned after construction; monitor silting and increased sediment loads to streams which the bridges cross; installation of erosion control and drainage works (including silt traps); prevention of damage to undiscovered significant archeological or historical findings; inspection of vegetation cover (removal and re-growth) on the basis of yearly remote sensing and field examination; ensuring that contractor(s) follows a work schedule, procedures and rules of conduct conducive to good community relations during construction.

- The ESMP must be updated to meet new risks or where inspections, monitoring or audit reveal that measures are ineffective and to achieve ongoing improvement.

### Traffic Management Plan

- The Contractor shall develop a Traffic Management Plan in order to minimize any reduction of road network capacity during construction, and assure safe access to the site. The traffic management master plan shall include at minimum the following: (i) works sequencing and impact minimization on the road network capacity; (ii) lane closures and detours on each section of works; (iii) roads corridors traffic management schedule; (iv) traffic management constraints; (v) specific and general approvals from the relevant statutory bodies; (vi) liaising with affected residents, local businesses, public utilities, emergency services and the media on affected roads.

- Attention shall be paid by the Contractor in maintaining the riding comfort of the roads/detours/diversions. The Contractor shall provide for and maintain pedestrian, public and vehicular traffic during the construction period where required in the project sections by any of the following methods: (i) construction and maintenance of diversion at the existing pavement level when road construction is on the half width of road; (ii) maintenance of existing road not yet taken up for construction; (iii) maintenance of completed half width of road opened to traffic; (iv) maintenance of traffic on exposed granular layers subject to traffic in carriageway under construction; (iv) diversion of traffic onto secondary roads.

- The pavement surface of the diversions shall be as the diverted road; the foundation shall be adequate for the traffic and period required. The Contractor shall implement adequate safety measures to reduce accident risks and shall install signboards and other safety devices. The Contractor’s responsibility shall include the rectification of any damage caused to the road, and to all accesses including any temporary or alternative access due to passage of traffic, rains or flood. The specific requirements of the diversions shall also depend upon the Contractor's method of working and the Contractor shall be deemed to have covered the same in quoting his price.

- The Contractor shall so conduct his operations as to offer the least possible obstruction and inconvenience to the public and he shall have under construction no greater length or amount of work than he can properly manage at a time with due regard to the rights of the public in maintaining a free flow of traffic.

### Environmental and Social Management Implementation Plan -- ESMIP

- The Contractor is to prepare the specific ESMIP using the Project EMP as guidelines. The broad purpose of the ESMIP is to ensure that the appropriate and specific mitigation measures are adopted during construction.

- The ESMIP shall describe Contractor’s procedures regarding environmental protection and conservation measures to be implemented. Measures will include but not limited to erosion and storm water control, dust control, noise pollution, surface water drainage and treatment from site facilities (workshops, asphaltic plant, concrete batching plant, crushing plant, fuel, oil bitumen storage areas), storage of material (fuel and oil, fine materials, crushing & batching plants), disposal of waste materials, protection of vegetation and fauna, occupational health and safety, community relations, liaison with Jamaica National Heritage Trust.

- The Contractor shall act proactively to manage potential environmental impacts and develop appropriate mitigation measures from an early stage in the project. The ESIMP should be approved by  the NWA/EU; supervisory firm and NEPA prior to the beginning of the civil works.

- The Contractor shall be responsible for ensuring that relevant legislation and regulations have been taken into account and all environmental permits and licenses required are in place. The Contractor will be responsible for camp sites, quarry/crushing site specific permits, beach licenses, as well as guaranty that all materials required for the services come from properly permitted facilities. The Contractor shall provide appropriately qualified personnel for the implementation of the ESMIP.

- The ESMIP shall state how different environmental management responsibilities are to be assigned to different people involved with the project. The ESMIP shall include an organizational chart clearly showing the Contractor’s and Supervisor Environmental Engineer position within the structure of personnel on the Project, including lines of communication and responsibility.

- All personnel on the project whose work may, directly or indirectly, cause a significant impact on the environment must be made aware of this and be given sufficient know-how to manage their activities accordingly. The Contractor shall ensure the ESMIP identifies how environmental awareness is to be achieved, how environmental competence training needs are identified and how training will be undertaken.

- The supervision firms will provide environmental engineer to oversee contractors’ compliance with the ESMIP, provide training to NWA/EU and Contractors’ staff and prepare monthly reports and any other reports prepared in relation to environmental, including emergencies that may occur (e.g. spills of hazardous substances). These environmental reports should be on a monthly basis presented to NWA/EU, NEPA and the Bank. NEPA will provide independent supervision over environment and social aspects of the program implementation by periodically reviewing construction activities, according to the ESMP and ESMIP specifications and requirements, including: site management (sanitary conveniences, tidiness of the site, equipment maintenance, fuelling and storage, site compound and stock piles, borrow pits and haul roads, fire control measures); erosion and storm water control (soil erosion and silt runoff), water pollution control (surface and groundwater); waste management and disposal; dust and air pollution; noise and vibrations; landscaping and vegetation protection; cultural and archaeological heritage.

- Training. The Supervisory Engineer will provide training to the following groups: (i) NWA/EU Environmental Engineer and senior staff: training on basic environmental management procedures; and (ii) Construction Contractors’ personnel will be trained on: environmental management, environmental emergency response, materials handling, waste material management and application of environmental and social management specifications. The above training will be included in the supervision firms’ budgets.

- The NWA’s Environmental Unit shall perform independent external audits, every year, to establish the effectiveness of environmental management. The audits shall include: assessing the degree of compliance with the ESMIP achieved; reviewing the continuing relevance of the plan in the light of experience gained and the instigation of changes where appropriate; reviewing the organizational and administrative frameworks described in the ESMIP; reviewing environmental monitoring data and its reporting; the identification of environmental aspects and impacts and the mitigation measures proposed to mitigate these; and identification of any deficiencies in environmental performance and information on measures.

## VIII. MONITORING AND SUPERVISION

- Monitoring the environmental and social impacts of the TIRP will occur on four levels: (i) an Environmental Specialist will be provided by the supervisor firm to the approval, monitoring and supervision of the ESMIP implementation and control and monitor the construction activities and their impacts; (ii) the EU/NWA will review the construction work and determine if the ESMIP is being adhered to and no new impacts occur; (iii) NEPA will provide independent audit over environment and social aspects of the road improvement program by periodically reviewing construction; and (iv) independent technical and environmental audit financed by the Program.

- The EU/NWA shall submit to the Bank quarterly monitoring reports, indicating the development and compliance with the environmental standards and requirements of the civil works.

- The ESMP should be seen as a dynamic document, with revisions being appropriate at various stages of the project and thus shall be subject to periodic audits to refocus the plans in light of experience gained and issues that have emerged.

- External environmental audits shall be performed on an annually basis financed by the Program, both over civil works implemented or under implementation, as well as over the EU/NWA supervision of works. Reports of this audit must be submitted to the Bank and to NEPA.

- The Bank Country Office of Jamaica and the Project Team will provide environmental and social impact review as part of normal Bank supervision of the TIRP.

## IX. RECOMMENDATIONS

- The ESMP must be included in the bidding documents, so that all environmental mitigation measures are properly addressed and funds are specified for.

- The ESMIPs must include specific environmental procedures, including but not limited to erosion and sedimentation control, water pollution, waste management, health and safety, air pollution and dust control, noise and vibrations. ESMIP must be prepared in coordination with the Project Management Plan and Traffic Management Plan, also to be prepared by the Contractor. The ESMIP shall be approved by the supervisor firm and EU/NWA

- Prior to first tendering of civil works, a sample of proposed rehabilitation projects shall be submitted to the Bank’s no objection, demonstrating that environmental and social impacts are addressed accordingly to the ESMP.

- No civil works must be tendered before obtaining the required permits, licenses or resettlement plans, when required.

- Prior to first disbursement, the Environmental Unit/NWA must be fully staffed and operational.

- The Environmental Unit established under NRSIP contract must provide adequate environmental supervision of project execution to comply with the ESMP.

- If involuntary resettlement of residents or vendors is required to clear the right-of-way or to increase road safety conditions, a preliminary report must be provided to the Bank explaining the conditions under which it will take place, and must comply with OP-710.

- The supervision firm will properly train the NWA/EU and Contractor’s staff, to provide environmental supervision over the project.

Environment and Safeguards Compliance Policy (OP-703); Involuntary Resettlement (OP-710); Natural Disaster (OP-704); Indigenous People (OP-765); Disclosure of Information (OP-102)

Environment and Safeguards Compliance Policy (OP-703); Involuntary Resettlement (OP-710); Natural Disaster (OP-704); Indigenous People (OP-765); Disclosure of Information (OP-102)

25 feet = 7,62 meters; total road width = 15,12 meters

Encroachments are defined as any fences, building or construction, erected upon or extending on or over any part of a road, as well as any materials thrown, drainages, pipes discharging or interrupting road operations.